Planning Standards for timber harvesting operations in Victoria’s State forests 2014 Appendix 5 to the Management Standards and Procedures for timber harvesting operations in Victoria’s State forests 2014

© The State of Victoria Department of Environment and Primary Industries 2014

This work is licensed under a Creative Commons Attribution 3.0 Australia licence. You are free to re-use the work under that licence, on the condition that you credit the State of Victoria as author. The licence does not apply to any images, photographs or branding, including the Victorian Coat of Arms, the Victorian Government logo and the Department of Environment and Primary Industries logo. To view a copy of this licence, visit http://creativecommons.org/licenses/by/3.0/au/deed.en

ISBN 978-1-74326-929-9 (online)

ISBN 978-1-74146-266-1 (Print)

Accessibility

If you would like to receive this publication in an alternative format, please telephone the DEPI Customer Service  Centre on 136186, email customer.service@depi.vic.gov.au  or via the National Relay Service on 133 677  www.relayservice.com.au. This document is also available on the internet at www.depi.vic.gov.au

Disclaimer

This publication may be of assistance to you but the State of Victoria and its employees do not guarantee that the publication is without flaw of any kind or is wholly appropriate for your particular purposes and therefore disclaims all liability for any error, loss or other consequence which may arise from you relying on any information in this publication.

| Procedure | Planning Standards for timber harvesting operations in Victoria’s State forests, 2014. Appendix 5 to the Management Standards and Procedures for timber harvesting operations in Victoria’s State forests 2014. |
| --- | --- |
| Date issued | October 2014 |
| Version | 1 |
| Commences |  |
| Last revision date |  |
| Replaces | Replaces any timber harvesting operation prescriptions contained in: - forest management plans; or - amendments or additions to forest management plans via regional guidance documents; or - Flora and Fauna Guarantee Act 1988 Action Statements. |
| Authority | Minister for Environment and Climate Change |
| Prepared by | Land Management Policy Division |
| Registry File No | FS/09/0078 |
| Scope | All DEPI planning staff, managing authorities, harvesting entities and operators |
| Review schedule | As required |
| Note | All printed copies of this document are uncontrolled. The latest version is available on the DEPI website, www.depi.vic.gov.au. |

Contents

Glossary	8

Acronymns	9

1.	Introduction	10

2.	Planning Procedures	11

2.1	Forest Management Zoning Scheme	11

2.1.1	Establishment and amendment	11

2.1.2	Management Actions	11

3.	Water Quality, River Health and Soil Protection	12

3.1	Stream protection – fixed zoning	12

3.1.1	East Gippsland FMA	12

3.1.2	Mid Murray FMA	12

3.2	Water supply protection areas – fixed zoning	12

3.2.1	Benalla-Mansfield FMA	12

3.2.2	East Gippsland FMA	13

3.2.3	Midlands FMA	13

3.3	Wetlands – fixed zoning	15

3.3.1	Mid Murray FMA	15

4.	Biodiversity	16

4.1	Linear reserves – fixed zoning	16

4.1.1	Central FMAs	16

4.1.2	East Gippsland FMA	16

4.1.3	Midlands FMA	16

4.1.4	North East FMAs	17

4.1.5	Otway FMA	17

4.2	Fauna – fixed zoning	17

4.2.1	Statewide	17

4.2.2	East Gippsland FMA	18

4.3	Fauna - detection based zoning	36

4.3.1	Statewide	36

4.4	Flora – fixed zoning	69

4.4.1	Tall Astelia	69

4.5	Flora – detection based zoning	69

4.5.1	Statewide	69

4.5.2	East Gippsland FMA	69

4.5.3	Midlands FMA	69

4.5.4	Mid Murray FMA	70

4.5.5	North East FMAs	70

4.5.6	Otway FMA	70

4.6	Vegetation communities – fixed zoning	111

4.6.1	Statewide	111

4.6.2	Bendigo FMA	111

4.6.3	Central Highlands FMAs	111

4.6.4	East Gippsland FMA	112

4.6.5	Gippsland FMAs and North East FMAs	113

4.6.6	Mid Murray FMA	114

4.6.7	Portland and Horsham FMA	115

4.7	Vegetation communities – detection based zoning	115

4.7.1	East Gippsland, Midlands and Otway FMAs	115

5.	Historic Places	116

5.1.1	Statewide – detection based zoning	116

5.1.2	Bendigo FMA, Central Highlands FMAs, Gippsland FMAs, Mid Murray FMA, North East FMAs and Portland Horsham FMA – fixed zoning	116

5.1.3	Midlands and Otway FMA – fixed zoning	116

6.	Landscape	139

6.1	Landscape – fixed zoning	139

6.1.1	Central Highlands FMAs	139

6.1.2	East Gippsland FMA	139

6.1.3	Gippsland FMAs	139

6.1.4	Midlands FMA	139

6.1.5	Mid Murray FMA	140

6.1.6	North East FMAs	140

6.2	Heritage Rivers and Natural Catchment Areas – fixed zoning	154

6.3	River Zones and Natural Features Zones – fixed zoning	154

6.3.1	Central FMAs	154

6.3.2	East Gippsland FMA	154

7.	Recreation – fixed zoning	155

7.1.1	Statewide	155

7.1.2	Midlands FMA	155

8.	Research – fixed zoning	167

8.1.1	Statewide	167

# Glossary

The first time a word from this glossary appears on each page of these Planning Standards, it has been highlighted in bold.  In this document, unless the context requires otherwise terms used have the same meaning as expressed in the Management Standards and Procedures or as follows:

‘JANIS criteria’ means the Nationally Agreed Criteria for the Establishment of a Comprehensive Adequate and Representative Reserve System for Forests in Australia.

‘Leadbeater’s Possum Management Unit’ means aggregations of one or more forest management blocks containing contiguous patches of ash-eucalypt forest within the known range of Leadbeater’s Possum. These spatial units are used to distribute retained habitat areas across the known range of Leadbeater’s Possum.

‘public native forest’ includes native forest landscape in State forest, parks and reserves.

‘viable’ means capable of living, developing, or germinating under favourable conditions.

# Acronymns

BOMA – Barking Owl Management Area

BPMA – Brush-tailed Phascogale Management Area

CAR – Comprehensive, Adequate and Representative

CPMA – Carpet Python Management Area

DC – Designated Catchment as identified in a Forest Management Plan

EVC – Ecological Vegetation Class

FMA – Forest Management Area

FMZ – Forest Management Zone

GMZ – General Management Zone

GRU – Geographic Representation Unit

Ha - hectare

JANIS - Joint ANZECC/MCFFA National Forest Policy Statement Implementation Subcommittee

LCC – Land Conservation Council

LMU – Leadbeater’s Possum Management Unit

MOMA – Masked Owl Management Area

NE – National Estate

POMA – Powerful Owl Management Area

SAP – Special Area Plan

SF – State forest

SMZ – Special Management Zone

SPZ – Special Protection Zone

SWSC – Special Water Supply Catchment

# Introduction

The Planning Standards for timber harvesting operations in Victoria’s State forests 2014 are an appendix to the Management Standards and Procedures which are incorporated into the Code.

Throughout this document the planning standards can be categorised as either fixed zoning requirements or detection based zoning requirements.

Fixed zoning requirements are the responsibility of DEPI to represent spatially in the corporate FMZ scheme.

Detection based zoning requirements are also the responsibility of DEPI to represent spatially in the corporate FMZ scheme however where evidence of a detection based value (as outlined in this document) is found in the field and it isn’t already included in the FMZ scheme, application must be made by the managing authority to the Secretary or delegate prior to commencement of the timber harvesting to amend the FMZ scheme.

# Planning Procedures

## Forest Management Zoning Scheme

### Establishment and amendment

      - In establishing and maintaining a FMZ scheme, the planning standards in this document that apply to SPZ and SMZ establishment and amendment should be adhered to.

      - Where accurate data is available DEPI will represent planning standards spatially in the corporate FMZ scheme.

      - Amendments to the FMZ scheme must be in accordance with the relevant regional forest agreement, the relevant forest management plan and the Code of Practice for Timber Production 2014.

      - DEPI may produce a Standard Operating Procedure for the purpose of managing amendments to the FMZ scheme.

### Management Actions

      - In conducting timber harvesting operations, the planning standards in this document that apply to SMZ or SPZ management actions (which may or may not be represented spatially in the corporate forest management zoning scheme) are to be adhered to.

# Water Quality, River Health and Soil Protection

## Stream protection – fixed zoning

### East Gippsland FMA

#### Establish SPZs for stream protection as outlined in table 1 below.

Table 1 East Gippsland stream protection SPZs.

| Minimum SPZ width (m) | Stream |
| --- | --- |
| Heritage Rivers |  |
| 100 | Arte River |
| 200 | Bemm River (downstream of Princes Hwy) |
| 150 | Bemm River (upstream of Princes Hwy), Goolengook River |
| >200 | Snowy River |
| Linear Reserves |  |
| 150 | Betka River, Brodribb River (from Sardine Creek upstream to Big River junction), Genoa River, Mueller River, Timbarra River (downstream of Timbarra), Wallagaraugh River, Wingan River (downstream of Wingan Swamp), Thurra River (downstream of Thurra junction) |
| 200 | Brodribb River (from Brodribb Flora reserve upstream to Sardine Creek), Simpson Creek, Hartland River (part), Hensleigh Creek and tributaries, Cann River (downstream of Cann River township), Little River (in the Blue Gum Flat area) |

### Mid Murray FMA

#### Establish a 10 m SPZ extending from the top of either bank of Yarran Creek in Gunbower Forest.

## Water supply protection areas – fixed zoning

### Benalla-Mansfield FMA

#### Include the designated catchments listed in Table 2 (Water supply protection areas) below in the SMZ. Within the SMZ:

#### apply winter closures and maximum coupe sizes in accordance with Table 2 (Water supply protection areas) below for timber harvesting, roadworks and other heavy earthworks; and

#### limit the total area of coupes using even aged harvesting and regeneration systems, such as clearfell or seed tree operations, to 5% of the area of public land in designated catchments over any 3 year period.

### East Gippsland FMA

#### Include the Rocky and Betka River catchments in the SMZ. Within the SMZ:

#### no new road crossings may be built on major streams; and

#### apply harvesting restrictions and stream buffers in accordance with Table 2 (Water supply protection areas) below.

### Midlands FMA

#### Include the designated catchments listed in Table 2 (Water supply protection areas) below in the SMZ. Within the SMZ:

#### apply winter closures and maximum coupe sizes in accordance with the Management Standards and Procedures for timber harvesting, roadworks and other heavy earthworks;

#### extend the period of closure should wet conditions commence earlier or continue beyond the standard seasonal closure dates;

#### manage harvesting and other soil disturbing operations in accordance with section 3.5 of the Management Standards and Procedures;

#### schedule timber harvesting operations to limit the proportion of regrowth up to 20 years old (including post-shelterwood 1 areas) to a maximum of 20% of the area of public native forest in designated catchments; and

#### limit clearfell, seed-tree or shelterwood 1 operations to 5% of the area of public land in designated catchments over any 3 year period.

Table 2 Water supply protection areas.

|  |  |  | Management actions |  |
| --- | --- | --- | --- | --- |
| FMA | Catchment | Status | Stream buffers or seasonal closure | Harvesting restrictions |
| Benalla-Mansfield | Upper Goulburn (Upper Delatite) | SWSC & SAP | 1 June – 31 Oct | 20 m B 10 m F and slope limit 30o |
| Benalla-Mansfield | Lake Nillahcootie | SWSC | 1 June – 31 Oct | 20 m B 10 m F and slope limit 30o |
| Benalla-Mansfield | Ryans Creek | SWSC & SAP; DC Category 1 Category 2 Category 3 | - 1 May – 31 Oct 20 m B 10 m F 1 May – 31 Oct 20 m B 10 m F | 40 ha max coupe size where harvesting permitted No harvesting No clearfall and slope limit <12o slope limit 30o |
| East Gippsland | Orbost (Rocky River) | SWSC & SAP | 100m on main river. 40m on major tributaries 30m on other watercourses 100m on off-take weir. | 40 ha maximum annual area harvested. |
| East Gippsland | Betka River | SWSC | 150m on main river. 20m on major tributaries and other permanent streams. | 120 ha (1% of catchment) max annual area harvested. |
| Midlands | Blackwood | DC | 1 June - 31 Oct | 35 ha maximum coupe size. |
| Midlands | Bullarto | DC | 1 June - 31 Oct | 20 ha maximum coupe size. |
| Midlands | Colbrook | DC | 1 June - 31 Oct | 5 ha maximum coupe size. |
| Midlands | Collier Gap | DC | 1 June - 31 Oct | 2 ha maximum coupe size. |
| Midlands | Djerriwarrh | SWSC & SAP; DC | 1 June - 30 Sept | 15 ha maximum coupe size. |
| Midlands | Korweinguboora | DC | 1 June - 31 Oct | 15 ha maximum coupe size. |
| Midlands | Hickmans Creek (Elmhurst) | DC | 1 June - 31 Oct | 40 ha maximum coupe size. |
| Midlands | Lal Lal Lake Environs | SWSC & SAP; DC | 1 June - 31 Oct | 5 ha maximum coupe size. |
| Midlands | Long Gully | DC | 1 June - 31 Oct | 15 ha maximum coupe size. |
| Midlands | Lake Merrimu | SWSC & SAP; DC | 1 June - 30 Sept | 40 ha maximum coupe size. Buffers apply to all waterways |
| Midlands | Moorabool | DC | 1 June - 30 Sept | 10 ha maximum coupe size. |
| Midlands | Mount Cole | DC | 1 June - 31 Oct | 20 ha maximum coupe size. |
| Midlands | Musical Gully | SWSC; DC | 1 June - 30 Sept | 2 ha maximum coupe size. |
| Midlands | Pykes Creek | SWSC; DC | 1 June - 30 Sept | 40 ha maximum coupe size. |
| Midlands | Rosslynne | SWSC & SAP; DC | 1 June - 30 Sept | 40 ha maximum coupe size. |
| Midlands | Shepherds Creek | DC | 1 June - 31 Oct | 40 ha maximum coupe size. |
| Midlands | Sugarloaf | DC | 1 June - 30 Sept | 20 ha maximum coupe size. |
| Midlands | Troy | SWSC; DC | 1 June - 30 Sept | 2 ha maximum coupe size. |
| Midlands | White Swan | DC | 1 June - 31 Oct | 20 ha maximum coupe size. |
| Midlands | Wombat | DC | 1 June - 31 Oct | 20 ha maximum coupe size. |

## Wetlands – fixed zoning

### Mid Murray FMA

#### Establish a 50 m radius SMZ, extending from the tree-line, around important water bodies and other open areas, such as Moira Grass plains. Within the SMZ:

#### improve structural diversity;

#### afford additional protection to large trees;

#### promote River Red Gum (Eucalyptus camaldulensis) regeneration to provide or restore roosting sites for colonially-nesting waterbirds;

#### improve roosting and nesting opportunities for birds, through thinning of dense stands to accelerate the development of heavier branching in the retained trees; and

#### prevent or reverse forest encroachment onto Moira Grass plains, through suppression or removal of River Red Gum (Eucalyptus camaldulensis) regeneration.

NOTE: Important water bodies have been identified by DEPI through remote sensing and field inspections.

In places, as a result of either the natural dynamics of the floodplain or water regulation, River Red Gum (Eucalyptus camaldulensis) is encroaching onto wetland areas, reducing the open area. Encroaching trees may be identified for removal from important wetlands. In these instances the tree line does not include the encroaching trees identified for removal.

# Biodiversity

## Linear reserves – fixed zoning

### Central FMAs

#### Establish and maintain a network of linear reserves to connect larger areas of SPZ with each other and with conservation reserves and to cross altitudinal gradients. The linear reserves are a minimum of 200 metres wide and managed as SPZ. Ensure that the linear reserve network:

#### links areas of high conservation value forest;

#### where feasible links patches of ash-eucalypt retained for Leadbeater’s Possum habitat;

#### includes areas of riparian vegetation, mid-slope and ridge vegetation; and

#### includes some linear reserves that cross altitudinal gradients.

### East Gippsland FMA

#### Establish and maintain a network of linear reserves to provide alternative links between conservation reserves and larger parts of the SPZ and SMZ and to span altitudinal and latitudinal gradients. The linear reserves are an average of 200 metres wide and managed as SPZ. Ensure that the linear reserve network:

#### generally comprises old forest containing high quality habitat;

#### builds on and complements existing Natural Features Zones, Heritage River corridors and stream buffers; and

#### is located to reduce the impact of potential barriers such as the Princes and Cann Valley Highways.

#### Avoid road construction in linear reserve SPZs. Where unavoidable, minimise road width and retain canopy closure over the road wherever possible.

### Midlands FMA

#### Establish and maintain a network of linear reserves to provide alternative links between conservation reserves and larger parts of the SPZ and SMZ. The linear reserves are to be managed as SPZ. Ensure that the linear reserve network:

#### provides links between gully, mid-slope and ridge habitats;

#### is sufficiently wide to enable large arboreal mammals to forest efficiently (in general 200 m is deemed to be an appropriate width);

#### comprises mature forest wherever possible; and

#### builds on and complements existing parks and conservation reserves, Heritage River corridors and stream buffers.

### North East FMAs

#### Establish and maintain a network of linear reserves to provide alternative links between conservation reserves and larger parts of the SPZ and SMZ. The linear reserves are to be managed as SPZ. Ensure that the linear reserve network:

#### is sufficiently wide to maintain resident populations of large arboreal mammals (in general 200 m average width, usually a buffer of approximately 100 m either side of a stream, or as otherwise specified in legislation);

#### comprises mature forest wherever possible;

#### takes into account the existing links provided through Code exclusions and non-productive forest; and

#### builds on and complements existing parks and conservation reserves, Heritage River corridors and stream buffers.

### Otway FMA

#### Establish and maintain a primary network of 200 m wide wildlife habitat corridors in SPZ linking to existing conservation reserves.

## Fauna – fixed zoning

### Statewide

#### Plan management actions for rare and endangered fauna in accordance with Table 3 (Fixed FMZ rules for fauna) below.

#### Maintain FMZ schemes for rare and endangered fauna in accordance with Table 3 (Fixed FMZ rules for fauna) below.

### East Gippsland FMA

#### Maintain well-documented sites that are particularly rich in birds in 20 ha SPZs.

#### Maintain well-documented sites that are particularly rich in mammal species in the SPZ or SMZ wherever practical.

#### Maintain important butterfly breeding and hilltop congregation areas within SPZ where they coincide with other FMZ values. Otherwise they will be included in the SMZ.  In SMZ areas exclude timber harvesting operations.

Table 3 Fixed FMZ rules for fauna.

| FMA | Common name | Scientific name | Zoning management actions | Management actions | Review |
| --- | --- | --- | --- | --- | --- |
| Bendigo | Barking Owl | Ninox connivens | Identify and maintain a target of 150 resident pairs as Barking Owl Management Areas (BOMA) across their main range in Victoria on either public or private land. Within the Bendigo FMA BOMAs are to be between 600-1000 ha. BOMA selection considerations include: - priority to protect nest sites or probable breeding areas based on the occurrence of owlets or adult roosting pairs - strength of evidence for existence of a resident pair - distribution throughout the main range of the species with a reasonable spread across habitats and land tenures - preference to protect suitable habitat within conservation reserves, especially in large reserves where the entire home range can be protected within the reserve. Maintain BOMAs that occur within State forest in SMZ. Maintain a 3ha SPZ over any specific records within the BOMA. BOMAs may overlap with management areas established for other species. | Implement prescriptions for BOMAs within SMZs in State forest that allow for modified timber harvesting practices that retain sufficient levels of habitat trees (hollow-bearing trees along with younger trees that may provide hollows in future). Refer to the Management Standards and Procedures clause 4.1.3.1 f) for Bendigo FMA SMZ habitat tree prescriptions. | Once state wide targets are met, review BOMA allocation when records of higher priority are discovered. |
| Bendigo | Brush-tailed Phascogale | Phascogale tapoatafa | Identify and maintain Brush-tailed Phascogale Management Areas (BPMAs) across public land within the Bendigo FMA in accordance with the following requirements: - each BPMA is to provide suitable habitat for a minimum of 25 female Brush-tailed Phascogales -each BPMA is to be a minimum of 1000ha, preferably already managed sympathetically for Brush-tailed Phascogales -priorities for BPMAs are confirmed 'hot spots', known presence of Brush-tailed Phascogales, reintroduction sites and suitable habitat. Include BPMAs that occur within State forest in SMZ. | Implement prescriptions for BPMAs within SMZs in State forest that protect and maintain Brush-tailed Phascogale populations and their habitat, such as retention of large trees and prohibition of removal of naturally fallen timber. Refer to the Management Standards and Procedures clause 4.1.3.1 f) for Bendigo FMA SMZ habitat tree prescriptions. | Review and change sites only if new sites are of a higher quality or are better able to be protected than existing sites. |
| Bendigo | Powerful Owl | Ninox strenua | Identify and maintain a target of 28 Powerful Owl Management Areas (POMA) of 1000 ha in area across public land in Bendigo FMA. Allocate POMAs across State forest and conservation reserves, with preference to protect suitable habitat within conservation reserves, especially in large reserves where the home range can be protected within the reserve. Where possible, locate POMAs over nest sites or probable breeding areas based on the occurrence of owlets or adult roosting pairs. Select Powerful Owl sites to meet the POMA target in the following order of priority: 1. confirmed nest trees utilised during the past 5 years 2. confirmed roost trees utilised during the past 5 years 3. repeated sighting or vocalisation during the past 5 years 4. incidental sighting or vocalisation during the past 5 years 5. historic record not reconfirmed in the past 5 years 6. potential habitat area (preferably based on formal analysis and modelling). In State forest, include POMAs in SMZ. | Within the SMZ modify timber harvesting practices to retain high levels of habitat trees consistent with research on Powerful Owl habitat characteristics. Refer to the Management Standards and Procedures clause 4.1.3.1 f) for Bendigo FMA SMZ habitat tree prescriptions. | Once regional targets are met, review POMA allocation when records of higher priority are discovered. |
| Bendigo | Swift Parrot | Lathamus discolor | Maintain a SMZ across all Swift Parrot Management Areas (SPMA) in State forest within the Bendigo FMA. SPMAs are located in Havelock State Forest/Timor State Forest Maldon State Forest Rushworth State Forest St.Arnaud State Forest Wareek State Forest (Chadwick Dam) Spring Plains/Argyle State Forest Illawarra State Forest Costerfield State Forest Diamond Dam Dunolly-Tarnagulla SF (Murderers Hill-Wet Gully) Kingower State Forest Lockwood South SF Mt. Hooghly SF Redcastle State Forest (Holt’s Flat) Sandon State Forest Sedgwick State Forest One-Eye State Forest | Within the SMZ: -protect all large trees (refer to the Management Standards and Procedures clause 4.1.3.1 f) for Bendigo FMA SMZ habitat tree prescriptions); and -exclude all management activities likely to disturb Swift Parrots while the birds are foraging within the area. | Review Swift Parrot SMZs if new information arises. |
| Central Highland FMAs | Baw Baw Frog | Philoria frosti | Maintain a SPZ over the potential habitat envelope of the Baw Baw Frog, based on the distribution of known frog localities (Victorian Biodiversity Atlas) and modelling of the distribution of breeding habitat (Liu and White 1999). |  |  |
| Central Highland FMAs | Brush-tailed Phascogale | Phascogale tapoatafa | Maintain a minimum of 30% of suitable habitat within SPZ. |  |  |
| Central Highland FMAs | Leadbeater's Possum | Gymnobelideus leadbeateri | Protect 30 % of Ash forest from timber harvesting operations in each LMU considering the following factors for selection of retained patches: - the aim to retain patches least likely to burn during bushfires; - the aim to retain patches of the oldest age class that will develop into old growth the quickest; and - the aim to retain patches that will consolidate other patches that may develop into old growth. Note preference should be given to meeting the 30 % target from conservation reserves or SPZs followed by areas informally reserved via timber harvesting exclusion areas in the Code. |  |  |
|  |  |  | Establish and maintain SPZs of 200 m radius centred on each verified Leadbeater’s Possum colony found since 1998 that is not located within forest areas severely impacted by the 2009 wildfire (Exclude colonies within Fire Severity classes 1 and 2) |  |  |
| Central Highland FMAs | Masked Owl | Tyto novaehollandiea novaehollandiea | Identify and maintain 20 Masked Owl Management Areas (MOMA) of at least 500ha across public land in the Central FMAs. MOMA selection considerations include: - preference for location near known records nearby or areas modelled as good quality habitat - strength of evidence for existence of a resident pair - distribution throughout the main range of the species with a reasonable spread across habitats and land tenures - preference to protect the best quality suitable habitat - requirement for the MOMA to comprise patches greater than 100ha in area that are contiguous with other forest where possible. MOMAs may overlap with management areas established for other species. Include State forest areas in SPZ. Where clear-fell or seed-tree systems are used, MOMAs are at least 500 ha in area and State forest areas are maintained as SPZ. Where these MOMAs are based on specific records (rather than habitat modelling), locate them within a 3.5 km radius (approximate area of 3,800 ha) of the record. |  | Once regional targets are met, review MOMA allocation when records of higher priority are discovered. |
| Central Highland FMAs | Powerful Owl | Ninox strenua | Identify and maintain a target of 50 Powerful Owl Management Areas (POMA) of at least 500 ha of suitable habitat across public land in the Central Highlands FMAs. Allocate POMAs across State forest and conservation reserves, with preference to protect suitable habitat within conservation reserves, especially in large reserves where the home range can be protected within the reserve. Where possible, locate POMAs over nest sites or probable breeding areas based on the occurrence of owlets or adult roosting pairs. Select Powerful Owl sites to meet the POMA target in the following order of priority: 1. confirmed nest trees utilised during the past 5 years 2. confirmed roost trees utilised during the past 5 years 3. repeated sighting or vocalisation during the past 5 years 4. incidental sighting or vocalisation during the past 5 years 5. historic record not reconfirmed in the past 5 years 6. potential habitat area (preferably based on formal analysis and modelling). Locate the POMA within a 3 km radius of the Powerful Owl site. Suitable habitat areas are areas of greater than 100 ha (contiguous if possible) dominated by old trees and areas likely to support high densities of prey species. Where forest stand characteristics limit the adequacy of the core habitat area, maintain additional habitat of up to 500 ha within the same 3 km radius. Determine the size of the core habitat area and any additional habitat requirements based on assessment of the suitability of both existing forest habitat and regrowth forest in relation to prey densities. In State forest include the core habitat area of the POMA in SPZ and any additional area in either SPZ or SMZ. | Within the SMZ modify timber harvesting practices to retain high levels of habitat trees consistent with research on Powerful Owl habitat characteristics. | Once regional targets are met, review POMA allocation when records of higher priority are discovered. Review this strategy when new information about owl populations, habitat requirements and sightings is available. |
| Central Highland FMAs | Sooty Owl | Tyto tenebricosa | Identify and maintain a target of 100 Sooty Owl Management Areas (SOMA) across public land in the Central FMAs. Locate SOMAs based on probable breeding areas based on the occurrence of owlets or adult roosting pairs and on habitat identified by habitat modelling. SOMAs may overlap with management areas established for other species. Allocate SOMAs across State forest and conservation reserves, with preference to protect suitable habitat within conservation reserves, especially in large reserves where the home range can be protected within the reserve. Where clearfell or seed tree harvesting systems are used, select a 500 ha area within a 3.5 km radius of the record (approximate area of 3800 ha) for each SOMA. Align SOMA boundaries with recognisable features, preferably natural, such as ridgelines or sub-catchments. Where possible SOMAs should comprise of patches greater than 100 ha in area and contiguous with other forest. Maximise the inclusion of habitats known to be used by the Sooty Owl, such as forest in headwaters, old-growth forest in gullies, forest with a diversity of preferred EVCs, forest of preferred growth stages such as mixed - senescent - mature, otherwise mature or mixed senescent - mature - regrowth, forest with large and / or dead hollow-bearing trees, forest with abundant Silver Wattle, Tree-ferns and Blanket-leaf, and forest in deep gullies. Avoid locating SOMAs in extensive areas of forest known to be less suitable, forest less than 28 m tall, treeless areas, regrowth forest or any of the dry EVCs. Include State forest areas of the SOMA in SPZ. |  |  |
| East Gippsland | Barking Owl | Ninox connivens | Identify and maintain a target of 150 resident pairs as Barking Owl Management Areas (BOMA) across their main range in Victoria on either public or private land. Within the East Gippsland FMA BOMAs are to be between 300-500 ha where clear-fell or seed-tree systems are used (State forest areas are to be maintained as SPZ) or between 600-1000 ha in area where selective harvesting systems are used (State forest areas are maintained as SMZ). BOMA selection considerations include: - priority to protect nest sites or probable breeding areas based on the occurrence of owlets or adult roosting pairs - strength of evidence for existence of a resident pair - habitat suitability - distribution throughout the main range of the species with a reasonable spread across habitats and land tenures - preference to protect suitable habitat within conservation reserves, especially in large reserves where the entire home range can be protected within the reserve. BOMAs may overlap with management areas established for other species. Where BOMAs are based on specific records (rather than habitat modelling), locate them within a 3.5km radius (approximate area of 3,800 ha) of the record. Maintain a 3ha SPZ over any specific records within these BOMAs. | Prepare a SMZ plan or prescriptions prior to harvesting commencement that specify protection of hollow-bearing trees for owls and prey species along with younger trees that may provide hollows in future. | Once state wide targets are met, review BOMA allocation when records of higher priority are discovered. |
| East Gippsland | Long-footed Potoroo | Potorous longipes | Maintain areas of State forest within the East Gippsland Long-footed Potoroo Core Protected Area within SPZ. |  |  |
| East Gippsland | Masked Owl | Tyto novaehollandiea novaehollandiea | Identify and maintain a target of 150 resident pairs as Masked Owl Management Areas (MOMA) across their main range in Victoria on either public or private land. Within the East Gippsland FMA MOMAs are to be at least 500 ha where clear-fell or seed-tree systems are used (State forest areas are to be maintained as SPZ) or at least 1000 ha in area where selective harvesting systems are used (State forest areas are maintained as SMZ). MOMA selection considerations include: - strength of evidence for existence of a resident pair - habitat suitability - distribution throughout the main range of the species with a reasonable spread across habitats and land tenures - preference to protect suitable habitat within conservation reserves, especially in large reserves where the entire home range can be protected within the reserve. MOMAs may overlap with management areas established for other species. Where these MOMAs are based on specific records (rather than habitat modelling), locate them within a 3.5 km radius (approximate area of 3,800 ha) of the record. Maintain a 3 ha SPZ over any specific records within these MOMAs. | Prepare a SMZ plan prior to harvesting commencement that specify retention of sufficient levels of habitat trees for Masked Owl. | Once state wide targets are met, review MOMA allocation when records of higher priority are discovered. |
| East Gippsland | Powerful Owl | Ninox strenua | Identify and maintain a target of 100 Powerful Owl Management Areas (POMA) across public land in East Gippsland FMA. Where possible, locate POMAs over nest sites or probable breeding areas based on the occurrence of owlets or adult roosting pairs. Select Powerful Owl sites to meet the POMA target in the following order of priority: 1. confirmed nest trees utilised during the past 5 years 2. confirmed roost trees utilised during the past 5 years 3. repeated sighting or vocalisation during the past 5 years 4. incidental sighting or vocalisation during the past 5 years 5. historic record not reconfirmed in the past 5 years 6. potential habitat area (preferably based on formal analysis and modelling). Allocate POMAs across State forest and conservation reserves, with preference to protect suitable habitat within conservation reserves, especially in large reserves where the home range can be protected within the reserve. Where clearfell harvesting systems are used, maintain a core area of suitable habitat of at least 800 ha (dependent on habitat type) within a 3.5km radius (approximate area of 3800 ha) for each POMA. Suitable habitat is areas dominated by old trees and areas likely to support high densities of prey species. Where forest stand characteristics limit the adequacy of the core habitat area, maintain additional habitat of up to 800ha within the same 3.5 km radius. Determine the size of the core habitat area and any additional habitat requirements based on assessment of the suitability of both existing forest habitat and regrowth forest in relation to prey densities. In State forest include at least 800 ha of core habitat area within the POMA in SPZ. Maintain additional habitat as SPZ and/or SMZ. Where selective harvesting systems are used, allocate POMAs of 1000 ha in area. In State forest include these POMAs in SMZ. | Within the SMZ modify timber harvesting practices to retain high levels of habitat trees consistent with research on Powerful Owl habitat characteristics. | Once regional targets are met, review POMA allocation when records of higher priority are discovered. |
| East Gippsland | Sooty Owl | Tyto tenebricosa | Identify and maintain a target of 131 Sooty Owl Management Areas (SOMA) across public land in East Gippsland FMA. Locate SOMAs based on probable breeding areas based on the occurrence of owlets or adult roosting pairs and on habitat identified by habitat modelling. SOMAs may overlap with management areas established for other species. Allocate SOMAs across State forest and conservation reserves, with preference to protect suitable habitat within conservation reserves, especially in large reserves where the home range can be protected within the reserve. Where clearfell or seed tree harvesting systems are used, select a 500 ha area within a 3.5 km radius of the record (approximate area of 3800 ha) for each SOMA. Align SOMA boundaries with recognisable features, preferably natural, such as ridgelines (sub-catchments or groups of sub-catchments are ideal). Maximise the inclusion of habitats known to be used by the Sooty Owl, such as forest in headwaters, old-growth forest in gullies, forest with a diversity of preferred EVCs, forest of preferred growth stages such as mixed - senescent - mature, otherwise mature or mixed senescent - mature - regrowth, forest with large and / or dead hollow-bearing trees, forest with abundant Silver Wattle, Tree-ferns and Blanket-leaf, and forest in deep gullies. Avoid locating SOMAs in extensive areas of forest known to be less suitable, forest less than 28 m tall, treeless areas, regrowth forest or any of the dry EVCs. Include State forest areas of the SOMA in SPZ. Where selective harvesting systems are used, allocate SOMAs of 1000 ha in area. In State forest include these SOMAs in SMZ. Maintain 3 ha SPZ over any specific records within the SOMA. | Within the SMZ, prepare SMZ plans specifying prescriptions for retention of sufficient levels of habitat trees prior to timber harvesting. |  |
| Gippsland FMAs | Barking Owl | Ninox connivens | Identify and maintain a target of 150 resident pairs as Barking Owl Management Areas (BOMA) across their main range in Victoria on either public or private land. Within the Gippsland FMA (part within Gippsland FMP) and Tambo FMA BOMAs are to be between 300-500 ha. BOMA selection considerations include: - priority to protect nest sites or probable breeding areas based on the occurrence of owlets or adult roosting pairs - strength of evidence for existence of a resident pair - habitat suitability - distribution throughout the main range of the species with a reasonable spread across habitats and land tenures - preference to protect suitable habitat within conservation reserves, especially in large reserves where the entire home range can be protected within the reserve. BOMAs may overlap with management areas established for other species. Where BOMAs are based on specific records (rather than habitat modelling), the locate them to fall within a 3.5 km radius (approximate area of 3,800 ha) of the record. Maintain BOMAs that occur within State forest in SPZ. |  | Once state wide targets are met, review BOMA allocation when records of higher priority are discovered. |
| Gippsland FMAs | Long-footed Potoroo | Potorous longipes | Maintain areas of State forest within the Great Divide Long-footed Potoroo Core Protected Area within SPZ. |  | Review if significant new information regarding the habitat requirements or preferences of the Long-footed Potoroo arise. |
| Gippsland FMAs | Masked Owl | Tyto novaehollandiea novaehollandiea | Identify and maintain a target of 150 resident pairs as Masked Owl Management Areas (MOMA) across their main range in Victoria on either public or private land. Within the Gippsland FMA MOMAs are to be at least 500 ha. MOMA selection considerations include: - preference for location near known records nearby or areas modelled as good quality habitat - strength of evidence for existence of a resident pair - distribution throughout the main range of the species with a reasonable spread across habitats and land tenures - preference to protect the best quality suitable habitat - requirement for the MOMA to comprise patches greater than 100ha in area that are contiguous with other forest where possible. MOMAs may overlap with management areas established for other species. Include State forest areas in SPZ. Where clear-fell or seed-tree systems are used, MOMAs are at least 500 ha in area and State forest areas are maintained as SPZ. Where these MOMAs are based on specific records (rather than habitat modelling), locate them within a 3.5 km radius (approximate area of 3,800 ha) of the record. |  | Once state wide targets are met, review MOMA allocation when records of higher priority are discovered. |
| Gippsland FMAs | Powerful Owl | Ninox strenua | Identify and maintain a target of 115 Powerful Owl Management Areas (POMA) of at least 500 ha of suitable habitat across public land in the Gippsland FMAs. Allocate POMAs across State forest and conservation reserves, with preference to protect suitable habitat within conservation reserves, especially in large reserves where the home range can be protected within the reserve. Where possible, locate POMAs over nest sites or probable breeding areas based on the occurrence of owlets or adult roosting pairs. Select Powerful Owl sites to meet the POMA target in the following order of priority: 1. confirmed nest trees utilised during the past 5 years 2. confirmed roost trees utilised during the past 5 years 3. repeated sighting or vocalisation during the past 5 years 4. incidental sighting or vocalisation during the past 5 years 5. historic record not reconfirmed in the past 5 years 6. potential habitat area (preferably based on formal analysis and modelling). Locate the POMA within a 3.5 km radius of the Powerful Owl site. Suitable habitat areas are areas of greater than 100ha (contiguous if possible) dominated by old trees and areas likely to support high densities of prey species. In State forest include the POMA in SPZ. |  | Review POMA allocation when records of higher priority are discovered subject to other management objectives. |
| Gippsland FMAs | Sooty Owl | Tyto tenebricosa | Identify and maintain a target of 169 Sooty Owl Management Areas (SOMA) across public land in the Gippsland FMAs. Locate SOMAs based on probable breeding areas based on the occurrence of owlets or adult roosting pairs and on habitat identified by habitat modelling. SOMAs may overlap with management areas established for other species. Allocate SOMAs across State forest and conservation reserves, with preference to protect suitable habitat within conservation reserves, especially in large reserves where the home range can be protected within the reserve. Where clearfell or seed tree harvesting systems are used, select a 500 ha area within a 3.5 km radius of the record (approximate area of 3800 ha) for each SOMA. Align SOMA boundaries with recognisable features, preferably natural, such as ridgelines or sub-catchments. Where possible SOMAs should comprise of patches greater than 100 ha in area and contiguous with other forest. Maximise the inclusion of habitats known to be used by the Sooty Owl, such as forest in headwaters, old-growth forest in gullies, forest with a diversity of preferred EVCs, forest of preferred growth stages such as mixed - senescent - mature, otherwise mature or mixed senescent - mature - regrowth, forest with large and / or dead hollow-bearing trees, forest with abundant Silver Wattle, Tree-ferns and Blanket-leaf, and forest in deep gullies. Avoid locating SOMAs in extensive areas of forest known to be less suitable, forest less than 28 m tall, treeless areas, regrowth forest or any of the dry EVCs. Include State forest areas of the POMA in SPZ. |  |  |
| Mid Murray | Carpet Python | Morelia spilota metcalfei | Maintain a Carpet Python Management Area (CPMA) of a minimum of 520 ha within the Gunbower public native forests in accordance with the following requirements for the area: -contains high quality (or potential high quality) habitat for Carpet Python -comprises in order of priority: conservation reserves; State forest SPZ (if required); and areas of State forest available for timber harvesting (if required). If the latter is required give preference to options that minimise loss of timber resources -preferably includes existing confirmed records. Maintain areas of the CPMA within State forest in SPZ. |  |  |
| Mid Murray | Colonially-nesting waterbirds |  | Maintain identified important water bodies and natural open areas within floodplain forests and a 50 m wide area of forest extending outward from the tree-line in SMZ. | Prepare SMZ plans to maintain or enhance the value of habitat within the SMZ for colonially-nesting waterbirds. |  |
| Mid Murray | Masked Owl | Tyto novaehollandiea novaehollandiea | Identify and maintain a target of 150 resident pairs as Masked Owl Management Areas (MOMA) across their main range in Victoria on either public or private land. MOMA selection considerations include: - strength of evidence for existence of a resident pair - habitat suitability - distribution throughout the main range of the species with a reasonable spread across habitats and land tenures - preference to protect suitable habitat within conservation reserves, especially in large reserves where the entire home range can be protected within the reserve. MOMAs may overlap with management areas established for other species. Where selective harvesting systems are used, MOMAs are 1000 ha in area and State forest areas are maintained as SMZ. Maintain a 3 ha SPZ over any specific records within these MOMAs. | Prepare a SMZ plan prior to harvesting commencement that specify retention of sufficient levels of habitat trees for Masked Owl. | Once state wide targets are met, review MOMA allocation when records of higher priority are discovered. |
| Mid Murray | Squirrel Glider | Petaurus norfolcensis | Maintain a linear SPZ of at least 50 m width either side of Goulburn River in State forest. Maintain a SMZ across all State forest adjacent to the Goulburn River Squirrel Glider SPZ. | Within the SMZ, protect Silver Wattle understorey and retain adequate large and hollow bearing trees for Squirrel Glider habitat during timber harvesting operations. |  |
| Midlands | Brush-tailed Phascogale | Phascogale tapoatafa | Maintain 30 % of Red Stringybark/Box and Stringybark/Peppermint Open Forest in the State forest SPZ and conservation reserves. Maintain sufficient areas of SMZ to sustain a Midlands FMA target population of 100 breeding females. | Implement prescriptions for BPMAs within SMZs in State forest that protect and maintain Brush-tailed Phascogale populations and their habitat, such as retention of large trees and prohibition of removal of naturally fallen timber. |  |
| Midlands | Greater Glider | Petauroides volans | Maintain a SPZ in the vicinity of the Spargo Creek Education Area along the Blakeville-Bunding Road and McGees Track to protect the major Midlands FMA Greater Glider population. Maintain a SMZ of approximately 1000 ha between the Daylesford-Ballan Road and Blakeville, south of the Spargo-Blakeville Road. | Within the SMZ: -protect Greater Glider nesting and feeding sites -restrict coupe size to 20ha -separate coupes by 50m wide corridors (these form part of the SPZ). |  |
| Midlands | Mountain Brushtail Possum | Trichosurus caninus | Maintain a SMZ of approximately 300 ha centred on the population near Victoria Mill in Mount Cole State forest. | Within the SMZ, conduct harvesting in accordance with prescriptions developed to maintain key habitat attributes for Mountain Brushtail Possum. |  |
| Midlands | Powerful Owl | Ninox strenua | Identify and maintain a target of 25 Powerful Owl Management Areas (POMA) across public land in the Midlands FMA area. Allocate POMAs across State forest and conservation reserves, with preference to protect suitable habitat within conservation reserves, especially in large reserves where the home range can be protected within the reserve. Where possible, locate POMAs over nest sites or probable breeding areas based on the occurrence of owlets or adult roosting pairs. Select Powerful Owl sites to meet the POMA target in the following order of priority: 1. confirmed nest trees utilised during the past 5 years 2. confirmed roost trees utilised during the past 5 years 3. repeated sighting or vocalisation during the past 5 years 4. incidental sighting or vocalisation during the past 5 years 5. historic record not reconfirmed in the past 5 years 6. potential habitat area (preferably based on formal analysis and modelling). Designate a POMA of 3.5 km radius over the Powerful Owl site. Suitable habitat is: - SPZ (regardless of the state of forest within) and SMZ within the POMA - mature forest - regrowth forest 30 or more years old - non-eucalypt (e.g. Blackwood in gullies) - 'other eucalypt' forest (non-productive) - mixed age forest if more than 50 % of the canopy cover is mature over logging regrowth - fire regrowth, if not salvaged or if more than 50% of the canopy is mature. Reserve 500 ha of mature forest within the POMA from timber harvesting in existing conservation reserves or SPZ within State forest. Maintain the remaining State forest area within the POMA within SMZ. | Within the SMZ: -retain all large, dense foliage Blackwood trees -manage timber harvesting activities to maintain at any time a total of 1000ha of suitable habitat within 3.5km of the Powerful Owl site. | Review POMA allocation when records of higher priority are discovered subject to other management objectives. |
| Midlands | Swift Parrot | Lathamus discolor | Maintain at least 30 % of dry forest and woodland in the Midlands FMA in the State forest SPZ and conservation reserves. |  |  |
| North East FMAs | Barking Owl | Ninox connivens | Identify and maintain 300-500 ha Barking Owl Management Areas (BOMA) across the Benalla - Mansfield and North East FMAs. BOMA selection considerations include: - priority to protect nest sites or probable breeding areas based on the occurrence of owlets or adult roosting pairs - strength of evidence for existence of a resident pair - habitat suitability - distribution throughout the main range of the species with a reasonable spread across habitats and land tenures - preference to protect suitable habitat within conservation reserves, especially in large reserves where the entire home range can be protected within the reserve. BOMAs may overlap with management areas established for other species. Where BOMAs are based on specific records (rather than habitat modelling), the locate them to fall within a 3.5 km radius (approximate area of 3,800 ha) of the record. Maintain BOMAs that occur within State forest in SPZ. |  | Review this strategy once 10 sites are established across the Benalla - Mansfield / North East FMA area. |
| North East FMAs | Long-footed Potoroo | Potorous longipes | Maintain areas of State forest within the Great Divide Long-footed Potoroo Core Protected Area within SPZ. |  | Review if significant new information regarding the habitat requirements or preferences of the Long-footed Potoroo arise. |
| North East FMAs | Masked Owl | Tyto novaehollandiea novaehollandiea | Identify and maintain Masked Owl Management Areas (MOMA) across public land. MOMA selection considerations include: - strength of evidence for existence of a resident pair - habitat suitability - distribution throughout the main range of the species with a reasonable spread across habitats and land tenures - preference to protect suitable habitat within conservation reserves, especially in large reserves where the entire home range can be protected within the reserve. - requirement for the MOMA to comprise patches greater than 100 ha in area that are contiguous with other forest where possible. MOMAs may overlap with management areas established for other species. Where clear-fell or seed-tree systems are used, MOMAs are at least 500 ha in area and State forest areas are maintained as SPZ. Where these MOMAs are based on specific records (rather than habitat modelling), locate them within a 3.5 km radius (approximate area of 3,800 ha) of the record. |  | Review this strategy once 10 sites are established across the Benalla - Mansfield / Benalla-Mansfield, North East FMA area. |
| North East FMAs | Powerful Owl | Ninox strenua | Identify and maintain a target of 125 Powerful Owl Management Areas (POMA) of at least 500 ha of suitable habitat across public land in the Benalla-Mansfield and Benalla-Mansfield, North East FMA area. Allocate POMAs across State forest and conservation reserves, with preference to protect suitable habitat within conservation reserves, especially in large reserves where the home range can be protected within the reserve. Where possible, locate POMAs in the best habitat (subject to other management objectives) over nest sites or probable breeding areas based on the occurrence of owlets or adult roosting pairs. Select Powerful Owl sites to meet the POMA target in the following order of priority: 1. confirmed nest trees utilised during the past 5 years 2. confirmed roost trees utilised during the past 5 years 3. repeated sighting or vocalisation during the past 5 years 4. incidental sighting or vocalisation during the past 5 years 5. historic record not reconfirmed in the past 5 years 6. potential habitat area (preferably based on formal analysis and modelling). Locate the POMA within a 3.5 km radius of the Powerful Owl site. Suitable habitat areas are areas of greater than 100 ha (contiguous if possible) dominated by old trees and areas likely to support high densities of prey species. Include State forest areas of the POMA in SPZ. |  | Review POMA allocation when records of higher priority are discovered subject to other management objectives. |
| North East FMAs | Sooty Owl | Tyto tenebricosa | Identify and maintain a target of 100 Sooty Owl Management Areas (SOMA) across public land in the Benalla-Mansfield and Benalla-Mansfield, North East FMA area. Locate SOMAs based on probable breeding areas based on the occurrence of owlets or adult roosting pairs and on habitat identified by habitat modelling. SOMAs may overlap with management areas established for other species. Allocate SOMAs across State forest, conservation reserves and other suitable public land areas with preference to protect suitable habitat within conservation reserves, especially in large reserves where the home range can be protected within the reserve. Where clearfell or seed tree harvesting systems are used, select a 500 ha area within a 3.5 km radius of the record (approximate area of 3800ha) for each SOMA. Align SOMA boundaries with recognisable features, preferably natural, such as ridgelines or sub-catchments. Where possible SOMAs should comprise of patches greater than 100 ha in area and contiguous with other forest. Maximise the inclusion of habitats known to be used by the Sooty Owl, such as forest in headwaters, old-growth forest in gullies, forest with a diversity of preferred EVCs, forest of preferred growth stages such as mixed - senescent - mature, otherwise mature or mixed senescent - mature - regrowth, forest with large and / or dead hollow-bearing trees, forest with abundant Silver Wattle, Tree-ferns and Blanket-leaf, and forest in deep gullies. Avoid locating SOMAs in extensive areas of forest known to be less suitable, forest less than 28 m tall, treeless areas, regrowth forest or any of the dry EVCs. Include State forest areas of the SOMA in SPZ. |  |  |
| North East FMAs | Southern Myotis | Myotis macropus | Maintain a 200 m wide linear SPZ either side of Goulburn River in State forest. |  |  |
| Otway | Masked Owl | Tyto novaehollandiea novaehollandiea | Identify and maintain a target of 150 resident pairs as Masked Owl Management Areas (MOMA) across their main range in Victoria on either public or private land. MOMA selection considerations include: - strength of evidence for existence of a resident pair - habitat suitability - distribution throughout the main range of the species with a reasonable spread across habitats and land tenures - preference to protect suitable habitat within conservation reserves, especially in large reserves where the entire home range can be protected within the reserve. MOMAs may overlap with management areas established for other species. Where selective harvesting systems are used, MOMAs are 1000 ha in area. Maintain a 3ha SPZ over any specific records within these MOMAs. | Prepare prescriptions prior to harvesting commencement that specify retention of sufficient levels of habitat trees for Masked Owl. | Once state wide targets are met, review MOMA allocation when records of higher priority are discovered. |
| Portland – Horsham | Barking Owl | Ninox connivens | Identify and maintain a target of 150 resident pairs as Barking Owl Management Areas (BOMA) across their main range in Victoria on either public or private land. Within the Portland Horsham FMA BOMAs are to be between 600-1000 ha.. BOMA selection considerations include: - priority to protect nest sites or probable breeding areas based on the occurrence of owlets or adult roosting pairs - strength of evidence for existence of a resident pair - distribution throughout the main range of the species with a reasonable spread across habitats and land tenures - preference to protect suitable habitat within conservation reserves, especially in large reserves where the entire home range can be protected within the reserve. Maintain BOMAs that occur within State forest in SMZ. Maintain 3 ha SPZ over any specific records within the BOMA. BOMAs may overlap with management areas established for other species. | Prepare a SMZ plan or prescriptions prior to harvesting commencement that specify protection of hollow-bearing trees for owls and prey species along with younger trees that may provide hollows in future. | Once state wide targets are met, review BOMA allocation when records of higher priority are discovered. |
| Portland – Horsham | Brush-tailed Phascogale | Phascogale tapoatafa | Identify and maintain Brush-tailed Phascogale Management Areas (BPMAs) across public land within the Portland and Horsham FMA in accordance with the following requirements: - each BPMA is to provide suitable habitat for a minimum of 25 female Brush-tailed Phascogales -each BPMA is to be a minimum of 1000 ha, preferably already managed sympathetically for Brush-tailed Phascogales -priorities for BPMAs are confirmed 'hot spots', known presence of Brush-tailed Phascogales, reintroduction sites and suitable habitat. Include BPMAs that occur within State forest in SMZ. | Implement prescriptions for BPMAs within SMZs in State forest that protect and maintain Brush-tailed Phascogale populations and their habitat, such as retention of large trees and prohibition of removal of naturally fallen timber. | Review and change sites only if new sites are of a higher quality or are better able to be protected than existing sites. |
| Portland – Horsham | Masked Owl | Tyto novaehollandiea novaehollandiea | Identify and maintain a target of 150 resident pairs as Masked Owl Management Areas (MOMA) across their main range in Victoria on either public or private land. MOMA selection considerations include: - strength of evidence for existence of a resident pair - habitat suitability - distribution throughout the main range of the species with a reasonable spread across habitats and land tenures - preference to protect suitable habitat within conservation reserves, especially in large reserves where the entire home range can be protected within the reserve. MOMAs may overlap with management areas established for other species. Where selective harvesting systems are used, MOMAs are 1000 ha in area and State forest areas are maintained as SMZ. Maintain a 3ha SPZ over any specific records within these MOMAs. | Prepare a SMZ plan prior to harvesting commencement that specify retention of sufficient levels of habitat trees for Masked Owl. | Once state wide targets are met, review MOMA allocation when records of higher priority are discovered. |
| Portland – Horsham | Powerful Owl | Ninox strenua | Identify and maintain a target of 40 Powerful Owl Management Areas (POMA) of 1000 ha in area across public land in the Portland and Horsham FMA. Allocate POMAs across State forest and conservation reserves, with preference to protect suitable habitat within conservation reserves, especially in large reserves where the home range can be protected within the reserve. Where possible, locate POMAs over nest sites or probable breeding areas based on the occurrence of owlets or adult roosting pairs. Select Powerful Owl sites to meet the POMA target in the following order of priority: 1. confirmed nest trees utilised during the past 5 years 2. confirmed roost trees utilised during the past 5 years 3. repeated sighting or vocalisation during the past 5 years 4. incidental sighting or vocalisation during the past 5 years 5. historic record not reconfirmed in the past 5 years 6. potential habitat area (preferably based on formal analysis and modelling). In State forest, include POMAs in SMZ. | Within the SMZ modify timber harvesting practices to retain high levels of habitat trees consistent with research on Powerful Owl habitat characteristics. | Once regional targets are met, review POMA allocation when records of higher priority are discovered. |

## Fauna - detection based zoning

### Statewide

#### Apply the management actions outlined in Table 4 (Detection based FMZ rules for fauna) below for zoned rare or threatened fauna.

#### Implement FMZ amendments and reviews in accordance with Table 4 (Detection based FMZ rules for fauna) below for new verified rare or threatened fauna records and FMZ amendment requirements outlined in section 2.

Table 4 Detection based FMZ rules for fauna.

| FMA | Common name | Scientific name | Zoning management actions | Management actions | Review |
| --- | --- | --- | --- | --- | --- |
| Bendigo | Barking Owl | Ninox connivens | Establish a SPZ of 3 ha over each verified nesting and roosting site utilised recently and frequently and located outside a Barking Owl Management Area. Establish a SMZ of 250-300 m radius (or equivalent linear area) over each verified nesting and roosting site unless they are already protected. | Apply standard Bendigo FMA SMZ habitat retention prescriptions. |  |
| Bendigo | Grey Goshawk | Accipiter novaehollandiae | Establish a SMZ of 250 m radius over each verified nesting site. | Within the SMZ, avoid timber harvesting, road construction and burning during breeding season. Within 100m of nest trees, exclude timber harvesting, road construction and burning at all times. |  |
| Bendigo | Powerful Owl | Ninox strenua | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Powerful Owl Management Area, unless already protected. | Apply standard Bendigo FMA SMZ habitat retention prescriptions. |  |
| Bendigo | Regent Honeyeater | Xanthomyza phrygia | Establish a SPZ including a 100 m wide buffer around regularly used Regent Honeyeater sites. Surround the SPZ with a 150 m wide SMZ. | Exclude all disturbance activities from the SPZ. Within the SMZ, retain at least 10 habitat trees per hectare. Habitat trees are large, vigorous individuals of the key eucalypt species. |  |
| Bendigo | Square-tailed Kite | Lophoictinia isura | Establish a SMZ of 250 m radius over each verified nesting site. | Within the SMZ, avoid timber harvesting, road construction and burning during breeding season. Within 100m of nest trees, exclude timber harvesting, road construction and burning at all times. |  |
| Bendigo | Squirrel Glider | Petaurus norfolcensis | Establish a SMZ over verified populations. | Prepare a SMZ plan prior to harvesting commencement. Within the SMZ, implement prescriptions that address maintenance of key habitat elements such as nesting hollows, large trees and suitable understorey species. |  |
| Central Highlands MAs | Barking Owl | Ninox connivens | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Barking Owl Management Area. A SMZ is not required where existing zoning or public land tenure provides equivalent or greater protection. | Prepare an SMZ plan prior to harvesting commencement. SMZ plans will protect hollow-bearing trees for owls and prey species along with younger trees that may provide hollows in future. These areas will be managed to maintain habitat capable of supporting adequate populations of birds and arboreal prey mammals to support breeding owls. Exclude timber harvesting in SMZs around nest trees during the breeding season. |  |
| Central Highlands MAs | Common Dunnart | Sminthopsis murina | Establish a SPZ of approximately 20 ha of suitable habitat over verified sites. |  |  |
| Central Highlands MAs | Leadbeater's Possum habitat | Gymnobelideus leadbeateri | Establish a SPZ over areas of Zone 1A habitat where there are more than 10 hollow bearing trees per 3 ha in patches greater than 3 ha. | Ensure Zone 1A habitat is not salvage logged. | Review retained habitat as ash forest areas change in relation to Zone 1A habitat criteria. |
| Central Highlands MAs | Leadbeater's Possum colony | Gymnobelideus leadbeateri | Establish a SPZ of 200 m radius centred on each verified Leadbeater’s Possum colony. |  | The effectiveness of this action in supporting the recovery of the Leadbeater's Possum will be reviewed after two years of surveying (commencing July 2014) or once 200 new colonies are located whose exclusion zones impact the GMZ or SMZ, whichever comes first. |
| Central Highlands MAs | Masked Owl | Tyto novaehollandiae novaehollandiae | Establish a SMZ of 250 m radius over each verified nesting and roosting site utilised recently and frequently and located outside a Masked Owl Management Area, unless already protected. | Protect habitat elements such as old hollow-bearing trees that may be used by Masked Owls and their prey. Exclude timber harvesting, road construction and other activities likely to disturb breeding activity during the breeding season. Protect all trees within a 100m radius of the nest tree from timber harvesting. |  |
| Central Highlands MAs | Powerful Owl | Ninox strenua | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Powerful Owl Management Area, unless already protected. | Outside of Powerful Owl Management Areas, habitat for foraging is provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. Exclude timber harvesting, road construction and other activities likely to disturb breeding activity during the breeding season. Protect all trees within a 100m radius of the nest tree from timber harvesting. | Review zoning associated with individual nesting and roosting sites every 5 years. Retain zones for nesting and roosting sites utilised frequently within the previous 5 years. |
| Central Highlands MAs | Smoky Mouse | Pseudomus fumeus | Establish a SMZ of approximately 100 ha over each verified record incorporating the detection site wherever possible. | Prepare a SMZ plan prior to timber harvesting or road construction commencement, incorporating any relevant information from studies of the species. | Review this strategy when 10 Smoky Mouse SMZ are established or in light of further research regarding the conservation status of the species and its response to disturbance |
| Central Highlands MAs | Sooty Owl | Tyto tenebricosa | Establish a SMZ of 250 m radius over each verified nesting and roosting site utilised recently and frequently and located outside a Sooty Owl Management Area, unless already protected. In these cases, habitat for foraging is already provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. | Exclude timber harvesting, road construction and other activities likely to disturb breeding activity during the breeding season. Protect all trees within a 100 m radius of the nest tree from timber harvesting. |  |
| Central Highlands MAs | Spot-tail Quoll | Dasyurus maculatus | Establish a 500 ha SPZ and a 1000 ha SMZ for all verified Quoll records in State forest throughout Victoria, up to a target of 50 sites. The location of the SPZ and SMZ will based on protecting preferred habitat features for Quolls. Protect verified den and latrine sites by at least a 200 m radius SPZ. The SPZ may include other detection sites, based on habitat quality and the proximity of existing protected habitat. Include detection sites outside the SPZ within the SMZ, unless there are compelling reasons for excluding them (e.g. a record in a clearly unsuitable location for habitat protection, proximity of existing protected habitat etc.). Prioritise site protection for Quolls according to habitat quality, current reservation status of the site, linkage to other reserves and the presence of complementary values. Records within 2 km of each other are generally regarded as the same animal unless proved otherwise. | Exclude the use of threatening poisons within 1 km of a record less than 5 years old. Develop and comply with a SMZ plan | Periodically review the selection of Quoll records afforded SPZ and SMZ protection, to ensure that once targets are reached and as new records accrue or other information becomes available, the network of protected habitat in the FMA is optimal for Quoll conservation. Substitution of protected Quoll sites must consider the extent and quality of habitat and the currency, reliability and type of record. |
| Central Highlands MAs | Spotted Tree Frog | Litoria spenceri | Until the species critical habitat is known, establish a SPZ of 300 m radius over verified locations where frogs have been recorded and where suitable habitat has been mapped (collectively referred to as 'stream habitat'). | Exclude timber harvesting, new road construction and other potentially threatening activities (such as prescribed fire) from the SPZ. |  |
| East Gippsland | Australian Grayling | Prototrodes maraena | Establish a SPZ extending 100 m from each bank for 1 km upstream and 1 km downstream of verified populations. | Avoid constructing new roads in the SPZ. | Review this strategy when 50 significant fish sites have been located. |
| East Gippsland | Barking Owl | Ninox connivens | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Barking Owl Management Area. A SMZ is not required where existing zoning or public land tenure provides equivalent or greater protection. | Prepare an SMZ plan prior to harvesting commencement. SMZ plans will protect hollow-bearing trees for owls and prey species along with younger trees that may provide hollows in future. These areas will be managed to maintain habitat capable of supporting adequate populations of birds and arboreal prey mammals to support breeding owls. |  |
| East Gippsland | Blue Mountains Tree Frog | Litoria citropa | Establish a SPZ extending 100 m from each bank for 500 m upstream and 500 m downstream for verified sites of more than 50 individuals. |  | Review this strategy when 20 sites are established. |
| East Gippsland | Common Brushtail Possum | Trichosurus vulpecula | Establish a SPZ of approximately 100 ha of suitable habitat where verified records report more than 2 individuals per hectare, more than 10 individuals per km or more than 15 individuals per hour of spotlighting, or where substantial populations are located in isolated or unusual habitat. |  |  |
| East Gippsland | Common Bent-wing Bat | Miniopteris schreibersii | Establish a SPZ of 100 m radius over verified breeding and roosting caves, trees and mines and over-wintering sites. |  |  |
| East Gippsland | Cox's Gudgeon | Gobiomorphus coxii | Establish a SPZ extending 100 m from each bank for 1 km upstream and 1 km downstream of verified populations. | Avoid constructing new roads in the SPZ. | Review this strategy when 50 significant fish sites have been located. |
| East Gippsland | Diamond Python | Morelia spilota spilota | Establish a SPZ of 100 ha of suitable habitat for each verified locality of this species. |  | Review this strategy when 50 sites have been located. |
| East Gippsland | Eastern Broad-nosed Bat | Scotorepens orion | Establish a SPZ of approximately 100 ha of older forest over sites where more than 10 individuals are captured in one trapping session. |  | Review this strategy when 20 sites are established. |
| East Gippsland | Eastern Horseshoe Bat | Rhinolophus megaphyllus | Establish a SPZ of 100 m radius over verified breeding and roosting caves, trees and mines and over-wintering sites. |  |  |
| East Gippsland | Eastern Pygmy Possum | Cercartetus nanus | Establish a SPZ of approximately 100 ha of suitable habitat where verified records report more than 5 individuals per standard pitfall line over 5 days, or where substantial populations are located in isolated or unusual habitat. |  |  |
| East Gippsland | Freshwater Herring | Potamalosa richmondia | Establish a SPZ extending 100 m from each bank for 1 km upstream and 1 km downstream of verified populations. | Avoid constructing new roads in the SPZ. | Review this strategy when 50 significant fish sites have been located. |
| East Gippsland | Giant Burrowing Frog | Heleioporus australiacus | Where verified records of Giant Burrowing Frog are located on first-order streams or sites away from streams, protect approximately 50 ha (preferably the entire sub catchment unit) in SPZ. Where verified records of Giant Burrowing Frog are located on second or higher order streams, establish a SPZ of 100 m width each side of the stream for 1 km upstream and downstream of the detection site. | Avoid new roading in the SPZ. | Review this strategy when 50 sites are located in Victoria. |
| East Gippsland | Glossy Black Cockatoo | Calyptorhynchus lathami lathami | Establish a SMZ of 250 m radius over each verified nesting site. | Within the SMZ, exclude timber harvesting, road construction and burning during the breeding season (December to May). Search the surrounding forest for other active nest sites (the species is known to nest in clusters). Within 100 m of nest sites, exclude timber harvesting, road construction and burning at all times. |  |
| East Gippsland | Greater Glider | Petauroides volans | Establish a SPZ of approximately 100 ha of suitable habitat where verified records report more than 2 individuals per hectare, more than 10 individuals per km or more than 15 individuals per hour of spotlighting, or where substantial populations are located in isolated or unusual habitat. |  |  |
| East Gippsland | Grey Goshawk | Accipiter novaehollandiae | Establish a SMZ of 250 m radius over each verified nesting site. | Within the SMZ, avoid timber harvesting, road construction and burning during breeding season. Within 100 m of nest trees, exclude timber harvesting, road construction and burning at all times. |  |
| East Gippsland | Koala | Phascolarctos cinereus | Establish a SPZ of approximately 100 ha of suitable habitat over verified resident populations or substantial populations located in isolated or unusual habitat. |  |  |
| East Gippsland | Large-footed Myotis | Myotis macropus | Establish a SPZ of 100 m radius over verified breeding and roosting caves, trees and mines and over-wintering sites. |  |  |
| East Gippsland | Little Falcon | Falco longipennis | Establish a SMZ of 250 m radius over each verified nesting site. | Within the SMZ, avoid timber harvesting, road construction and burning during breeding season. Within 100 m of nest trees, exclude timber harvesting, road construction and burning at all times. |  |
| East Gippsland | Long -footed Potoroo | Potorous longipes | Establish a SMZ of approximately 150 ha for each Long-footed Potoroo detection site that is outside the Core Protected Area (i.e. within existing SPZ and conservation reserves). As far as possible, SMZ boundaries will follow recognisable landscape features such as ridges, spurs and watercourses. | Within each SMZ, at least one third (50 ha) will be protected from timber harvesting and new roading. This will be known as Long-footed Potoroo Retained Habitat. The LFP Retained Habitat will include the best LFP habitat in the SMZ, which will generally be in gullies and on lower, sheltered slopes. The LFP Retained Habitat may include areas otherwise unavailable for timber harvesting due to restrictions under the Code. The SMZ will also have a general restriction of one third of the total area that can be harvested in any three year period. If more than one coupe is to be harvested in a SMZ in the same year, the coupes must be separated by at least the equivalent of another coupe width. The SMZ, with the LFP Retained Habitat clearly delineated, will be shown as part of the Forest Management Area zoning scheme. If the 150 ha area includes any part of an existing conservation reserve or SPZ, these areas will retain their existing reservation or zoning status but will be considered for inclusion as part of the area of retained habitat. In such cases, the final area designated as SMZ may be correspondingly smaller. |  |
| East Gippsland | Masked Owl | Tyto novaehollandiae novaehollandiae | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) over each verified nesting and roosting site utilised recently and frequently and located outside a Masked Owl Management Area, unless already protected. In these cases, habitat for foraging is already provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. | In SMZ, protect habitat elements such as old hollow-bearing trees that may be used by Masked Owls and their prey. Exclude timber harvesting, road construction and burning during the breeding season. |  |
| East Gippsland | Mountain Galaxias | Galaxias olidus | Establish a SPZ extending 100 m from each bank for 1 km upstream and 1 km downstream of verified populations. | Avoid constructing new roads in the SPZ. | Review this strategy when 50 significant fish sites have been located. |
| East Gippsland | Orbost Spiny Crayfish | Euastacus diversus | Establish a SPZ extending 100 m from each bank for 1 km upstream and 1 km downstream of verified detection sites. | Avoid constructing new roads and stream crossings within the SPZ. Manage nearby regeneration burns to ensure the SPZ is not burnt. | Review this strategy when 20 sites are established. |
| East Gippsland | Peregrine Falcon | Falco peregrinus | Establish a SMZ of 250 m radius over each verified nesting site. | Within the SMZ, avoid timber harvesting, road construction and burning during breeding season. Within 100 m of nest trees, exclude timber harvesting, road construction and burning at all times. |  |
| East Gippsland | Powerful Owl | Ninox strenua | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Powerful Owl Management Area, unless already protected. | Within the SMZ, exclude timber harvesting, road construction and burning during the breeding season. |  |
| East Gippsland | Rich reptile and amphibian sites |  | Establish a SPZ of at least 10 ha at sites verified as being particularly rich in reptiles or amphibians |  |  |
| East Gippsland | Significant fish |  | Establish a SPZ extending 100 m from each bank for 1 km upstream and 1 km downstream of verified sites supporting 3 or more threatened fish species, or verified sites supporting 6 or more native freshwater fish species. | Avoid constructing new roads in the SPZ. | Review this strategy when 50 significant fish sites have been located. |
| East Gippsland | Sooty Owl | Tyto tenebricosa | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) over each verified nesting and roosting site utilised recently and frequently and located outside a Sooty Owl Management Area, unless already protected. In these cases, habitat for foraging is already provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. | Prepare an SMZ plan specifying the prescriptions to be applied within the Sooty Owl SMZ. Exclude timber harvesting, road construction and burning during the breeding season. |  |
| East Gippsland | Southern Barred Frog | Mixophyes balbus | Where verified records of Southern Barred Frog are located on first-order streams or sites away from streams, protect approximately 50 ha (preferably the entire sub catchment unit) in SPZ. Where verified records of Southern Barred Frog are located on second or higher order streams, establish a SPZ of 100 m width each side of the stream for 1 km upstream and downstream of the detection site. | Avoid new roading in the SPZ. | Review this strategy when an Action Statement is prepared. |
| East Gippsland | Spot-tail Quoll | Dasyurus maculatus | Establish a 500 ha SPZ and a 1000 ha SMZ for all verified Quoll records in State forest up to a target number of sites (to be determined). The location of the SPZ and SMZ will based on protecting preferred habitat features for Quolls. The SPZ will include known den and latrine sites (protected by at least a 200 m radius), and may include other detection sites, based on habitat quality and the proximity of existing protected habitat. Detection sites not included in the SPZ will generally be included within the SMZ, unless there are compelling reasons for excluding them (e.g. a record in a clearly unsuitable location for habitat protection, proximity of existing protected habitat etc.). Site protection for Quolls will be prioritised according to habitat quality, current reservation status of the site, linkage to other reserves and the presence of complementary values. Records within 2 km of each other will be generally regarded as the same animal unless proved otherwise. | Prepare a SMZ plan prior to harvesting commencement considering available information on Quoll and prey habitat preferences and use. | Periodically review the selection of Quoll records to be afforded protection to ensure that, once targets are reached and as new records accrue or other information becomes available, the network of protected habitat in the FMA is optimal for Quoll conservation. To assist this review, develop guidelines for the substitution of protected Quoll sites based on the extent and quality of habitat and on the currency, reliability and type of record. |
| East Gippsland | Square-tailed Kite | Lophoictinia isura | Establish a SMZ of 250 m radius over each verified nesting site. | Within the SMZ, avoid timber harvesting, road construction and burning during breeding season. Within 100 m of nest trees, exclude timber harvesting, road construction and burning at all times. |  |
| East Gippsland | White-bellied Sea-eagle | Haliaeetus leucogaster | Establish a SMZ of 250 m radius over each verified nesting site. | Within the SMZ, avoid timber harvesting, road construction and burning during breeding season. Within 100 m of nest trees, exclude timber harvesting, road construction and burning at all times. |  |
| East Gippsland | Yellow-bellied Glider | Petaurus australis | Establish a SPZ of approximately 100 ha of suitable habitat where verified records report more than 0.2 individuals per hectare, more than 5 individuals per km or more than 7 individuals per hour of spotlighting, or where substantial populations are located in isolated or unusual habitat. |  |  |
| East Gippsland | Yellow-bellied Sheathtail-bat | Saccolaimus flaviventris | Establish a SPZ of approximately 100 ha of older forest over sites where more than 10 individuals are captured in one trapping session. |  | Review this strategy when 20 sites are established. |
| Gippsland FMAs | Alpine Tree Frog | Litoria verreauxii alpina | Establish a SPZ of approximately 50 ha over each verified site. |  |  |
| Gippsland FMAs | Alpine Water Skink | Eulamprus kosiuskoi | Establish a SMZ over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Barking Owl | Ninox connivens | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Barking Owl Management Area. A SMZ is not required where existing zoning or public land tenure provides equivalent or greater protection. | Prepare an SMZ plan prior to harvesting commencement. SMZ plans will protect hollow-bearing trees for owls and prey species along with younger trees that may provide hollows in future. These areas will be managed to maintain habitat capable of supporting adequate populations of birds and arboreal prey mammals to support breeding owls. |  |
| Gippsland FMAs | Eastern Bent-wing Bat | Miniopteris schreibersii oceanensis | Establish a SPZ of 100 m radius over verified breeding and roosting caves and mines, and over-wintering sites. |  |  |
| Gippsland FMAs | Eastern Horseshoe Bat | Rhinolophus megaphyllus | Establish a SPZ of 100 m radius over all breeding and roosting caves and mines and known overwintering sites. |  |  |
| Gippsland FMAs | Giant Burrowing Frog | Heleioporus australiacus | Where verified records of Giant Burrowing Frog are located on first-order streams or sites away from streams, protect approximately 50 ha (preferably the entire sub catchment unit) in SPZ. Where verified records of Giant Burrowing Frog are located on second or higher order streams, establish a SPZ of 100 m width each side of the stream for 1km upstream and downstream of the detection site. Note: For the purposes of this prescription, a first order stream is the headwaters of a catchment and is the smallest stream mapped on the 1:100 000 Natmap series. Second order streams are the next level of stream further down the catchment. For first order streams the size of each catchment will vary, however target size is approximately 50 ha. For off stream records establish a SPZ of 50ha over the record or equivalent area of suitable habitat nearby. | Avoid new roading in the SPZ. | Review this strategy when 50 sites are located in Victoria, or in light of further research on this species. |
| Gippsland FMAs | Giant Gippsland Earthworm | Megascolides australis | Establish a SMZ over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. Develop and comply with a SMZ plan |  |
| Gippsland FMAs | Glossy Black Cockatoo | Calyptorhynchus lathami lathami | Establish a SMZ of 250 m radius over verified nesting sites. | Within the SMZ, exclude timber harvesting, road construction and burning during the breeding season (December to May). Search the surrounding forest for other active nest sites (the species is known to nest in clusters). Within 100m of nest sites, exclude timber harvesting, road construction and burning at all times. |  |
| Gippsland FMAs | Grey Goshawk | Accipiter novaehollandiae | Establish a SMZ of 250 m radius over verified nesting sites. | Within the SMZ, avoid timber harvesting, road construction and burning during breeding season. Within 100 m of nest trees, exclude timber harvesting, road construction and burning at all times. |  |
| Gippsland FMAs | Grey-headed Flying-fox | Pteropus poliocephalus | Consider establishing a SPZ of 100 m radius for a roost site that has a pattern of regular seasonal use. |  |  |
| Gippsland FMAs | Large-footed Myotis | Myotis macropus | Establish a SPZ of 100 m radius over verified breeding and roosting caves, trees and mines and over-wintering sites. |  | Review this strategy when 20 sites are established or when significant information has been obtained from further research. |
| Gippsland FMAs | Masked Owl | Tyto novaehollandiae novaehollandiae | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Masked Owl Management Area, unless already protected. In these cases, habitat for foraging is already provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. | In SMZ, protect habitat elements such as old hollow-bearing trees that may be used by Masked Owls and their prey. Avoid burning operations within 250 m of known nest sites during the breeding season (1 May to 30 November). |  |
| Gippsland FMAs | New Holland Mouse | Pseudomys novaehollandiae | Establish a SPZ of 100 ha of preferred habitat including the detection site wherever possible. |  |  |
| Gippsland FMAs | Powerful Owl | Ninox strenua | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site, unless already protected. | Outside of Powerful Owl Management Areas, habitat for foraging is provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. Avoid burning operations within 250m of known nest sites during the breeding season (1 May to 30 November). Develop and comply with a SMZ plan |  |
| Gippsland FMAs | Smoky Mouse | Pseudomus fumeus | Establish a SMZ of approximately 100 ha over each verified record incorporating the detection site wherever possible. | Prepare a SMZ plan prior to timber harvesting or road construction commencement, incorporating any relevant information from studies of the species. | Review this strategy when 10 Smoky Mouse SMZ are established or in light of further research regarding the conservation status of the species and its response to disturbance |
| Gippsland FMAs | Sooty Owl | Tyto tenebricosa | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Sooty Owl Management Area, unless already protected. In these cases, habitat for foraging is already provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. | Prepare an SMZ plan specifying the prescriptions to be applied within the Sooty Owl SMZ. Avoid burning within 250 m of nest sites during the breeding season (1 May - 30 November) |  |
| Gippsland FMAs | South Gippsland Spiny Crayfish | Euastacus neodiversus |  | Develop management actions to protect South Gippsland Crayfish habitat at verified sites in State forest. |  |
| Gippsland FMAs | Spot-tail Quoll | Dasyurus maculatus | Establish a SPZ of approximately 500 ha for each verified record including the detection site where appropriate. Include areas of undisturbed mature forest, riparian areas and rocky outcrops in the SPZ where possible. In addition to the SPZ, establish a SMZ of approximately 1 000 ha contiguous to the SPZ. Protect verified den and latrine sites by at least a 200 m radius SPZ where they are not otherwise protected. | Prepare SMZ plan prior to harvesting commencement. The objectives of the SMZ will be to maintain habitat for both arboreal and non-arboreal Spot-tail Quoll prey, while providing continuing opportunities for timber harvesting. Harvesting within the SMZ may comprise single tree or group selection or small patch fellings over the entire area. Alternatively, carefully sited and scheduled seed tree or clearfall harvesting may be used, ensuring, in either case that important prey habitat components such as hollow-bearing trees are retained. Where clearfall or seed tree harvesting systems are proposed, harvesting operations may through time extend over the entire SMZ but must be scheduled to ensure at least 500 ha of suitable prey habitat is available at any point in time (in addition to the SPZ). The adoption of harvesting strategies for SMZs should have regard to the assessed significance of the zone as Spot-tailed Quoll habitat, existing patterns of public land use and forest zoning, the silvicultural characteristics of forest types in the zone, the proportion of the zone which is unproductive for sawlog harvesting and the timber resource values of the productive areas. | Review the selection of Quoll records afforded SPZ and SMZ protection when 10 zones have been established in State forest or when significant information has been obtained from further research. the purpose of the review is to ensure that once targets are reached and as new records accrue or other information becomes available, the network of protected habitat in the FMA is optimal for Quoll conservation. Substitution of protected Quoll sites must consider the extent and quality of habitat and the currency, reliability and type of record. |
| Gippsland FMAs | Spotted Tree Frog | Litoria spenceri | Until the species critical habitat is known, establish a SPZ of 300 m radius over all verified frog sites and mapped habitat areas. Establish a SMZ of a further 700m either side of the SPZ and 1 km upstream or to the ridge top (if the stream is less than 1 km long) along all streams that flow into the frog habitat. | Exclude timber harvesting, new road construction and other potentially threatening activities (such as prescribed fire) from the SPZ. Within the SMZ: - do not construct new stream crossings - ensure any new road (and the fill slope toe of any new road) is located at least 50 m from any stream, unless site specific sediment management operations are put in place to prevent sediments entering perennial and ephemeral streams in the management zone - apply buffer and filter widths set out in section 3.3 Table 10 (Minimum widths in metres for buffer strips and filter strips applicable to various waterway categories, in relation to water quality risk and slope for coupes in catchments up to 1 km upstream of known Spotted Tree Frog sites or coupes in Barred Galaxias and Mountain Galaxias SMZs) of the Management Standards and Procedures. - give particular attention to suspension of logging and snig track drainage during wet weather - excavate 50 cm deep pits below the original track surface and immediately upslope of bars constructed to drain snig tracks and other tracks - ensure landings are constructed, or are drained to dispose of discharge at least 50m from the upslope edge of buffers and filters. |  |
| Gippsland FMAs | Square-tailed Kite | Lophoictinia isura | Establish a SMZ of 250 m radius over verified nesting sites. | Within the SMZ, avoid timber harvesting, road construction and burning during breeding season. Within 100 m of nest trees, exclude timber harvesting, road construction and burning at all times. |  |
| Gippsland FMAs | Warragul Burrowiing Crayfish | Engaeus sternalis | Establish a SMZ over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | White-bellied Sea-eagle | Haliaeetus leucogaster | Establish a SMZ of 500 m radius over verified nesting sites. | Within the SMZ, avoid timber harvesting, road construction and burning during breeding season. Within 100 m of nest trees, exclude timber harvesting, road construction and burning at all times. |  |
| Gippsland FMAs | Yellow-bellied Sheathtail-bat | Saccolaimus flaviventris | Establish a SPZ of 100 m radius over verified roost sites with patterns of regular seasonal use. Establish a further SMZ of 100 m radius around the SPZ for verified sites of active colonies. | Schedule timber harvesting, roading and fuel reduction burning operations within the SMZ to minimise disturbance to colonies. |  |
| Mid Murray | Bandy Bandy | Vermicella annulata | Establish a SMZ of approximately 50 ha over verified sites, where practicable incorporating the detection site. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Mid Murray | Barking Owl | Ninox connivens | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Barking Owl Management Area. A SMZ is not required where existing zoning or public land tenure provides equivalent or greater protection. | Prepare an SMZ plan prior to harvesting commencement. SMZ plans will protect hollow-bearing trees for owls and prey species along with younger trees that may provide hollows in future. These areas will be managed to maintain habitat capable of supporting adequate populations of birds and arboreal prey mammals to support breeding owls. Exclude all potentially disturbing activities during the breeding season (1 July - 31 December). |  |
| Mid Murray | Hooded Scaly-foot | Pygopus schraderi | Establish a SMZ of approximately 50 ha over verified populations, where practicable incorporating the detection site. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Mid Murray | Powerful Owl | Ninox strenua | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site, unless already protected. | Outside of Powerful Owl Management Areas, habitat for foraging is provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. Exclude all potentially disturbing activities during the breeding season (1 April to 30 September). |  |
| Mid Murray | Regent Honeyeater | Xanthomyza phrygia | Establish a SPZ including a 100 m wide buffer around regularly used Regent Honeyeater sites. Surround the SPZ with a 150 m wide SMZ. | Exclude all disturbance activities from the SPZ. Within the SMZ, retain at least 10 habitat trees per hectare. Habitat trees are large, vigorous individuals of the key eucalypt species. |  |
| Mid Murray | Southern Myotis | Myotis macropus | Establish a SMZ of 250 m radius over verified breeding and roosting populations. | Within the SMZ, exclude all potentially disturbing activities during breeding season. Breeding season is 1 October to 31 March. Within 100m of the breeding and roosting trees, exclude timber harvesting at all times. |  |
| Mid Murray | Superb Parrot | Polytelis swainsonii | Establish a SMZ of 250 m radius over verified nest trees. | Within the SMZ, exclude all potentially disturbing activities during breeding season. Breeding season is 1 September to 31 December. Within 100m of the breeding and roosting trees, exclude timber harvesting at all times. | Review this strategy statewide every 10 years from 2003, or if the total area of buffer zones in Victorian State forests exceeds 200 ha, whichever occurs first. |
| Mid Murray | White-bellied Sea-eagle | Haliaeetus leucogaster | Establish a SMZ of 250 m radius over verified nesting populations. | Within the SMZ, avoid timber harvesting, road construction and burning during breeding season. Breeding season is 1 May to 31 December. Within 100m of nest trees, exclude timber harvesting, road construction and burning at all times. |  |
| Mid Murray | Woodland Blind Snake | Ramphotyphlops proximus | Establish a SMZ of approximately 50 ha over verified populations, where practicable incorporating the detection site. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Midlands | Barking Owl | Ninox connivens | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Barking Owl Management Area. A SMZ is not required where the site is already protected. | Prepare an SMZ plan prior to harvesting commencement. SMZ plans will protect hollow-bearing trees for owls and prey species along with younger trees that may provide hollows in future. These areas will be managed to maintain habitat capable of supporting adequate populations of birds and arboreal prey mammals to support breeding owls. |  |
| Midlands | Common Bent-wing Bat | Miniopteris schreibersii | Establish a SMZ of 100 m radius over verified roosting and over-wintering sites. | Exclude timber harvesting, road construction and other activities likely to disturb over-wintering bats between 1 May to 31 October. |  |
| Midlands | Common Dunnart | Sminthopsis murina | Establish a SPZ of approximately 20 ha of suitable habitat over verified sites. |  | Review this strategy when 50 sites have been established in State forest. |
| Midlands | Four-toed Skink | Hemiergis peronii | Establish a SMZ over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Midlands | Grey Goshawk | Accipiter novaehollandiae | Establish a SMZ of 250 m radius over verified nesting sites. | Within the SMZ, avoid timber harvesting, road construction and burning activities during breeding season. Breeding season is 1 July to 30 November. Within 100 m of nest trees, exclude timber harvesting, road construction and burning at all times. | Review this strategy when 10 sites have been established in State forest. |
| Midlands | Masked Owl | Tyto novaehollandiae novaehollandiae | Establish a SMZ of 250 m radius over verified nesting and roosting sites utilised recently and frequently and located outside a Masked Owl Management Area. | Protect habitat elements such as old hollow-bearing trees that may be used by Masked Owls and their prey. Exclude timber harvesting, road construction and other activities likely to disturb breeding activity during the breeding season. Protect all trees within a 100 m radius of the nest tree from timber harvesting. |  |
| Midlands | Mountain Dragon | Tympanocryptis diemensis | Establish a SMZ over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Midlands | Powerful Owl | Ninox strenua | Establish a SMZ of 250 m radius over verified nesting and roosting sites, unless already protected. | Outside of Powerful Owl Management Areas, habitat for foraging is provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. Exclude timber harvesting, road construction and other activities likely to disturb breeding activity during the breeding season. Protect all trees within a 100 m radius of the nest tree from timber harvesting. |  |
| Midlands | Regent Honeyeater | Xanthomyza phrygia | Establish a SPZ including a 100 m buffer around regularly used Regent Honeyeater sites. |  |  |
| Midlands | Samphire Skink | Morethia adelaidensis | Establish a SMZ over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Midlands | Spot-tail Quoll | Dasyurus maculatus | Establish a 500 ha SPZ and a 1000 ha SMZ for all verified Quoll records in State forest, up to a target of 10 sites. The location of the SPZ and SMZ will based on protecting preferred habitat features for Quolls. Protect verified den and latrine sites by at least a 200 m radius SPZ. The SPZ may include other detection sites, based on habitat quality and the proximity of existing protected habitat. Include detection sites outside the SPZ within the SMZ, unless there are compelling reasons for excluding them (e.g. a record in a clearly unsuitable location for habitat protection, proximity of existing protected habitat etc.). Prioritise site protection for Quolls according to habitat quality, current reservation status of the site, linkage to other reserves and the presence of complementary values. Records within 2 km of each other are generally regarded as the same animal unless proved otherwise. | Exclude the use of poisons such as 1080 within 2 km of Quoll records. Develop and comply with a SMZ plan | Periodically review the selection of Quoll records afforded SPZ and SMZ protection, to ensure that once targets are reached and as new records accrue or other information becomes available, the network of protected habitat in the FMA is optimal for Quoll conservation. Substitution of protected Quoll sites must consider the extent and quality of habitat and the currency, reliability and type of record. |
| Midlands | Spotted Galaxis | Galaxius truttaceaus | Establish a SMZ over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. Develop and comply with a SMZ plan |  |
| Midlands | Square-tailed Kite | Lophoictinia isura | Establish a SMZ of 250 m radius over verified nesting sites. | Within the SMZ, avoid timber harvesting, road construction and burning activities during breeding season. Breeding season is 1 July to 30 November. Within 100m of nest trees, exclude timber harvesting, road construction and burning at all times. | Review this strategy when 10 sites have been established in State forest |
| Midlands | Squirrel Glider | Petaurus norfolcensis | Establish a SMZ of approximately 500 ha of suitable habitat over verified populations. | Prepare a SMZ plan prior to harvesting commencement. Within the SMZ, implement prescriptions that address maintenance of key habitat elements such as nesting hollows, large trees and suitable understorey species (in particular Silver Wattle Acacia dealbata). Exclude timber harvesting from a core area of approximately 100 ha. | Review this strategy when 10 sites have been established in State forest |
| Midlands | Wedge-tailed Eagle | Aquila audax | Establish a SMZ of 250 m radius over verified nesting sites. | Within the SMZ, avoid timber harvesting, road construction and burning activities during breeding season. Breeding season is 1 July to 30 November. Within 100m of nest trees, exclude timber harvesting, road construction and burning at all times. | Review this strategy when 10 sites have been established in State forest |
| Midlands | Woodland Blind Snake | Ramphotyphlops proximus | Establish a SMZ over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. Develop and comply with a SMZ plan |  |
| North East FMAs | Alpine She-oak Skink | Cyclodomorphus praealtus |  | Develop management actions to protect Alpine She-oak Skink habitat at verified sites. Develop and comply with a SMZ plan |  |
| North East FMAs | Alpine Tree Frog | Litoria verreauxii alpina | Establish a SMZ over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. Develop and comply with a SMZ plan |  |
| North East FMAs | Alpine Water Skink | Eulamprus kosiuskoi | Establish a SMZ over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Apostlebird | Struthidea cinerea | Establish a SMZ over verified sites of resident populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Bandy Bandy | Vermicella annulata | Establish a SPZ of approximately 50 ha over each verified site, where practicable incorporating the detection site. Review the strategy when 20 SPZ are established. |  | Review the strategy when 20 SPZ are established. |
| North East FMAs | Barking Owl | Ninox connivens | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Barking Owl Management Area. A SMZ is not required where existing zoning or public land tenure provides equivalent or greater protection. | Prepare an SMZ plan prior to harvesting commencement. SMZ plans will protect hollow-bearing trees for owls and prey species along with younger trees that may provide hollows in future. These areas will be managed to maintain habitat capable of supporting adequate populations of birds and arboreal prey mammals to support breeding owls. |  |
| North East FMAs | Carpet Python | Morelia spilota variegata | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Long -footed Potoroo | Potorous longipes | Establish a SMZ of approximately 150 ha for each Long-footed Potoroo detection site that is outside the Core Protected Area (i.e. within existing SPZ and conservation reserves). As far as possible, SMZ boundaries will follow recognisable landscape features such as ridges, spurs and watercourses. | Within each SMZ, at least one third (50 ha) will be protected from timber harvesting and new roading. This will be known as Long-footed Potoroo Retained Habitat. The LFP Retained Habitat will include the best LFP habitat in the SMZ, which will generally be in gullies and on lower, sheltered slopes. The LFP Retained Habitat may include areas otherwise unavailable for timber harvesting due to restrictions under the Code. The SMZ will also have a general restriction of one third of the total area that can be harvested in any three year period. If more than one coupe is to be harvested in a SMZ in the same year, the coupes must be separated by at least the equivalent of another coupe width. The SMZ, with the LFP Retained Habitat clearly delineated, will be shown as part of the Forest Management Area zoning scheme. If the 150 ha area includes any part of an existing conservation reserve or SPZ, these areas will retain their existing reservation or zoning status but will be considered for inclusion as part of the area of retained habitat. In such cases, the final area designated as SMZ may be correspondingly smaller. |  |
| North East FMAs | Masked Owl | Tyto novaehollandiae novaehollandiae | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Masked Owl Management Area, unless already protected. In these cases, habitat for foraging is already provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. | In SMZ, protect habitat elements such as old hollow-bearing trees that may be used by Masked Owls and their prey. |  |
| North East FMAs | Mountain Pygmy Possum | Burramys parvus | Establish a SMZ over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Powerful Owl | Ninox strenua | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site, unless already protected. | Prepare SMZ plan prior to harvesting commencement. Outside of Powerful Owl Management Areas, habitat for foraging is provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. |  |
| North East FMAs | Regent Honeyeater | Xanthomyza phrygia | Establish a SPZ including a 100 m wide buffer around regularly used Regent Honeyeater sites. Surround the SPZ with a 150 m wide SMZ. | Exclude all disturbance activities from the SPZ. Within the SMZ, retain at least 10 habitat trees per hectare. Habitat trees are large, vigorous individuals of the key eucalypt species. |  |
| North East FMAs | Smoky Mouse | Pseudomus fumeus | Establish a SMZ of approximately 100 ha over verified records incorporating the detection site wherever possible. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations or road construction commencement, incorporating any relevant information from studies of the species. | Review this strategy when 10 Smoky Mouse SMZ are established or in light of further research regarding the conservation status of the species and its response to disturbance. |
| North East FMAs | Sooty Owl | Tyto tenebricosa | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Sooty Owl Management Area, unless already protected. In these cases, habitat for foraging is already provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. | Prepare an SMZ plan specifying the prescriptions to be applied within the Sooty Owl SMZ. |  |
| North East FMAs | Southern Bent-wing Bat | Miniopteris schreibersii | Establish a SPZ of 100 m radius over verified breeding and roosting caves and mines, and over-wintering sites. |  |  |
| North East FMAs | Southern Horseshoe Bat | Rhinolophus megaphyllus | Establish a SPZ of 100 m radius over verified breeding and roosting caves and mines, and over-wintering sites. |  |  |
| North East FMAs | Southern Myotis | Myotis macropus | Establish a SPZ of 300 m radius over verified breeding and roosting sites. |  | Review this strategy when 20 sites are established or when significant information has been obtained from further research. |
| North East FMAs | Spot-tail Quoll | Dasyurus maculatus | Establish a SPZ of approximately 500 ha for each verified record. Include areas of undisturbed mature forest, riparian areas and rocky outcrops in the SPZ where possible. In addition to the SPZ, establish a SMZ of approximately 1000 ha contiguous to the SPZ. Protect verified den and latrine sites by at least a 200 m radius SPZ where they are not otherwise protected. | Prepare SMZ plan prior to harvesting commencement. The objectives of the SMZ will be to maintain habitat for both arboreal and non-arboreal Spot-tail Quoll prey, while providing continuing opportunities for timber harvesting. Harvesting within the SMZ may comprise selective harvesting operations over the entire area, or carefully sited and scheduled seed tree or clearfall harvesting, ensuring, in either case that important prey habitat components such as hollow-bearing trees are retained. Where clearfall or seed tree harvesting systems are proposed, harvesting operations may through time extend over the entire SMZ. However harvesting must be scheduled to ensure at least 500 ha of suitable prey habitat is available at any point in time (in addition to the SPZ). The adoption of harvesting strategies for SMZs should have regard to the assessed significance of the zone as Spot-tailed Quoll habitat, existing patterns of public land use and forest zoning, the silvicultural characteristics of forest types in the zone, the proportion of the zone which is unproductive for sawlog harvesting and the timber resource values of the productive areas. | Review the selection of Quoll records afforded SPZ and SMZ protection when 10 zones have been established in State forest or the Action Statement is revised. The purpose of the review is to ensure that once targets are reached and as new records accrue or other information becomes available, the network of protected habitat in the FMA is optimal for Quoll conservation. Substitution of protected Quoll sites must consider the extent and quality of habitat and the currency, reliability and type of record. |
| North East FMAs | Spotted Tree Frog | Litoria spenceri | Until the species critical habitat is known, establish a SPZ of 300 m radius over all verified frog sites and mapped habitat areas. Establish a SMZ of a further 700 m either side of the SPZ and 1 km upstream or to the ridge top (if the stream is less than 1 km long) along all streams that flow into the frog habitat. | Exclude timber harvesting, new road construction and other potentially threatening activities (such as prescribed fire) from the SPZ. Within the SMZ: - do not construct new stream crossings - ensure any new road (and the fill slope toe of any new road) is located at least 50 m from any stream, unless site specific sediment management operations are put in place to prevent sediments entering perennial and ephemeral streams in the management zone - apply buffer and filter widths set out in section 3.3 Table 10 (Minimum widths in metres for buffer strips and filter strips applicable to various waterway categories, in relation to water quality risk and slope for coupes in catchments up to 1 km upstream of known Spotted Tree Frog sites or coupes in Barred Galaxias and Mountain Galaxias SMZs) of the Management Standards and Procedures. - give particular attention to suspension of logging and snig track drainage during wet weather - excavate 50 cm deep pits below the original track surface and immediately upslope of bars constructed to drain snig tracks and other tracks - ensure landings are constructed, or are drained to dispose of discharge at least 50 m from the upslope edge of buffers and filters. |  |
| North East FMAs | Squirrel Glider | Petaurus norfolcensis | Establish a SMZ of approximately 500 ha of suitable habitat over verified populations. | Prepare a SMZ plan prior to harvesting commencement. Within the SMZ, implement prescriptions that address maintenance of key habitat elements such as nesting hollows, large trees and suitable understorey species (in particular Silver Wattle Acacia dealbata). Exclude timber harvesting from a core area of approximately 100 ha. | Review this strategy when 10 sites are established in State forest. |
| North East FMAs | Swift Parrot | Lathamus discolor | Establish a SMZ for verified resident populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations or prescribed burning activities. |  |
| North East FMAs | White-bellied Sea-eagle | Haliaeetus leucogaster | Establish a SMZ over verified resident populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Woodland Blind Snake | Ramphotyphlops proximus | Establish a SPZ of approximately 50 ha over each verified site, where practicable incorporating the detection site. Review the strategy when 20 SPZ are established. |  | Review the strategy when 20 SPZ are established. |
| Otway | Masked Owl | Tyto novaehollandiae novaehollandiae | Establish a SPZ of 3 ha over each verified nesting and roosting site utilised recently and frequently and located outside a Masked Owl Management Area, unless already protected. In these cases, habitat for foraging is already provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. | Protect habitat elements such as old hollow-bearing trees that may be used by Masked Owls and their prey for 250-300 m radius (or equivalent linear area) from each verified nesting and roosting site protected by a 3ha Masked Owl SPZ. |  |
| Otway | Powerful Owl | Ninox strenua | Establish a SPZ of 3 ha over each verified nesting and roosting site, unless already protected. |  |  |
| Otway | Spot-tail Quoll | Dasyurus maculatus | Establish a SPZ of approximately 500 ha for each verified record. The location of the SPZ will be based on protecting preferred habitat features for Quolls. The SPZ will include known den and latrine sites (protected by at least a 200 m radius), and may include other detection sites, based on habitat quality and the proximity of existing protected habitat. Site protection for Quolls will be prioritised according to habitat quality, current reservation status of the site, linkage to other reserves and the presence of complementary values. Records within 2 km of each other will be generally regarded as the same animal unless proved otherwise. |  | Periodically review the selection of Quoll records afforded SPZ protection particularly when the Action Statement is revised. The purpose of the review is to ensure that once targets are reached and as new records accrue or other information becomes available, the network of protected habitat in the FMA is optimal for Quoll conservation. Substitution of protected Quoll sites must consider the extent and quality of habitat and the currency, reliability and type of record. |
| Otway | Yellow-bellied Glider | Petaurus australis | Establish a SPZ of approximately 100 ha of suitable habitat where verified records report more than 0.2 individuals per hectare, more than 5 individuals per km or more than 7 individuals per hour of spotlighting, or where substantial populations are located in isolated or unusual habitat. |  |  |
| Portland Horsham | Australian Grayling | Prototrodes maraena | Establish a SPZ extending 100 m from each bank for 1 km upstream and 1 km downstream of verified populations. | Avoid constructing new roads in the SPZ. | Review this strategy when 50 significant fish sites have been located. |
| Portland Horsham | Barking Owl | Ninox connivens | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Barking Owl Management Area. A SMZ is not required where existing zoning or public land tenure provides equivalent or greater protection. | Prepare an SMZ plan prior to harvesting commencement. SMZ plans will protect hollow-bearing trees for owls and prey species along with younger trees that may provide hollows in future. These areas will be managed to maintain habitat capable of supporting adequate populations of birds and arboreal prey mammals to support breeding owls. Exclude all potentially disturbing activities during the breeding season (1 July - 31 December). |  |
| Portland Horsham | Black Falcon | Falco subniger | Establish a SMZ of 250 m radius over each verified nesting site. | Within the SMZ, avoid timber harvesting, road construction and burning during breeding season. Within 100 m of nest trees, exclude timber harvesting and road construction and avoid burning at all times. |  |
| Portland Horsham | Bush Stone-curlew | Burhinus gralllarius | Establish a SMZ of 250 m radius over each verified nesting site. | Within the SMZ, avoid any burning during breeding season. At all other times retain any logs and other fallen timber in the area. |  |
| Portland Horsham | Common Dunnart | Sminthopsis murina | Establish a SPZ of approximately 20 ha of suitable habitat over verified sites. |  |  |
| Portland Horsham | Eltham Copper Butterfly | Paralucia pyrodiscus lucida | Establish a SMZ over verified populations. | Protect the host plant Bursaria spinosa (Sweet Bursaria) within 250 m of any records of this species. |  |
| Portland Horsham | Glenelg Freshwater Mussel | Hyridella glenelgensis | Establish a SPZ extending 100 m from each bank for 1 km upstream and 1 km downstream of verified populations. | Disturbance that could impact on water quality must be avoided within SPZ. |  |
| Portland Horsham | Golden Sun Moth | Synemon plana | Establish a SMZ over verified populations. | Detailed planning is required to ensure population is protected from timber harvesting. |  |
| Portland Horsham | Grey Goshawk | Accipiter novaehollandiae | Establish a SMZ of 250 m radius over each verified nesting site. | Within the SMZ, avoid timber harvesting, road construction and burning during breeding season. Within 100m of nest trees, exclude timber harvesting and road construction and avoid burning at all times. |  |
| Portland Horsham | Malleefowl | Leipoa ocellata | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius over each verified active nesting mound. |  |  |
| Portland Horsham | Masked Owl | Tyto novaehollandiae novaehollandiae | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site utilised recently and frequently and located outside a Masked Owl Management Area, unless already protected. In these cases, habitat for foraging is already provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. | In SMZ, protect habitat elements such as old hollow-bearing trees that may be used by Masked Owls and their prey. |  |
| Portland Horsham | Powerful Owl | Ninox strenua | Establish a SPZ of 3 ha and a SMZ of 250-300 m radius (or equivalent linear area) around each verified nesting and roosting site, unless already protected. | Outside of Powerful Owl Management Areas, habitat for foraging is provided in areas excluded from timber harvesting by general prescription including wildlife corridors, steep areas and unmerchantable areas and areas protected for other management purposes. Exclude all potentially disturbing activities during the breeding season (1 April to 30 September). |  |
| Portland Horsham | Regent Honeyeater | Xanthomyza phrygia | Establish a SPZ including a 100 m wide buffer around regularly used Regent Honeyeater sites. Surround the SPZ with a 150 m wide SMZ. | Exclude all disturbance activities from the SPZ. Within the SMZ, retain at least 10 habitat trees per hectare. Habitat trees are large, vigorous individuals of the key eucalypt species. |  |
| Portland Horsham | Samphire Skink | Morethia adelaidensis | Establish a SMZ over verified populations. | Detailed planning is required to ensure population is protected from timber harvesting. |  |
| Portland Horsham | Smoky Mouse | Pseudomus fumeus | Establish a SMZ of approximately 100 ha over verified records incorporating the detection site wherever possible. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations or road construction commencement, incorporating any relevant information from studies of the species. | Review this strategy when 10 Smoky Mouse SMZ are established or in light of further research regarding the conservation status of the species and its response to disturbance. |
| Portland Horsham | Spot-tail Quoll | Dasyurus maculatus | Establish a 500 ha SPZ and a 1000 ha SMZ for all verified Quoll records in State forest, up to a target of 10 sites. The location of the SPZ and SMZ will based on protecting preferred habitat features for Quolls. Protect verified den and latrine sites by at least a 200 m radius SPZ. The SPZ may include other detection sites, based on habitat quality and the proximity of existing protected habitat. Include detection sites outside the SPZ within the SMZ, unless there are compelling reasons for excluding them (e.g. a record in a clearly unsuitable location for habitat protection, proximity of existing protected habitat etc.). Prioritise site protection for Quolls according to habitat quality, current reservation status of the site, linkage to other reserves and the presence of complementary values. Records within 2 km of each other are generally regarded as the same animal unless proved otherwise. | Exclude the use of poisons such as 1080 within 2 km of Quoll records. | Periodically review the selection of Quoll records afforded SPZ and SMZ protection, to ensure that once targets are reached and as new records accrue or other information becomes available, the network of protected habitat in the FMA is optimal for Quoll conservation. Substitution of protected Quoll sites must consider the extent and quality of habitat and the currency, reliability and type of record. |
| Portland Horsham | Squirrel Glider | Petaurus norfolcensis | Establish a SMZ of approximately 500 ha of suitable habitat over verified populations. | Prepare a SMZ plan prior to harvesting commencement. Within the SMZ, implement prescriptions that address maintenance of key habitat elements such as nesting hollows, large trees and suitable understorey species (in particular Silver Wattle Acacia dealbata). Exclude timber harvesting from a core area of approximately 100 ha. | Review this strategy when 10 sites have been established in State forest. |
| Portland Horsham | Sun Moth (5091) | Synemon sp c.f. selene | Establish a SMZ over verified populations. | Detailed planning is required to ensure population is protected from timber harvesting. |  |
| Portland Horsham | Tree Goanna | Varanus varius | Establish a SMZ over verified populations. | Detailed planning is required to ensure population is protected from timber harvesting. |  |
| Portland Horsham | Variegated Pygmy Perch | Nannoperca variegata | Establish an SPZ extending 40 m from each bank for 40 m upstream and 40 m downstream of verified populations. | Disturbance that could impact on water quality must be avoided within SPZ. |  |
| Portland Horsham | White-bellied Sea-eagle | Haliaeetus leucogaster | Establish a SMZ of 250 m radius over verified nesting populations. | Within the SMZ, avoid timber harvesting, road construction and burning during breeding season. Breeding season is 1 May to 31 December. Within 100m of nest trees, exclude timber harvesting, road construction and burning at all times. |  |
| Portland Horsham | White-footed Dunnart | Sminthopsis leucopus | Establish a SPZ of approximately 20 ha of suitable habitat over verified sites. |  |  |
| Statewide | Brush-tailed Phascogale | Phascogale tapoatafa | If Brush-tailed Phascogale is verified as occurring within State forest in FMA's where it was not understood to occur, consider establishing Brush-tailed Phascogale Management Areas (BPMAs) if the sites meet the review criteria. Establish new BPMA's in accordance with the following requirements: -each BPMA is to provide suitable habitat for a minimum of 25 female Brush-tailed Phascogales -each BPMA is to be a minimum of 1000ha, preferably already managed sympathetically for Brush-tailed Phascogales -priorities for BPMAs are confirmed 'hot spots', known presence of Brush-tailed Phascogales, reintroduction sites and suitable habitat. Include BPMAs that occur within State forest in SMZ. | Implement prescriptions for BPMAs within SMZs in State forest that protect and maintain Brush-tailed Phascogale populations and their habitat, such as retention of large trees and prohibition of removal of naturally fallen timber. In the Benalla-Mansfield and North East FMA's prepare an SMZ plan prior to commencement of timber harvesting. | Review and change sites only if new sites are of a higher quality or are better able to be protected than existing sites. |

## Flora – fixed zoning

### Tall Astelia

#### Maintain as SPZ one sub catchment in each of the major watersheds in which Tall Astelia occurs totalling 450 ha. These areas are Bjoksten Creek (La Trobe watershed), Seven Acre Creek upstream from Bunyip Road (Bunyip watershed) and Tomahawk Creek tributary (Yarra watershed).

## Flora – detection based zoning

### Statewide

#### Apply the management actions outlined in Table 5 (Detection based FMZ rules for flora) below for zoned rare or threatened flora values.

#### Implement FMZ amendments and reviews in accordance with Table 5 (Detection based FMZ rules for flora) below for new verified rare or threatened flora records and FMZ amendment requirements outlined in section 2.

### East Gippsland FMA

#### Include all verified populations of species regarded as threatened at a State or national level in the SPZ or SMZ.

#### Include substantial representative populations of species that are rare in Victoria, and poorly known, have few records or are at the edge of their range in East Gippsland in the SPZ or SMZ.

#### Wherever practicable include populations in larger parts of the SPZ or SMZ in combination with other values.

#### Plan disturbances in SMZ in consultation with DEPI biologists to ensure the species is adequately protected.

### Midlands FMA

#### Include all verified populations of species regarded as ‘endangered’ at a State or national level in the SPZ, except where otherwise specified in this document. The SPZ needs to be of sufficient size to include all of the local population, and should include a buffer large enough to protect the population from external impacts.

#### Include verified populations of species regarded as ‘vulnerable’ at a State or national level in the SPZ or SMZ after consideration of their level of representation in the existing conservation reserve system, except where otherwise specified in this document.

### Mid Murray FMA

#### Include all verified populations of species regarded as ‘endangered’ or ‘vulnerable’ at a State or national level in the SPZ or SMZ after consideration of their regional status, level of representation in the existing conservation reserve system and the nature of potential threats, except where otherwise specified in this document. Wherever practicable include populations in larger parts of the SPZ or SMZ in combination with other values. The SPZ or SMZ needs to be of sufficient size to include all of the local population, and where required, should include a buffer large enough to protect the population from external impacts and identified threats.

### North East FMAs

#### Include all verified populations of species regarded as ‘endangered’ at a State or national level in the SPZ, except where otherwise specified in this document. The SPZ needs to be of sufficient size to include all of the local population, and should include a buffer large enough to protect the population from external impacts.

#### Include verified populations of species regarded as ‘vulnerable’, ‘rare’, ‘depleted’, insufficiently known’, endemic’, ‘disjunct’ and ‘edge of range’ at a State or national level in the SPZ or SMZ after consideration of their level of representation in the existing conservation reserve system, except where otherwise specified in this document. Species that are well represented in the formal reserve system or which are locally abundant and tolerant of disturbance may not warrant inclusion in the SPZ or SMZ.

#### Wherever practicable include populations in larger parts of the SPZ or SMZ in combination with other values.

### Otway FMA

#### Include verified populations of species regarded as rare or threatened at a State or national level in the SPZ as appropriate.

Table 5 Detection based FMZ rules for flora.

| FMA | Common name | Scientific name | Zoning management actions | Management actions | Review |
| --- | --- | --- | --- | --- | --- |
| Bendigo | Bristly Greenhood | Pterostylis setifera | Establish a SMZ over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Bendigo | Broad-lip Diuris | Diuris X palachila | Establish a SMZ over verified populations | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Bendigo | Clover Glycine | Glycine latrobeana | Establish a SMZ over verified populations | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Bendigo | Emerald-lip Greenhood | Pterostylis smaragdyna | Establish a SMZ over verified populations | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Bendigo | Golden Cowslips | Diuris behrii | Establish a SMZ over verified populations | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Bendigo | Large Rustyhood | Pterostylis maxima | Establish a SMZ over verified populations | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Bendigo | Long-tail Greenhood | Pterostylis woollsii | Establish a SMZ over verified populations | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Bendigo | Lowly Greenhood | Pterostylis despectans | Establish a SMZ over verified populations | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. Within the SMZ, timber harvesting activities are limited to thinning operations conducted in accordance with plans developed for the specific purpose of aiding the survival of Lowly Greenhood. |  |
| Bendigo | Midlands Spider Orchid | Caladenia clavescens | Establish a SMZ over verified populations | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Bendigo | Sikh's Whiskers | Pterostylis boormanii | Establish a SMZ over verified populations | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Bendigo | Sturdy Leek-orchid | Prasophyllum validum | Establish a SMZ over verified populations | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Bendigo | Trailing Hop-bush | Dodonea procumbens | Establish a SMZ over verified populations | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Marsh Sun-orchid | Thelymitra longiloba | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Slender Leek-orchid | Prasophyllum parviflorum | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Austral Crane's-bill | Geranium solanderi var. solanderi s.s. | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Austral Moonwort | Botrychium australe | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Austral Toad-flax | Thesium australe | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Hairy Anchor Plant | Discaria pubescens | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Bantam Bush-pea | Pultenaea parrisiae | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Betka Bottlebrush | Callistemon kenmorrisonii | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Binung | Christella dentata | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Black Stem | Adiantum formosum | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Blackfellow's Hemp | Commersonia rossii | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Blunt Sandalwood | Santalum obtusifolium | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Bog Saw-sedge | Gahnia subaequiglumis ( | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Bonnet Orchid | Cryptostylis erecta | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Buff Hazelwood | Symplocos thwaitesii | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Cabbage Fan-palm | Livistona australis | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Coastal Greenhood | Pterostylis alveata | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Colquhoun Grevillea | Grevillea celata | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Devious Bent-grass | Deyeuxia decipiens | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Dwarf Milkwort | Polygala japonica | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Fairy Lanterns | Thismia rodwayi | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Fine Bent-grass | Deyeuxia parviseta var. boormanii | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Forest Weft-moss | Thuidium laeviusculum s.s. | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Forrester's Bottlebrush | Callistemon forresterae | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Genoa River Correa | Correa lawrenceana var. genoensis | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Granite Greenhood | Pterostylis tunstallii | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Green Leek-orchid | Prasophyllum lindleyanum | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Green Wattle | Acacia irrorata subsp. irrorata | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | King Greenhood | Pterostylis baptistii | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Kydra Dampiera | Dampiera fusca | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Lanky Buttons | Leptorhynchos elongatus | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Large Tick-trefoil | Desmodium brachypodum | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Leafless Tongue-orchid | Cryptostylis hunteriana | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Leafy Greenhood | Pterostylis cucullata | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Limestone Blue Wattle | Acacia caerulescens | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Little Kooka Wattle | Acacia nanopravissima ms. | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Long Rope-rush | Calorophus elongatus | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Low Bush-pea | Pultenaea subspicata | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Maiden's Wattle | Acacia maidenii | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Maroon Leek-orchid | Prasophyllum frenchii | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Marsh Leek-orchid | Prasophyllum niphopedium | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Matted Parrot-pea | Dillwynia prostrata | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Naked Sun-orchid | Thelymitra circumsepta | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Native Quince | Alectryon subcinereus | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Orange-blossom Orchid | Sarcochilus falcatus | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Pink Mountain-correa | Correa lawrenceana var. cordifolia | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Prawn Greenhood | Pterostylis pedoglossa | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Prickly Tree-fern | Cyathea leichhardtiana | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Purple Diuris | Diuris punctata var. punctata | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Rock Orchid | Thelychiton speciosus | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Rough Eyebright | Euphrasia scabra | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Rusty Velvetbush | Lasiopetalum ferrugineum var ferrugineum | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Sandpaper Fig | Ficus coronata | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Slender Gingidia | Gingidia harveyana | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Slender Mud-grass | Pseudoraphis paradoxa | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Slender Parrot-pea | Almaleea capitata | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Slender Tree-fern | Cyathea cunninghamii | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Spiral Sun-orchid | Thelymitra matthewsii | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Star Cucumber | Sicyos australis | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Tasmanian Wax-flower | Philotheca virgata | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Thick-lip Spider-orchid | Caladenia tessellata | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Tiny Spyridium | Spyridium cinereum | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Two-colour Panic | Panicum simile | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Upright Pomaderris | Pomaderris virgata | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | White Billy-buttons | Craspedia alba | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Wild Sorghum | Sarga leiocladum | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Wiry Wallaby-grass | Plinthanthesis paradoxa | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Wombargo Wattle | Acacia tabula ms. | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Yellow Elderberry | Sambucus australasica | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| East Gippsland | Yellow Hyacinth-orchid | Dipodium hamiltonianum | Establish a SMZ of 250 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Alpine Bush-pea | Pultenaea fasciclata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Alpine Buttons | Leptorhynchos sp. 1 | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Alpine Marianth | Rhytidosporum inconspicuum | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Alpine Wattle | Acacia alpina | Establish a SMZ of 200 m radius over verified populations. |  |  |
| Gippsland FMAs | Aniseed Boronia | Boronia galbraithiae | Establish a SPZ over each verified population. |  |  |
| Gippsland FMAs | Austral Toad-flax | Thesium australe | Establish a SPZ over each verified population. |  |  |
| Gippsland FMAs | Austral Trefoil | Lotus australis | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Bassian Pomaderris | Pomaderris oraria ssp. Oraria | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Baw Baw Sally | Eucalyptus pauciflora ssp. Acerina | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Birch Pomaderris | Pomaderris betulina ssp. Betulina | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Blotched Hyacinth-orchid | Dipodium variegatum | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Blue-tongue Greenhood | Pterostylis oreophila | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Blunt-leaf Pomaderris | Pomaderris helianthemifolia | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Bristly Helmet-orchid | Corysanthes hispidus | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Cliff Cudweed | Euchiton umbricola | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Clover Glycine | Glycine latrobeana | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Coast Bitter-bush | Adriana quadripartita (pubescent form) | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Cobberas Grevillea | Grevillea brevifolia ssp. Brevifolia | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Cobra Greenhood | Pterostylis grandiflora | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Colquhuon Grevillea | Grevillea celata | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Common Spleenwort | Asplenium trichomanes | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Convex Pomaderris | Pomaderris subcapitata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Crested Hair-grass | Koeleria cristata (=K. macrantha) | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Dark Wire-grass | Aristida calycina var. Calycina | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Delicate Bush-pea | Pultenaea tenella | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Delicate New Holland Daisy | Vittadina tenuissima | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Dense Mint-bush | Prostanthera decussata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Dwarf Milkwort | Polygala japonica | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Eastern Bitter-bush | Adriana tomentosa var. Tomentosa (glabrous and pubescent form) | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Eastern Pomaderris | Pomaderris discolor | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Eichler's Buttercup | Ranunculus eichlerianus | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Elusive Cress | Irenepharsus magicus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Erect Midge-orchid | Corunastylis arrecta | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Errinundra Pepper | Tasmannia xerophila ssp. Robusta | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Fairy Bluebell | Wahlenbergia densifolia | Establish a SPZ over each verified population. |  |  |
| Gippsland FMAs | Fairy Caladenia | Petalochilus alatus | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Filmy Maidenhair | Adiantum diaphanum | Establish a SPZ over each verified population. |  |  |
| Gippsland FMAs | Fisch's Greenhood | Pterostylis fischii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Gippsland Hemp Bush | Gynatrix macrophylla | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Gippsland Lakes Peppermint | Eucalyptus aff. Willisii (Gippsland Lakes) | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Globe-hood Sun-orchid | Thelymitra X chasmogama | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Golden Grevillea | Grevillea chrysophaea | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Golden Moths | Diuris lanceolata s.l. | Establish a SPZ over each verified population. |  |  |
| Gippsland FMAs | Golden Pomaderris | Pomaderris aurea | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Green-striped Greenhood | Pterostylis chlorogramma | Establish a SPZ over each verified population. |  |  |
| Gippsland FMAs | Green-Top Sedge | Carex chlorantha | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Grey Beard-heath | Leucopogon attenuatus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Grey Scentbark | Eucalyptus ignorabilis s.s. | Establish a SPZ of 200 m radius around each verified population. |  |  |
| Gippsland FMAs | Hairy Anchor Plant | Discaria pubescens | Establish a SPZ over each verified population. |  |  |
| Gippsland FMAs | Hard-head Bush-pea | Pultenaea capitellata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Heath Platysace | Platysace ericoides | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Honey Hood | Stegostyla hildae | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Lanky Buttons | Leptorhynchos elongatus | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Lanky Fescue | Austrofestuca eriopoda | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Large-leaf Ray-flower | Cyphanthera anthocercidea | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Leafless Pink-bells | Tetratheca subaphylla | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Leafy Greenhood | Pterostylis cucullata | Establish a SPZ over each verified population. |  |  |
| Gippsland FMAs | Lilac Berry | Trochocarpa clarkei | Establish a SPZ over each verified population. |  |  |
| Gippsland FMAs | Limestone Blue Wattle | Acacia caerulescens | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Limestone Pomaderris | Pomaderris oraria ssp. Calcicola | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Long-flower Beard-heath | Leucopogon juniperinus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Long-tongue Summer Greenhood | Pterostylis aestiva | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Mallee Ash | Eucalyptus kybeanensis | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Marsh Daisy | Brachyscome radicans | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Marsh Greenhood | Pterstylis uliginosa | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Marsh Leek-orchid | Prasophyllum niphopedium | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Mat Cudweed | Euchiton traversii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Metallic Sun-orchid | Thelymitra epipactoides | Establish a SPZ over each verified population. |  |  |
| Gippsland FMAs | Monkey Mint-bush | Prostanthera walteri | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Mossy Knawel | Scleranthus singuliflorus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Mountain Aciphyll | Aciphylla simplicifolia | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Mountain Dandelion | Taraxacum aristum | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Mountain Kangaroo-apple | Solanum linearifolium | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Mountain Leafless Bossiaea | Bossiaea bracteosa | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Mountain Leek-orchid | Prasophyllum montanum | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Mountain Wheat-grass | Australopyrum velutinum | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Naked Sun-orchid | Thelymitra circumsepta | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Native wintercress | Barbarea grayi | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Netted Daisy-bush | Olearia speciosa | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Omeo Gum | Eucalyptus neglecta | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Orange-tip Caladenia | Petalochilus aurantiacus | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Outcrop Guinea-flower | Hibbertia hermanniifolia | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Oval-leaf Grevillea | Grevillea miqueliana ssp. Miqueliana | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Oval-leaf Pseudoanthus | Pseudanthus ovalifolius | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Ovens Everlasting | Ozothamnus stirlingii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Pinkwood | Beyeria viscosa | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Pinnate Goodenia | Goodenia macmillanii | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Poverty Wattle | Acacia dawsonii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Prickly Tree-fern | Cythea leichhardtiana | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Privet Pomaderris | Pomaderris ligustrina ssp. Ligustrina | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Prostrate Cone-bush | Isopogon prostratus | Establish a SPZ over each verified population. |  |  |
| Gippsland FMAs | Purple Diuris | Diuris punctata | Establish a SPZ over each verified population based on field inspection. |  |  |
| Gippsland FMAs | Purple Eyebright | Euphrasia collina ssp. Muelleri | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Rock daisy | Brachyscome petrophila | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Rock Grevillea | Grevillea willisii | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Rough Eyebright | Euphrasia scabra | Establish a SPZ over each verified population. |  |  |
| Gippsland FMAs | Rough-grain Love-grass | Eragrostis trachycarpa | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Sandfly Zieria | Zieria smithii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Scented Daisy-bush | Olearia adenophora | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Selma Saddle Grevillea | Grevillea sp. Aff. Monslacana | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Shingle Fireweed | Senecio diaschides | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Shining Anchor Plant | Discaria nitida | Establish a SPZ over each verified population. |  |  |
| Gippsland FMAs | Shiny Leionema | Leionema lamprophyllum ssp. lamprophyllum | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Short-awned Wheat-grass | Elymus multiflorus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Skeleton Vine | Clematis microphylla var. Leptophylla | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Slender Parrot-pea | Almaleea capitata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Slender Pomaderris | Pomaderris phylicifolia ssp. Ericoides | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Slender Saw-sedge | Gahnia microstachya | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Slender Tick-trefoil | Desmodium varians | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Slender Tree-fern | Cythea cunninghamii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Slender Violet-bush | Hybanthus monopetalus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Small-leaf Bush-pea | Pultenaea foliolosa | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Snow Aciphyll | Aciphylla glacialis | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Snow Coprosma | Coprosma nivalis | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Soft-ledge Grass | Poa hothamensis var. Parviflora | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Spicy Everlasting | Ozothamnus argophyllus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Spinning Gum | Eucalyptus perriniana | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Spiral Sun-orchid | Thelymitra matthewsii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Sprawling Cassia | Senna aciphylla | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Spreading Knawel | Scleranthus fasciculatus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Stalked Brooklime | Gratiola pedunculata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Stalked Guinea-flower | Hibbertia pedunculata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Sticky Wattle | Acacia howittii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Stony Bush-pea | Pultenaea setulosa | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Strawberry Buttercup | Ranunculus collinus | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Striped Pomaderis | Pomaderris pilifera | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Strzeleckii Gum | Eucalyptus strzeleckii | Establish a SPZ over each verified population. |  |  |
| Gippsland FMAs | Subalpine Baekea | Baeckea latifolia | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Tailed Eyebright | Euphrasia caudata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Tall Ground-berry | Acrotriche leucocarpa | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Tall Vanilla-lily | Arthropodium sp. 1 | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Tangled Pseudoanthus | Pseudanthus divaricatissimus | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Tasmanian Bladderwort | Utricularia monanthos | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Thick Eyebright | Euprasia crassiuscula | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Tight Bedstraw | Galium curvihirtum | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Tiny Arrowgrass | Triglochin minutissimum | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Toothed leionema | Leionema bilobum ssp. 3 (West Gippsland) | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Tullard Ard Grevillea | Grevillea brevifolia ssp. Polychroma | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Tussock Sedge | Carex iynx | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Upright Panic | Entolasia stricta | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Velvet Apple-berry | Billardiera scandens var. Brachyantha | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Velvety Geebung | Persoonia subvelutina | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Wallaby-bush | Beyeria lasiocarpa | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Wedge Guinea-flower | Hibbertia diffusa | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Wild Sorghum | Sorghum leiocladum | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Winged Everlasting | Ozothamnus adnatus | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Woolly-bear Wattle | Acacia lucasii | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Gippsland FMAs | Woolly-head Pomaderris | Pomaderris eriocephala | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Gippsland FMAs | Wrinkle-nut Lignum | Muehlenbeckia rhyticarya | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Mid Murray | Buloke Mistletoe | Amyema linophylla ssp. orientale | Establish a SMZ of 100 m radius over verified populations that are not already protected. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Mid Murray | Button Rush | Lipocarpha microcephala | Establish a SMZ of 100 m radius over verified populations that are not already protected. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Mid Murray | Delicate Love-grass | Eragrostis tenellula | Establish a SMZ of 100 m radius over verified populations that are not already protected. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Mid Murray | Dwarf Swainson-pea | Swainsona phacoides | Establish a SMZ of 100 m radius over verified populations that are not already protected. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Mid Murray | Mueller Daisy | Brachyscome muelleroides | Establish a SMZ of 100 m radius over verified populations that are not already protected. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Mid Murray | River Swamp Wallaby-grass | Amphibromus fluitans | Establish a SMZ of 100 m radius over verified populations that are not already protected. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Mid Murray | Small Scurf-pea | Cullen parvum (syn. Psoralea parva) | Establish a SMZ of 100 m radius over verified populations that are not already protected. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Mid Murray | Winged Pepper-cress | Lepidium monoplocoides | Establish a SMZ of 100 m radius over verified populations that are not already protected. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Midlands | Clover Glycine | Glycine latrobeana | Establish a SPZ over each verified population based on field inspection. |  | Review this strategy when 10 sites and 1000 individuals are protected in State forest. |
| Midlands | Mt William Beard-heath | Leucopogon neurophyllus | Establish a SPZ over each verified population. |  |  |
| Midlands | Purple Diuris | Diuris punctata | Establish a SPZ over each verified population based on field inspection. |  | Review this strategy when 10 sites and 1000 individuals are protected. |
| Midlands | Small Pepper-cress | Lepidium hyssopifolium | Establish a SPZ over each verified population. | Conduct pre-harvest surveys for the species in the vicinity of Bolworrah and Trentham. |  |
| North East FMAs | Alpine Colobanth | Colobanthus affinis | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Alpine Cranesbill | Geranium sessiliflorum ssp. brevicaule | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Alpine Fen-sedge | Carex hypandra | Establish a SPZ of 200 m radius over each verified population. |  |  |
| North East FMAs | Alpine Finger-fern | Grammitis poeppigiana | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Alpine Groundsel | Senecio pectinatus var. major | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Alpine Pennywort | Schizeilema fragoseum | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Alpine Phebalium | Phebalium squamulosum ssp. alpinum | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Alpine Rush | Juncus brevibracteus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Alpine Stackhousia | Stackhousia pulvinaris | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Alpine Wattle | Acacia alpina | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Antelope Greenhood | Pterostylis laxa | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Archers Sedge | Carex archeri | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Austral Moonwort | Botrychium australe | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Austral Toad-flax | Thesium australe | Establish a SPZ over each verified population. |  |  |
| North East FMAs | Beechworth Silver Stringybark | Eucalyptus cinerea ssp. cinerea | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Bent-leaf Wattle | Acacia flexifolia | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Blue-leaf Tussock-grass | Poa sieberiana var. cyanophylla | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Blue-tongue Greenhood | Pterostylis oreophila | Establish a SPZ of 200 m radius over each verified population. |  |  |
| North East FMAs | Blunt-leaf Pomaderris | Pomaderris helianthemifolia | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Bogong Daisy-bush | Olearia frostii | Establish a SPZ of 200 m radius over each verified population. |  |  |
| North East FMAs | Bristly Helmet-orchid | Corybas hispidus | Establish a SPZ of 200 m radius over each verified population. |  |  |
| North East FMAs | Brittle Bladder-fern | Cystopteris tasmanica | Establish a SPZ of 200 m radius over each verified population. |  |  |
| North East FMAs | Broad-leaf Hop-bush | Dodonaea rhombifolia | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Broad-lip Leek-orchid | Prasophyllum patens | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Buffalo Sallee | Eucalyptus mitchelliana | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Button Rush | Lipocarpha microcephala | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Catkin Wattle | Acacia dallachiana | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Chinese Lespedeza | Lespedeza juncea ssp. sericea | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Cliff Cudweed | Euchiton umbricolus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Clover Glycine | Glycine latrobeana | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Cobberas Grevillea | Grevillea brevifolia | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Comb Wheat-grass | Australopyrum retrofractum | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Common Spleenwort | Asplenium trichomanes | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Concave Pomaderris | Pomaderris subplicata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the population is adequately protected during timber harvesting operations. |  |
| North East FMAs | Convex Pomaderris | Pomaderris subcapitata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Crimson Grevillea | Grevillea polybractea | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Crimson Spider-orchid | Caladenia concolor | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Cryptic Heath | Epacris celata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Cupped Bush-pea | Pultenaea vrolandii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Currawang | Acacia doratoxylon | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Deanes Wattle | Acacia deanei ssp. deanei | Establish a SPZ of 200 m radius over each verified population. |  |  |
| North East FMAs | Delicate Bush-pea | Pultenaea tenella | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Delicate Cranesbill | Geranium sp. 6 | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Dense Mint-bush | Prostanthera decussata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Dookie Daisy | Brachyscome gracilis ssp. gracilis | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Dusky Bush-pea | Pultenaea polifolia | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Dwarf Buttercup | Ranunculus millanii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Dwarf Milkwort | Polygala japonica | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Eichlers Buttercup | Ranunculus eichlerianus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Fir Clubmoss | Huperzia australiana | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Fischs Greenhood | Pterostylis fischii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Flat-leaf Bush-pea | Pultenaea platyphylla | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Fog Club-sedge | Isolepis montivaga | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Fringed Rice-flower | Pimelea ligustrina ssp. ciliata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Gippsland Hemp Bush | Gynatrix macrophylla | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Glandular Early Nancy | Wurmbea biglandulosa ssp. biglandulosa | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Golden Pomaderris | Pomaderris aurea | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Graceful Sun-orchid | Thelymitra simulata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Grey Beard-heath | Leucopogon attenuatus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Grey Rice-flower | Pimelea treyvaudii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Gunns Alpine Buttercup | Ranunculus gunnianus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Hairy Anchor Plant | Discaria pubescens | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the population is adequately protected during timber harvesting operations. Exclude the population from burning operations. |  |
| North East FMAs | Hairy Eyebright | Euphrasia lasianthera | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Hairy Hop-bush | Dodonaea boroniifolia | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Highland Bush-pea | Pultenaea williamsonii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Jerry-jerry | Ammannia multiflora | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Kerrawang | Rulingia dasyphylla | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Ladys Mantle | Alchemilla sp. 1 | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Lanky Buttons | Leptorhynchos elongatus | Establish a SPZ of 200 m radius over each verified population. |  |  |
| North East FMAs | Large Velvet Wallaby-grass | Austrodanthonia pilosa var. paleacea | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Leafy Greenhood | Pterostylis cucullata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the population is adequately protected during timber harvesting operations. |  |
| North East FMAs | Lilac Berry | Trochocarpa clarkei | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Lilac Bitter-cress | Cardamine lilacina s.s. | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Long-tongue Summer Greenhood | Pterostylis aestiva | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Mallee Ash | Eucalyptus kybeanensis | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Mossy Knawel | Scleranthus singuliflorus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Mountain Leafless Bossiaea | Bossiaea bracteosa | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Mountain Mat-rush | Lomandra oreophila | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Mountain Swainson-pea | Swainsona recta | Establish a SPZ of 200 m radius over each verified population. |  |  |
| North East FMAs | Muellers Bent | Agrostis muelleriana | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Mugga | Eucalyptus sideroxylon s.s. | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Narrow Goodenia | Goodenia macbarronii | Establish a SPZ over each verified population. |  |  |
| North East FMAs | Narrow-leaf Star-hair | Astrotricha linearis | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Native Wintercress | Barbarea grayi | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Omeo Gum | Eucalyptus neglecta | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Poverty Wattle | Acacia dawsonii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Purple Diuris | Diuris punctata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Purple Eyebright | Euphrasia collina ssp. muelleri | Establish a SPZ of 200 m radius over each verified population. |  |  |
| North East FMAs | River Hook-sedge | Uncinia nemoralis | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | River Leafless Bossiaea | Bossiaea riparia | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Rock Grevillea | Grevillea willisii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Rock Tussock-grass | Poa petrophila | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Rough Eyebright | Euphrasia scabra | Establish a SPZ over each verified population. |  |  |
| North East FMAs | Royal Grevillea | Grevillea victoriae s.s. | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Sand Rush | Juncus psammophilus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Scaly Greenhood | Pterostylis hamata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Sickle-leaf Rush | Juncus falcatus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Silver Stringybark | Eucalyptus alligatrix ssp. alligatrix | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Silver Tea-tree | Leptospermum multicaule | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Slender Club-sedge | Isolepis congrua | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Slender Love-grass | Eragrostis exigua | Establish a SPZ of 200 m radius over each verified population. |  |  |
| North East FMAs | Slender Parrot-pea | Almaleea capitata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Slender Violet-bush | Hybanthus monopetalus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Small-leaf Bush-pea | Pultenaea foliolosa | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Snow Aciphyll | Aciphylla glacialis | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Snow Beard-heath | Leucopogon montanus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Snow Pennywort | Diplaspis nivis | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Snow Wallaby-grass | Rytidosperma nivicolum | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Snowdrop Wood-sorrel | Oxalis magellanica | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Snowy River Wattle | Acacia boormanii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Spinning Gum | Eucalyptus perriniana | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Spreading Clubmoss | Lycopodium scariosum | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Spreading Knawel | Scleranthus fasciculatus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Stalked Adders-tongue | Ophioglossum petiolatum | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Stony Bush-pea | Pultenaea lapidosa | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Summer Leek-orchid | Prasophyllum canaliculatum | Establish a SPZ of 200 m radius over each verified population. |  |  |
| North East FMAs | Swamp Fern | Thelypteris confluens | Establish a SPZ of 200 m radius over each verified population. |  |  |
| North East FMAs | Tailed Eyebright | Euphrasia caudata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Thick Bent-grass | Deyeuxia crassiuscula | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Tick Indigo | Indigofera adesmiifolia | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Tingaringy Gum | Eucalyptus glaucescens | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Tiny Bent | Agrostis australiensis | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Tiny Daisy | Brachyscome ptychocarpa | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Tiny Midge-orchid | Genoplesium nudum | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Tussock Woodrush | Luzula alpestris | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Umbrella Grass | Digitaria divaricatissima | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Velvet Apple-berry | Billardiera scandens var. brachyantha | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Velvety Geebung | Persoonia subvelutina | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Warby Range Swamp Gum | Eucalyptus cadens | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the population is adequately protected during timber harvesting operations. |  |
| North East FMAs | Wedge Diuris | Diuris dendrobioides | Establish a SPZ of 200 m radius over each verified population. |  |  |
| North East FMAs | Wedge Oschatzia | Oschatzia cuneifolia | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Winged Water-starwort | Callitriche umbonata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Woolly Wattle | Acacia lanigera var. lanigera | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Yellow Flat-sedge | Cyperus flavidus | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| North East FMAs | Yellow Hyacinth-orchid | Dipodium hamiltonianum | Establish a SPZ over each verified population. |  |  |
| Portland Horsham | Clover Glycine | Glycine latrobeana | Establish a SMZ over verified populations based on field inspection. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Portland Horsham | Dergholm Guinea-flower | Hibbertia humifusa subsp. debilis | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Portland Horsham | Green-striped Greenhood | Pterostylis chlorogramma | Establish a SPZ over each verified population. |  |  |
| Portland Horsham | Leafy Greenhood | Pterostylis cucullata | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the population is adequately protected during timber harvesting operations. |  |
| Portland Horsham | Purple Eyebright | Euphrasia collina ssp. Muelleri | Establish a SPZ of 200 m radius over each verified population. |  |  |
| Portland Horsham | Spiral Sun-orchid | Thelymitra matthewsii | Establish a SMZ of 200 m radius over verified populations. | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |
| Portland Horsham | Trailing Hop-bush | Dodonea procumbens | Establish a SMZ over verified populations | Conduct a site inspection and detailed planning in consultation with DEPI to ensure the species is adequately protected during timber harvesting operations. |  |

## Vegetation communities – fixed zoning

### Statewide

#### Include in the SPZ all rainforest stands over 0.4 ha in area (whether already mapped or newly detected) that fall within the area categorised as National Priority 1 in the Sites of Significance for Rainforest spatial layer.  The SPZ should extend to the nearest watershed boundary.

#### Maintain a CAR reserve system consistent with the JANIS criteria.

### Bendigo FMA

#### Protect EVCs categorised as Endangered and Rare by ECC (2001) within the SPZ where practicable.

### Central Highlands FMAs

#### Include in the SPZ all rainforest stands over 0.4 hectares in area (whether already mapped or newly detected) that fall within the area categorised as State or Regional in the  Sites of Significance for Rainforest spatial layer.   Also include in the SPZ the relevant buffer as identified below in Table 6 (Buffer widths for Rainforest Sites of Significance by category and priority).

#### Include in SPZ all stands of old growth forest in State forest that are greater than 5 ha in area from Wet Forest, Montane Wet Forest, Damp Forest, Montane Damp Forest, Riparian Forest, Sub-alpine Woodland and non eucalypt EVCs.

#### Include at least 60% of old growth forest in the Montane Dry Woodland, Heathy Dry Forest and Heathy Woodland EVCs in conservation reserves and SPZ.

#### When selecting areas of old growth to include in SPZ consider:

#### representation of the old-growth component of each EVC in each GRU; and

#### combining old-growth forest conservation with other strategies to create larger, consolidated areas of SPZ with identifiable management boundaries.

#### The following proportion of each EVC should be incorporated into conservation reserves or the SPZ:

#### 30 % of EVCs that occupy more than 1% of the public land in the Central Highlands FMA;

#### between 30 % and 90 % of EVCs that occupy between 0.1 % (approximately 6000 ha) and 1 % of the public land in the Central Highlands FMA, depending on the extent of the particular EVC; and

#### 90 % of EVCs that occupy less than 0.1 % of the public land in the Central Highlands FMA.

#### To meet the EVC reservation targets, EVCs should, where possible, be selected from areas:

#### in existing conservation reserves and the SPZ;

#### identified as being good representative examples of the EVC;

#### identified as containing threatened wildlife, flora or communities;

#### identified as containing concentrations of endemic, edge-of-range or disjunct flora;

#### identified as containing high levels of floristic and faunal richness;

#### which are minimally affected by recorded human disturbance;

#### containing other significant biological values; and

#### that are unsuitable for timber production.

### East Gippsland FMA

#### Where rainforest stands coincide with linear reserves, include in the SPZ the rainforest stand plus a 100m buffer. Exceptions may apply where an alternative logical boundary exists within the buffer, for example an existing road. Avoid road construction across linear reserves containing rainforest wherever practicable.

#### Prioritise rainforest stands categorised as State and Regional in the Sites of Significance for Rainforest spatial layer for sub-catchment protection in SPZ according to the following principles:

#### give preference to sites of State significance before sites of Regional significance;

#### give preference to core areas that are substantially undisturbed and which also help fulfil conservation guidelines for other values (such as Sooty Owl, Long-footed Potoroo and representation of EVCs or old growth forest);

#### provide a geographic spread of rainforest areas with sub-catchment protection across the FMA; and

#### give lower priority to sites of significance that are large in relation to the rainforest they include, significantly disturbed, or close to conservation reserves with similar rainforest stands.

#### Examples of around 100 hectares of highly variable EVCs such as Lowland Forest and Banksia Woodland may be included in conservation reserves and SPZ.

#### The following proportion of each old growth forest EVC should be incorporated into conservation reserves or the SPZ:

#### all viable examples of rare or depleted (generally less than 10% of the extant distribution) old growth forest EVCs wherever possible; and

#### at least 60% of the extent of all other old growth forest EVCs present in 1995.

#### When selecting areas of old growth to include in SPZ consider the points listed in clause 4.6.4.4 and

#### analysis of the representation of each old-growth forest EVC in each GRU;

#### maintaining representative and viable examples of old-growth across the landscape;

#### protection of mosaics of old-growth forest, negligibly disturbed forest and naturally disturbed forest to provide for recruitment of old-growth forest in the long term; and

#### combining old-growth forest conservation with other strategies to create larger, consolidated areas in the SPZ.

### Gippsland FMAs and North East FMAs

#### In the Gippsland FMAs, include in the SPZ all Cool Temperate Rainforest and Warm Temperate Rainforest stands over 0.4 hectares in area (whether already mapped or newly detected) that fall within the area categorised as State or Regional in the  Sites of Significance for Rainforest spatial layer.   Also include in the SPZ the relevant buffer as identified below in table 6.

Table 6 Buffer widths for Rainforest Sites of Significance by category and priority.

| Site of Significance category | Priority |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| National | See 4.6.1.1 | 100 m | 60 m | 60 m |
| State | 60 m | 60 m | 40 m | 40 m |
| Regional | 40 m | 40 m | 40 m | 40 m |

Note: Priority areas are identified in the Sites of Significance for Rainforest spatial layer.

#### The following proportion of each old growth forest EVC should be incorporated into conservation reserves or the SPZ:

#### all viable examples of rare or depleted (generally less than 10 % of the extant distribution) old growth forest EVCs wherever possible; and

#### at least 60 % of the extent of all other old growth forest EVCs at the time of assessment.

#### When selecting areas of old growth to include in SPZ consider the points listed in clause 4.6.5.2.

### Mid Murray FMA

#### Conduct reviews of the CAR reserve system in accordance with the JANIS criteria.

#### Select vegetation communities for protection in conservation reserves and SPZ where possible from areas:

#### known to contain, or to have once contained, threatened flora or fauna;

#### identified as being good representative examples of the community;

#### that support the requirements of other conservation strategies relevant to forest management zoning in the Mid Murray FMA;

#### that help to establish an inter-linked protected area network across the FMA; and

#### that are the least suitable for timber harvesting.

### Portland and Horsham FMA

#### Maintain a CAR reserve system within the FMAs consistent with the JANIS criteria.

## Vegetation communities – detection based zoning

### East Gippsland, Midlands and Otway FMAs

#### Apply the management actions outlined in table 7 below for zoned rare or threatened vegetation community values.

#### Implement FMZ amendments and reviews in accordance with Table 7 (Detection based FMZ rules for vegetation communities) below for new verified rare or threatened vegetation communities and FMZ amendment requirements outlined in section 2.

Table 7 Detection based FMZ rules for vegetation communities.

| FMA | Common name | Scientific name | Zoning management actions |
| --- | --- | --- | --- |
| East Gippsland | Heathland (all heathland EVCs and other treeless vegetation) |  | Establish a SPZ over all heathland and other treeless vegetation communities. |
| Midlands | Snow Gum | Eucalyptus pauciflora | Establish a SPZ over vegetation communities dominated by Snow Gum on Mt Cole. |
| Midlands | Swamp Gum | Eucalyptus ovata | Establish a SPZ over vegetation communities dominated by Swamp Gum near Enfield. |
| Otway | Heathy Woodland, Wet Heath and Coastal Complex |  | Include all examples of these vegetation communities within SPZ. |

# Historic Places

### Statewide – detection based zoning

#### Plan management actions for historic places in accordance with the principles of the Burra Charter (International Council on Monuments and Sites), which states that ‘places of cultural significance should be safeguarded and not put at risk or left in a vulnerable state’.

#### Consider the significance, susceptibility to loss and specialist cultural heritage advice when determining management actions for a previously unrecorded historic place or a SMZ where management actions are unspecified.

### Bendigo FMA, Central Highlands FMAs, East Gippsland FMA, Gippsland FMAs, Mid Murray FMA, North East FMAs and Portland Horsham FMA – fixed zoning

#### In the Bendigo FMA, Central Highlands FMAs, East Gippsland FMA, Gippsland FMAs, Mid Murray FMA, North East FMAs and Portland Horsham FMA maintain SPZs and SMZs over historic places in accordance with Table 8 (FMZ rules for historic places) below.

### Midlands FMA and Otway FMA – fixed zoning

#### In the Midlands and Otway FMAs, include the most significant and/or representative historic places in the SPZ or SMZ, depending on their susceptibility to loss.

Table 8 FMZ rules for historic places.

| FMA | Locality | HPS no. | Area/Site no. | Site name | Significance | Zoning management actions | Management actions |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bendigo | Argyle State Forest | 7920 |  | Hodsman's Gate Hut Site | Local | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Barp State Forest | 7029 |  | Belgian / Perseverence Quartz Gold Mine | Local | Maintain a SMZ of 100 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Barp State Forest | 7028 |  | Belgian Reef Cyanide Vat | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Bealiba State Forest | 7313 |  | Bealiba Forest Timber Camp | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Bealiba State Forest | 7265 |  | Three Grain Gully Alluvial Workings / First Inkerman Diggings | State | Maintain a SMZ over the site. | Develop and comply with a SMZ plan |
| Bendigo | Clunes State Forest | 6261 |  | Mount Glasgow Reef Workings | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Costerfield State Forest | 5423 | VHR: H1298 | Bombay Mine & Cyanide Works | State | Maintain a SPZ of 100 m radius over the site. |  |
| Bendigo | Costerfield State Forest | 5592 | VHR: H1298 | Costerfield Mine Main Shaft Site | State | Maintain a SPZ of 100 m radius over the site. |  |
| Bendigo | Costerfield State Forest | 6226 |  | Minerva Mine | Local | Maintain a SPZ of 100 m radius over the site. |  |
| Bendigo | Costerfield State Forest | 6297 |  | New Alison Mine | Local | Maintain a SMZ of 100 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Costerfield State Forest | 6434 |  | Old Alison Shaft | Local | Maintain a SMZ of 100 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Costerfield State Forest | 6818 |  | Taits Shaft (South Costerfield Gold & Antimony Mining Co.) | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 5412 |  | Bluchers Gully Puddler No. 1 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 8020 |  | Bluchers Gully Puddler No. 2 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 8021 |  | Bluchers Gully Puddler No. 3 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 8022 |  | Bluchers Gully Puddler No. 4 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 5415 |  | Bluchers Reef Cyanide Works | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 5348 |  | Bully Gully Battery, Distillery, Puddler, House Site & Cyanide Works | State | Maintain a SPZ over the site. |  |
| Bendigo | Craigie State Forest | 5481 |  | Cambrian Mine & Puddler | Local | Maintain a SMZ of 100 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 5688 |  | Eaglehawk Dam & Puddler | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 5691 |  | Eaglehawk Gully Puddler No. 1 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 7327 |  | Eaglehawk Gully Puddler No. 2 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 7326 |  | Four Mile Flat Puddler | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 7990 |  | Golden Point (Eaglehawk Reef) Burial Ground | Local | Maintain a SPZ of 50 m radius over the site. |  |
| Bendigo | Craigie State Forest | 5908 |  | Great Extended Co. | Local | Maintain a SMZ of 40 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 7339 |  | Interface between modern and old workings - White Horse Gully | State | Maintain a SMZ of 100 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 7344 |  | Main Lead Puddler | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 7961 |  | Mosquito School Site | Local | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 7960 |  | Puddler, off Centenary Track | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 6646 |  | Schicer Gully Dam and Puddler | Local | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 7930 |  | Schicer Gully Dam Eucalyptus Distillery | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 6821 |  | Talisman Mine Workings & adjacent workings | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 5692 |  | Western Dam Puddler | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 7341 |  | White Horse Gully Puddler & Cyanide Works | Local | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 7337 |  | White Horse Gully Puddler No. 1 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Craigie State Forest | 8031 |  | White Horse Gully Puddler No. 2 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Daisy Hill State Forest | 5403 |  | Blacksmith's Gully Eucalyptus Distilliing Plant | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Daisy Hill State Forest | 5447 |  | Brogan's Hut and Paddock | Local | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Daisy Hill State Forest | 7462 |  | Eucalyptus Distillery | Local | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Daisy Hill State Forest | 5964 |  | Hendricksons | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Daisy Hill State Forest | 6479 |  | Perserverence Co. / Prince of Wales Co. | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Daisy Hill State Forest | 7325 |  | Puddler and Cyanide Works, Bonny Jean Dam | Local | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Daisy Hill State Forest | 7324 |  | Puddler and Cyanide Works, Cockatoo Lead | Local | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Dunolly / Waanyarra State Forests | 7018 |  | Almeida Reef Workings and Mining Settlement Site | Local | Maintain a SMZ of 400 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Dunolly / Waanyarra State Forests | 7045 |  | Burnt Tree Gully Puddling Machine & Dam | Local | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Dunolly / Waanyarra State Forests | 5609 |  | Cumberland Lead Puddler 7 Dam | Local | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Dunolly / Waanyarra State Forests | 7971 |  | Grave Site | Local | Maintain a SPZ of 10 m radius over the site. |  |
| Bendigo | Dunolly / Waanyarra State Forests | 7991 |  | Old Waanyarra Burial Ground | Local | Maintain a SPZ of 50 m radius over the site. |  |
| Bendigo | Dunolly/Waanyarra State Forests | 7242 |  | Specimen Reef Quartz Mining Co. Gold Mine | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Dunolly/Waanyarra State Forests | 7254 |  | Swipers Gully Puddler and Dam | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Dunolly/Waanyarra State Forests | 8018 |  | Tarnagulla Forest Camp | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Dunolly/Waanyarra State Forests | 5088 |  | Waanyarra Eucalyptus Distillery | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Dunolly/Waanyarra State Forests | 7972 |  | Waanyarra School | Local | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Dunolly/Waanyarra State Forests | 7299 |  | Wild Duck Lead Puddling Machine | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Eglington State Forest | 6385 |  | North German Gully Puddlers | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Fryers Range State Forest | 8131 |  | Charlie Sanger's Main Hut and Mining Area | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Fryers Range State Forest | 8134 |  | Hut Site and Grave Columbine Creek | Local | Maintain a SPZ of 50 m radius over the site. |  |
| Bendigo | Fryers Range State Forest | 8012 |  | Mineral Spring Site | Local | Maintain a SMZ of 10 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Fryers Range State Forest | 8132 |  | Red Hill Hydraulic Gold Sluicing Site Timber Bridge | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Fryers Range State Forest | 8016 |  | Stone Aqueduct | Local | Maintain a SMZ of 10 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Goughs Range State Forest | 6942 |  | Welcome Reef Syndicate Battery, Workings & Mud Brick House Sites | Local | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Graytown State Forest | 5761 |  | Fontainbleu Reef Puddler and Battery | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Graytown State Forest | 7923 |  | Mt. Black-Bailieston Track Charcoal Pit No. 4 | State | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Graytown State Forest | 7924 |  | Mt. Black-Bailieston Track Charcoal Pit No. 5 | State | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Graytown State Forest | 7958 |  | Nine Mile Diggings | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Graytown State Forest | 7868 |  | Plowright's Eucalyptus Distillery Site | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Graytown State Forest | 7957 |  | Scrubby Diggings | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Harvest Home State Forest | 7969 |  | Old Lead Water Race | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Havelock State Forest | 5671 |  | Dooley's Dam & Puddler | Local | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Havelock State Forest | 7343 |  | Lucknow Lead Puddler | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Havelock State Forest | 6471 |  | Pegleg Gully Puddler No. 1 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Havelock State Forest | 8025 |  | Pegleg Gully Puddler No. 2 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Havelock State Forest | 8026 |  | Pegleg Gully Puddler No. 3 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Havelock State Forest | 8027 |  | Pegleg Gully Puddler No. 4 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Inglewood State Forest | 8023 |  | Eaglehawk Gully Puddler No. 1 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Inglewood State Forest | 5690 |  | Eaglehawk Gully Puddler No. 2 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Inglewood State Forest | 7862 |  | F.G. & B. Eucalyptus Distillery Site 1 | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Inglewood State Forest | 5923 |  | Green Hills Gully Puddler | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Inglewood State Forest | 7378 |  | Jimmy Ghan's Eucalyptus Distillery/Tom Emment's Eucalyptus Factory | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Kimbolton State Forest | 8141 |  | Lyell Track Mine | Local | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Kingower State Forest | 7864 |  | FG. & B. EuCalyptus Distillery Site 3 | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Kingower State Forest | 7946 |  | Hidden cemetery | Local | Maintain a SPZ of 50 m radius over the site. |  |
| Bendigo | Lillicur State Forest | 7944 |  | Daisy Creek Puddler No. 1 | Local | Maintain a SMZ of 20 m, radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Lillicur State Forest | 5610 |  | Daisy Creek Puddler No. 2 | State | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Lillicur State Forest | 6213 |  | Mia Mia Flat Puddler Machine | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Llanelly State Forest | 7072 |  | Corfu Reef Quartz Gold Workings | Local | Maintain a SMZ of 100 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Llanelly State Forest | 7119 |  | Halfway Diggings settlement | Local | Maintain a SMZ of 150 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Llanelly State Forest | 7248 |  | Star Reef Quartz Gold Mine | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Longlea State Forest | 7975 |  | Native Gully School Site | Local | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Lyell State Forest | 7948 |  | Mosquito Gully Mine & Battery | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Moliagul State Forest | 7998 |  | Puddler and Dam No. 1 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Moliagul State Forest | 7999 |  | Puddler and Dam No. 2 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Moliagul State Forest | 8000 |  | Puddler and Dam No. 3 | Local | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Mt Hooghly State Forest | 7030 |  | Bet Bet Lead Puddling Machine | State VHR:111247 | Maintain a SPZ of 100 m radius over the site. |  |
| Bendigo | Mt Hooghly State Forest | 7031 |  | Bet Bet Reef Battery | Local | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Mt Hooghly State Forest | 7038 |  | Bromley Cemetery | Local | Maintain a SPZ of 50 m radius over the site. |  |
| Bendigo | Mt Hooghly State Forest | 7063 |  | Clovers Gully Puddling Machine | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Mt Hooghly State Forest | 7243 |  | Spillers Hill Alluvial Gold Workings | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Mt Hooghly State Forest | 7284 |  | Walkers Reef Battery Site, Shafts, Cyanide Vats, Whim Platform | Local | Maintain a SMZ of 100 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Muckleford State Forest | 5325 |  | Bacon Gully Puddlers & Alluvial Sinkings | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Muckleford State Forest | 5435 |  | Boswarva Hill Reef Workings | Local | Maintain a SMZ of 100 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Muckleford State Forest | 1794 |  | Demo Track Quartz Kiln | No known significance | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Muckleford State Forest | 5582 |  | Dunn's Reef Workings | No known significance | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Muckleford State Forest | 5798 |  | Gardners Gully Workings, Puddler & Mud Brick House | Local | Maintain a SMZ of 100 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Muckleford State Forest | 5844 |  | Golden Age Co. (Red White and Blue) Mine | Local | Maintain a SMZ of 100 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Muckleford State Forest | 6539 |  | Great South German Co. (Prince of Wales) & Puddler | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Muckleford State Forest | 6037 |  | John Bull G M. Co. | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Muckleford State Forest | 1373 |  | Kangaroo Dam and Puddler | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Muckleford State Forest | 1793 |  | Kangaroo Dam Quartz Kiln | To be assessed | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Muckleford State Forest | 6533 |  | Pnnce Llewellyn Co. | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Muckleford State Forest | 6835 |  | Thornhill Co. Quartz Roasting Kiln | State | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Muckleford State Forest | 7013 |  | Young Australian Co. Battery & Quartz Kilns | Local | Maintain a SMZ of 150 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | One Eye State Forest | 6638 |  | Sawpit Gully Puddlers | State VHR: H1246 | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Redcastle State Forest | 5606 |  | Curly Dog Dam & Puddler | Local | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Redcastle State Forest | 6348 |  | New Why Not Co. Battery | Local | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Redcastle State Forest | 6864 |  | Puddler | Local | Maintain a SMZ of 10 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Redcastle State Forest | 7978 |  | Redcastle Internment Camp | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Redcastle State Forest | 7369 |  | Welcome Reef Dam and Puddler No. 1 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Redcastle State Forest | 8030 |  | Welcome Reef Dam and Puddler No. 2 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Redcastle State Forest | 6939 |  | Welcome Reef Mine Site (Redcastle/Clarkes Reef Co) | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 5300 |  | Antonia Gully Puddler | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 5301 |  | Antonio Gully Brickworks and Puddler | Local | Maintain a SMZ of 100 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 7953 |  | Canadian Gully Puddlers | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 7485 |  | Charlie Bell's Crushing Battery | State | Maintain a SMZ of 15 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 7949 |  | Cheong's Chinese Garden | Local | Maintain a SPZ of 50 m radius over the site. |  |
| Bendigo | Rushworth State Forest | 7371 |  | Chinaman's Flat Brickworks and Puddler | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 5602 |  | Crown Cross Co. Mine | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 7955 |  | Double Grave Site | Local | Maintain a SPZ of 10 m radius over the site. |  |
| Bendigo | Rushworth State Forest | 7951 |  | Grave Site | Local | Maintain a SPZ of 10 m radius over the site. |  |
| Bendigo | Rushworth State Forest | 7952 |  | Holstein-Schleisweig Mine | Local | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 7943 |  | Main Gully Burial Ground (Old Rushworth Cemetery) | Local | Maintain a SPZ of 30 m radius over the site. |  |
| Bendigo | Rushworth State Forest | 7942 |  | Main Gully Chinese Camp | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 6424 |  | Nuggetty Gully Puddler | Local | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 6484 |  | Perseverence Co. Battery | Local | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 6277 |  | Perseverence Reef Cyanide Works | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 8028 |  | Phoenix Reef Mine Site | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 6524 |  | Poverty Diggings | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 6608 |  | Rushworth Gold Mines Pty Ltd | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 6667 |  | Shellback Gully Puddler No. 1 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 8029 |  | Shellback Gully Puddler No. 2 & Gold Workings | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 7954 |  | South Canadian Gully Puddlers | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Rushworth State Forest | 6750 |  | Specimen Hill Hut Site | Local | Maintain a SMZ of 10 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Sandon State Forest | 5954 |  | Harry Lauder Company | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Sandon State Forest | 8015 |  | Old House Site | Local | Maintain a SMZ of 10 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | St Arnaud State Forests | 3645 |  | Eucalyptus Distillery, north-east of St Arnaud | To be assessed | Maintain a SMZ of 30 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | St Arnaud State Forests | 8001 |  | Inspection Pit No. 1 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | St Arnaud State Forests | 8002 |  | Inspection Pit No. 2 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | St Arnaud State Forests | 8003 |  | Inspection Pit No. 3 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | St Arnaud State Forests | 8004 |  | Inspection Pit No. 4 | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | St Arnaud State Forests | 7983 |  | J.H. Vernons Eucalyptus Distillery | To be assessed | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | St Arnaud State Forests | 6025 |  | Jerejaw Reef Workings | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | St Arnaud State Forests | 7995 |  | Nook Mine | Local | Maintain a SMZ of 100 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | St Arnaud State Forests | 8033 |  | St. Arnaud-Teddington Water Supply Pipe | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Timor State Forest | 7964 |  | Hughes Dam Eucalyptus Distillery Site | Local | Maintain a SMZ of 15 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Upper Loddon State Forest | 8135 |  | Lancaster Hotel Site | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Upper Loddon State Forest | 8145 |  | Loddon River Structure (Chimney) No. 1 | To be assessed | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Upper Loddon State Forest | 8146 |  | Loddon River Structure (Chimney) No. 2 | To be assessed | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Upper Loddon State Forest | 8014 |  | Stone Chimney Site | Local | Maintain a SMZ of 10 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Wareek State Forest | 7988 |  | Chinese Baths | Local | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Wareek State Forest | 6060 |  | Kangaroo Gully Puddling Machine | Local | Maintain a SMZ of 20 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Wareek State Forest | 7976 |  | Possum Gully Mining Precinct | Local | Maintain a SMZ of 250 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Wareek State Forest | 6894 |  | Victoria Company | Local | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Bendigo | Wellsford State Forest | 7789 |  | Wellsford Eucalyptus Distillery | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Broadford District | 19 | 318 | Comet mill | State | Maintain a SPZ of 50 m radius over historic features. Maintain a SMZ of 50 m width around the remaining area of the Historic and Cultural Features Reserve. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Central Highlands FMAs | Erica District | 1415 |  | Coopers Creek copper mine | State | Maintain a SPZ of 50 m radius over the historic features. Maintain a SMZ of 50m width around the remaining area. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Central Highlands FMAs | Noojee District | 1098 | Timb77 | Kirchubel sawmill | Regional, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 50 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Central Highlands FMAs | Noojee District | 1094 | Timb79 | Kirchubel tramway | Regional | Maintain a SPZ of 50 m radius over the tramway. Maintain a SMZ of 50 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Central Highlands FMAs | Dandenong FMA | 158 |  | Ada No. 2 mill | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Central Highlands FMAs | Dandenong FMA | 159 |  | Ada River make-up | National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 50 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Central Highlands FMAs | Dandenong FMA | 1253 |  | Ezards mill (Starlings Gap) | Local | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 50 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Central Highlands FMAs | Dandenong FMA | 1250 |  | High Lead summit | Regional, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 50 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Central Highlands FMAs | Dandenong FMA | 1251 |  | Incline to the New Ada mill | Regional, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 50 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Central Highlands FMAs | Dandenong FMA | 160 |  | Little Ada River make-up | Regional | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 50 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Central Highlands FMAs | Dandenong FMA |  |  | Mississippi No. 1 mill | National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Central Highlands FMAs | Dandenong FMA | 1252 |  | New Ada mill site | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Central Highlands FMAs | Dandenong FMA | 161 |  | New Federal mill site and huts | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Central Highlands FMAs | Dandenong FMA | 165 |  | New Federal mill site and huts | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Central Highlands FMAs | Dandenong FMA |  |  | Walk into History tramways (Starlings Gap to High Lead carpark) |  | Maintain a SPZ of 50 m width over the tramway. Maintain a SMZ of 50m width outwards from the tramway SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland | Murrungowar |  | 832/06 | 17 mile tree (polling place in 1905) | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland |  | 7544 |  | Bairnsdale-Orbost Railway | State, National Estate | Maintain a SPZ of 20 m either side of railway line. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland | Black-Allen line |  | 902/02 | Border cairn | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland |  |  |  | Buchan-Jindabyne Rd (built with Great Depression labour) | State, National Estate | Maintain a SPZ of 20 m either side of railway line. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland |  |  | 892/07 | Clarkeville gold-mining settlement | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland | Murrungowar |  | 832/02 | Close family allotment | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland | Orbost- Bairnsdale railway line |  | 801/02 | Cosstick’s Weir and trestle bridges | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland | Orbost- Bairnsdale railway line |  | 801/08 | Colquhoun railway siding | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland | Bonang goldfields |  | 893/05 | Croesus mine | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland |  |  | 895/02 | Dartmoor Hut | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland |  |  | 895/05 | Delegate River Tunnel (built by Chinese workers in the 1860s) | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland |  |  | 890/07 | Hungerford’s farm | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland |  |  |  | Lake Tyers House Rd (old coach route) | State, National Estate | Maintain a SPZ of 20 m either side of railway line. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland | Tara Range |  | 807/03 | Monarch mine | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland | Glen Arte |  | 830/05 | Timber-workers’ huts | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland |  |  |  | Walter Baldwin-Spencer's journey | State, National Estate | Maintain a SPZ of 20 m either side of railway line. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland |  |  | 892/10 | Wilson’s hut (slab hut) | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland | Mt Tara fire tower |  | 807/02 | World war II sites and goldmining areas | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| East Gippsland |  |  | 827/07 | Young’s creek weir (former Orbost water supply) | State, National Estate | Maintain a SPZ of 50 m radius over the site. Maintain a SMZ of 150 m width around the SPZ. | Seek specialist historic place management advice for timber harvesting operations within SMZ. |
| Gippsland FMAs |  | 7544 |  | Bairnsdale-Orbost Railway | State | Maintain a SPZ of 20 m either side of railway line. |  |
| Gippsland FMAs |  | 7565 |  | Battery Site (Cassilis) | Local | Maintain a SMZ of 10 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 2515 |  | Battery Site (Cast Iron Point Area) | Local | Maintain a SMZ of 10 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 5357 |  | Beehive Mine and Boiler | Regional | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 1342 |  | Bismark Mine & Machinery Site | State | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 7003 |  | Black Cat Battery | Regional | Maintain a SMZ of 15 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 7034 |  | Black Snake Battery & Cyanide Works | Regional | Maintain a SPZ of 100 m radius over the site. |  |
| Gippsland FMAs |  | 2700 |  | Boiler and Steam Engine | Regional | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 7493 |  | Bullumawaal Township, including Bullumwaal and Bullumwaal South Cemeteries | RNE-n, Regional | Maintain a SMZ of 600 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 7490 |  | Chairman’s Stand | RNE-n, Regional | Maintain a SPZ over the site. |  |
| Gippsland FMAs |  | 7056 |  | Cherry’s Battery | Regional | Maintain a SPZ of 25 m radius over the site. |  |
| Gippsland FMAs |  | 7067 |  | Commotion Battery Site | Regional | Maintain a SPZ of 25 m radius over the site. |  |
| Gippsland FMAs |  | 7074 |  | Crinoline Battery | State | Maintain a SPZ of 25 m radius over the site. |  |
| Gippsland FMAs |  | 7563 |  | Dahlsens Mill & Tramway | Local | Maintain a SMZ of 50 m radius over the mill and 20m either side of the tramway. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 7557 |  | Dawson City Mining Township | Of interest | Maintain a SPZ over the site. |  |
| Gippsland FMAs |  | 7080 |  | Deptford Proprietary Mine | State | Maintain a SPZ of 100 m radius over the site. |  |
| Gippsland FMAs |  | 7081 |  | Deptford Township | Regional | Maintain a SMZ of 100 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 2925 |  | Dogs Grave | RNE-n, Local | Maintain a SPZ of 10 m radius over the site. |  |
| Gippsland FMAs |  | 7082 |  | Dogtown | Regional | Maintain a SPZ of 100 m radius over the site. |  |
| Gippsland FMAs |  | 7543 |  | Donnellys Creek Alluvial Workings | Regional | Maintain a SPZ of 20 m either side of creek. |  |
| Gippsland FMAs |  | 1346 |  | Edwards Hill Township SIte | State | Maintain a SPZ of 150 m radius over the site. |  |
| Gippsland FMAs |  | 2523 |  | Eldorado No. 1 Mine | Local | Maintain a SMZ of 10 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 2524 |  | Eldorado No. 2 Mine | Local | Maintain a SMZ of 10 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  |  |  | Freda Treasure Reserve |  | Maintain a SPZ over the site. |  |
| Gippsland FMAs |  | 5781 |  | Freestone Creek Battery | Local | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 7498 |  | Froud’s Sawmill | RNE-n, Regional | Maintain a SPZ of 100 m radius over the site. |  |
| Gippsland FMAs |  | 7088 |  | Gambetta Reef Battery Site | State, VHR | Maintain a SPZ of 200 m radius over the site. |  |
| Gippsland FMAs |  | 1344 |  | Gippsland Consols Machinery Site | State | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 1345 |  | Gippsland Consols Mine | State | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 7499 |  | Gippsland Timber Co. Mill | RNE-n, Regional | Maintain a SPZ of 20 m radius over the site. |  |
| Gippsland FMAs |  | 7500 |  | Goodwood Timber & Tramway Co. | RNE-n, State | No machine movement within mill site, bounded on the east by Morris Creek, on the north by Goodwood Road, and on the west and south by lines running due north and due east from 486500 5745000. Maintain a SPZ of 5 m either side of tramway formations, and 20 m radius over all standing bridges. |  |
| Gippsland FMAs |  | 7502 |  | Hallett’s Sawmill | RNE-n, Regional | Maintain a SPZ of 100 m radius over the site. |  |
| Gippsland FMAs |  | 7120 |  | Hans Battery & Mine Site | Regional | Maintain a SPZ of 100 m radius over the site. |  |
| Gippsland FMAs |  | 7122 |  | Harrison’s Cut Diversion Sluice | State, VHR | Maintain a SMZ of 200 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 5956 |  | Haunted Stream Battery Site | Regional | Maintain a SPZ of 25 m radius over the site. |  |
| Gippsland FMAs |  | 7130 |  | Houghton’s Flat Diversion Tunnel | State, VHR | Maintain a SPZ of 200 m radius over the site. |  |
| Gippsland FMAs |  | 1001 |  | Incline Tramway | Local | Maintain a SPZ of 5 m either side of tramway. |  |
| Gippsland FMAs |  | 7503 |  | Jamieson & Thompson Sawmill | RNE-n, Regional | Maintain a SPZ of 25 m radius over the site. |  |
| Gippsland FMAs |  | 2812 |  | Jirnkee Water Race | Local | Maintain a SMZ of 5 m either side of race. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 7153 |  | Lady Vera Battery | RNE-n, State | Maintain a SPZ of 50 m radius over the site. |  |
| Gippsland FMAs |  | 7157 |  | Lily Creek Battery and Mine Site | State | Maintain a SPZ of 100 m radius over the site. |  |
| Gippsland FMAs |  | 7545 |  | Little Mississippi Tramway | Local | Maintain a SMZ of 5 m either side of tramway. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 2751 |  | Maidentown Township Site | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 75119 |  | Marthavale Mill Site | Local | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 7572 |  | McDonalds Hut | Of interest | Maintain a SPZ of 10 m radius over the site. |  |
| Gippsland FMAs |  | 7564 |  | Mine Site (Cassilis) | Local | Maintain a SMZ of 100 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 2127 |  | Mississippi Creek Quarry & Tramline | Regional | Maintain a SPZ of 5 m either side of tramway. |  |
| Gippsland FMAs |  | 7177 |  | Morning Star Mine and Waterwheel | State, VHR | Maintain a SPZ of 200 m radius over the site. |  |
| Gippsland FMAs |  | 7497 |  | Moscow Villa | Regional | Maintain a SPZ of 10 m radius over the site. |  |
| Gippsland FMAs |  | 7183 |  | Mountain Maid Battery | Regional | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 7184 |  | Mountaineer Mine Site | Regional | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 1963 |  | New Chum Battery | State, VHR | Maintain a SPZ of 100 m radius over the site. |  |
| Gippsland FMAs |  | 7501 |  | Peter Ah Sen’s Sawmill | RNE-n, State | Maintain a SPZ of 150 m radius over the site. |  |
| Gippsland FMAs |  | 7222 |  | Red Rose Battery | Regional | Maintain a SMZ of 25 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 6790 |  | Stirling | Local | Maintain a SPZ over the site. |  |
| Gippsland FMAs |  | 7496 |  | T. Evans Grave Site | Local | Maintain a SMZ of 10 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 7266 |  | Tierneys Creek Battery & Mine Site | Regional | Maintain a SPZ of 100 m radius over the site. |  |
| Gippsland FMAs |  | 2584 |  | Tubal Cain Mine | Local | Maintain a SMZ of 10 m radius over the site. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 7273 |  | Tubal Cain Mine & Battery | Regional | Maintain a SPZ of 25 m radius over the site. |  |
| Gippsland FMAs |  | 2703 |  | Tyers-Traralgon Pipeline (incl. quarry and kiln) | RNE-n, Regional | Maintain a SMZ of 10 m either side of track. | Develop and comply with a SMZ plan |
| Gippsland FMAs |  | 7449 |  | Washington Winch | RNE-n, State, VHR | Maintain a SPZ of 200 m radius over the site. |  |
| Gippsland FMAs |  | 7562 |  | White Bridge Sawmill | Of interest | Maintain a SPZ of 20 m radius over the site. |  |
| Gippsland FMAs |  | 7296 |  | White Star No. 1 Mine Site | RNE-n, State | Maintain a SPZ of 50 m radius over the site. |  |
| Gippsland FMAs |  | 7297 |  | White Star No. 2 Mine Site | RNE-n, State | Maintain a SPZ of 50 m radius over the site. |  |
| Gippsland FMAs |  | 7005 |  | Yahoo Creek Battery | Local | Maintain a SMZ of 10 m radius over the site. | Develop and comply with a SMZ plan |
| Horsham | Settlement Road | 5090 | B61 | Wail Plantation |  | Maintain a SMZ of 10 m radius over the site. | Develop and comply with a SMZ plan |
| Mid-Murray | Gunbower State Forest | 2969 | PPR | Condidorio's Bridge, over Gunbower Creek | State, VHR | Maintain a SPZ of 10m either side of bridge abutments |  |
| North East FMAs | Bright |  |  | Pioneer Mine |  | Maintain a SMZ of 50 m radius over the site. | Develop and comply with a SMZ plan |
| North East FMAs | Mitta Mitta |  |  | Grand Junction Gold Sluicing claim | Regional | Maintain a SPZ encompassing Pioneer and Union Gold Sluicing Claim, Pride of Mitta Mitta Claim, Mammoth Sluicing Claim, and Long Point Sluicing Claim. |  |
| North East FMAs | Mitta Mitta |  |  | Long Point Gold Sluicing Claim | Regional | Maintain a SPZ encompassing Pioneer and Union Gold Sluicing Claim, Pride of Mitta Mitta Claim, Mammoth Sluicing Claim, and Grand Junction Sluicing Claim. |  |
| North East FMAs | Mitta Mitta |  |  | Pioneer and Union Gold. Sluicing Claim | RNE-r, VHR, State | Maintain a SPZ encompassing Pride of Mitta Mitta Claim, Mammoth Sluicing Claim, Grand Junction Claim and Long Point Sluicing Claim. |  |

Notes

RNE – , Sites listed on the Register of National Estate

RNE – I,  Sites registered on the interim list for the Register of National Estate

RNE – n,  Sites nominated to the Register of National Estate

RNE – r,  Sites ranked as having National Estate values

VHR – , Sites listed on the Victorian Heritage Register

# Landscape

## Landscape – fixed zoning

### Central Highlands FMAs

#### Maintain landscape management SPZs and SMZs in accordance with Table 9 (Landscape management FMZ rules)  below.

### East Gippsland FMA

#### Maintain SPZs and SMZs for landscape management in accordance with Table 9 (Landscape management FMZ rules) below.

### Gippsland FMAs

#### Maintain SPZs and SMZs for landscape management in accordance with Table 9 (Landscape management FMZ rules) below.

#### Plan and implement timber harvesting operations in areas zoned as SMZ for landscape in a manner that ensures the scenic value is maintained.

#### In the middle ground from the features listed in Table 9 (Landscape management FMZ rules) design timber harvesting operations and new roads to minimise their visual impact.

### Midlands FMA

#### Maintain the following sites in SPZ:

##### Victoria Mill Scenic Reserve;

##### Glut Scenic Reserve and surrounds as identified in map 19 of the 1982 LCC Ballarat Study Area Final Recommendations report;

##### scenic, floristic and recreational features associated with Richards picnic ground, Ditchfield picnic ground and Cave Hill Creek;

##### Ben Nevis Scenic Reserve;

##### Governor’s Rock Scenic Reserve; and

##### the Waterfalls Scenic Reserve.

### Mid Murray FMA

#### Maintain SPZs and SMZs for landscape management in accordance with Table 9 (Landscape management FMZ rules) below.

#### Within the landscape management SMZs and adjacent , manage timber harvesting operations as follows:

##### retain large old trees close to the roads (subject to public safety);

##### during peak periods of visitor use, conduct only those timber harvesting operations and silvicultural works (like thinning) which maintain visual amenity; and

##### ensure harvesting debris is removed from the corridor.

#### Site log landings and sleeper cutting decks at least 60 m from roads subject to landscape management SPZs.

### North East FMAs

#### Maintain SPZs and SMZs for landscape management in accordance with Table 9 (Landscape management FMZ rules) below.

#### Plan and implement timber harvesting operations and new road construction in areas zoned as SMZ for landscape in a manner that ensures the scenic value is maintained.

Table 9 Landscape management FMZ rules.

| FMA | Place | Significance | Zoning management actions | Management Actions |
| --- | --- | --- | --- | --- |
| Central Highlands FMAs | Forested escarpments of the Mounts Tallarook and Disappointment forest as viewed from the Hume Highway | LCC Rec E7 | Maintain prominent views of Mt Disappointment in SMZ. Mt Tallarook in GMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Murchison Fall and Strath Falls and their environs | N E, LCC Rec E7 | Maintain a SPZ over the sites. |  |
| Central Highlands FMAs | Natural environs of the Mounts Despair and Disappointment lookouts | LCC Rec E7 | Maintain a 50m radius SPZ around Mt Disappointment summit. Mt Despair in GMZ. |  |
| Central Highlands FMAs | Whittlesea - Yea Road |  | Maintain a 50 m SMZ either side of Whittlesea - Yea Road. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Melba Highway |  | Maintain a 100 m SMZ either side of Melba Highway. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Environs of the Toolangi-Black Range Forest Drive | LCC Rec E7 | Maintain in GMZ. |  |
| Central Highlands FMAs | Sylvia Creek falls and its environs | LCC Rec E7 | Maintain a SPZ over the site. |  |
| Central Highlands FMAs | Myers Creek Road |  | Maintain a 50 m SMZ either side of Myers Creek Road. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Murrindindi Road from Murrindindi to Murrindindi Cascades |  | Maintain a 50 m SMZ either side of Murrindindi Road from Murrindindi to Murrindindi Cascades. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Mt St Leonard |  | Maintain prominent views in SMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Dom Dom Saddle | N E | Maintain prominent views in SMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Maroondah Highway | National Trust Classified | Maintain a 100 m SMZ either side of Maroondah highway. Include prominent views in SMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Forested escarpments as viewed from the Maroondah and Goulburn Valley Highways | LCC Rec E7 | Maintain prominent views of Black Range and Blue Range in SMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Acheron Way | N E | Maintain a 50 m SMZ either side of Acheron Way. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Marysville Road (Narbethong to Marysville) |  | Maintain a 100 m SMZ either side of Marysville Road (Narbethong to Marysville). Include prominent views from Marysville in SPZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Steavenson Falls Road |  | Maintain a 50 m SMZ either side of Steavenson Falls Road. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Marysville - Buxton Road |  | Maintain a 50 m SMZ either side of Marysville - Buxton Road. Include prominent views of Mt. Strickland, Triangle Block and Mt. Margaret in SMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Natural environs of Lady Talbot Drive, Marysville | N E, LCC Rec E7 | Maintain a 50 m SMZ either side of Lady Talbot Drive. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Environs of the Marysville to Woods Point, Warburton to Woods Point and Jamieson to Woods Points Roads and the Lake Mountain access road. | LCC Rec E7 | Maintain a 50 m SMZ either side of each road and an SPZ over Woods Point. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Rubicon Forest | N E | Historic and Cultural Features Reserve. |  |
| Central Highlands FMAs | Morris Lookout | N E | Maintain a SMZ over the lookout. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Enoch’s Point township | N E | Maintain a 100 m radius SPZ over the township. |  |
| Central Highlands FMAs | Eildon Road |  | Maintain a 50 m SMZ either side of Eildon Road. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Big River | N E, Heritage River | Maintain a 200 m SPZ either side of Big River. |  |
| Central Highlands FMAs | Matlock Mining Area | N E | Maintain a 100 m radius SPZ over the township. |  |
| Central Highlands FMAs | Noojee township and valley | N E, LCC Rec E7 | Maintain a 50 m SPZ either side of Baw Baw road and Powelltown - Noojee road. Include prominent views from Powelltown - Noojee road in SMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Powelltown - Noojee Road |  | Maintain a 50 m SMZ either side of Powlltown - Noojee Road. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Spion Kopje Lookout | N E, LCC Rec E7 | Maintain a 50 m SPZ either side of the walking track and lookout. |  |
| Central Highlands FMAs | Prominent views from Neerim |  | Maintain a SMZ either side of Tarago River and over North Hells Gate lookout. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | South Hells Gate | N E | Maintain prominent views from Neerim in SMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Labertouche Caves |  | Maintain a SMZ over the site. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Mount Horsfall | N E | Maintain in GMZ. |  |
| Central Highlands FMAs | Toorongo Falls | LCC Rec E7 | Maintain a 100 m radius SPZ over Toorongo Falls and prominent views in SMZ. |  |
| Central Highlands FMAs | Environs of the Mount Toorongo lookout | LCC Rec E7 | Maintain prominent views of Mt. Toorongo in SMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Baw Baw Road | LCC Rec E7 | Maintain a 50 m SMZ either side of Baw Baw Road. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Thomson River corridor | N E | Maintain a 200 m radius SPZ over Thomson reservoir. |  |
| Central Highlands FMAs | Alpine Walking Track | N E, LCC Rec E7 | Maintain a 50 m SPZ either side of Alpine Walking Track. |  |
| Central Highlands FMAs | Cascade Creek | LCC Rec E7 | Maintain a SPZ over Cascade Creek at the junction of Thomson Valley Road. |  |
| Central Highlands FMAs | Thomson Valley - Mt St Gwinear Rd |  | Maintain a 50 m SMZ either side of Thomson Valley - Mt St Gwinear Rd. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Thomson Valley Road from Erica to the Thomson Reservoir | LCC Rec E7 | Maintain a 50 m SMZ either side of Thomson Valley Road from Erica to the Thomson Reservoir. Include prominent views of Mt. Erica and Erica surrounds in SMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Opposite Rawson Caravan park |  | Maintain 50 m of SMZ in the area opposite Rawson Caravan park. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Environs of the Walhalla tramway; Thomson River bridge | LCC Rec E7 | Maintain a 200 m SPZ either side of Thomson River. Include prominent views of Cascade Block in SMZ. |  |
| Central Highlands FMAs | Caringal Scout Camp |  | Maintain a SMZ over the site. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Moe - Walhalla Road |  | Maintain a 50 m SMZ either side of Moe - Walhalla Road. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Landscape of Pauls Range | LCC Rec E7 | Maintain a 50 m SMZ either side of Chum Creek Road. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Myers Creek Scenic Reserve | N E | Maintain a 50 m SMZ either side of Myers Creek Road. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Yarra River and environs | National Trust Classified, N E | Maintain a 200 m SPZ either side of river. |  |
| Central Highlands FMAs | Landscape values of the land bordering the Corranderrk and O’Shannassy aqueducts | LCC Rec E7 | Maintain prominent views of Mt. Toole-be-wong in SMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Views from Mt Donna Buang and from the Warburton Highway |  | Maintain a 100 m SMZ along the Warburton Highway. Include prominent views from Mt Donna Buang and in particular of Mt Little Joe and Mt Tugwell in SMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Acheron Way | N E | Maintain a 50 m SMZ either side of Acheron Way. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Warburton–Woods Point Road / Reefton Spur Road and Upper Yarra Reservoir Rd |  | Maintain a 50 m SMZ either side of Warburton–Woods Point Road / Reefton Spur Road and Upper Yarra Reservoir Rd. Include prominent views of Cement Creek, Armstrong Creek in SMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Yarra State Forest Drive | N E | Maintain in GMZ. |  |
| Central Highlands FMAs | Little Yarra Valley | National Trust Recorded | Maintain prominent views of Britannia Range in SMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Natural environs of Mount Beenak | LCC Rec E7 | Maintain prominent views from Little Yarra Road in SMZ. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Powelltown - Noojee Road |  | Maintain a 50 m SMZ either side of Powelltown - Noojee Road. | Develop and comply with a SMZ plan |
| Central Highlands FMAs | Seven Acre Rock |  | Maintain prominent views of Blue Range in SMZ. | Develop and comply with a SMZ plan |
| East Gippsland | Buchan. Dawson Range, Site # 312/01 | N E | Maintain site in SPZ. |  |
| East Gippsland | Buchan. Dawson Range to Snowy River National Park, Sites # 810/06, 810/06, 813/01 and 813/07) | N E | Maintain sites in SPZ. |  |
| East Gippsland | Buchan. Wattle Gully, Site # 812/09 |  | Maintain site in SMZ. Within the SMZ, limit the area of coupes which are highly visible from Gelantipy Road to 20 ha and leave at least 3 years between harvesting of adjacent coupes. | Develop and comply with a SMZ plan |
| East Gippsland | Buchan. W-Tree Creek headwaters, Site # 813/06 | N E | Maintain site in SMZ. Within the SMZ, limit the area of coupes which are highly visible from Gelantipy Road to 20ha and leave at least 3 years between harvesting of adjacent coupes. | Develop and comply with a SMZ plan |
| East Gippsland | Buchan. Black Satin Creek, Site # 810/07 |  | Maintain site in SMZ. Within the SMZ, limit the area of coupes which are highly visible from Gelantipy Road to 20ha and leave at least 3 years between harvesting of adjacent coupes. | Develop and comply with a SMZ plan |
| East Gippsland | Buchan. Genoa Gorge, Site #877/04 | N E | Maintain site in SPZ. |  |
| Gippsland FMAs | Boola Road | Scenic corridor along tourist road | Maintain a 20 m SMZ either side of the road. | Develop and comply with a SMZ plan |
| Gippsland FMAs | Thomson River | Victorian Heritage River | Maintain a 300 m SPZ adjacent to the river from Coopers Creek to Cowwarr Weir, incorporating LCC Natural Features Zone. |  |
| Gippsland FMAs | Avon River | LCC recommendation, Indicative aesthetic value (national estate) | Maintain a 100 m SPZ adjacent to the river upstream of Avon – Mt Hedrick Natural Features & Scenic Reserve. |  |
| Gippsland FMAs | Barkly River | LCC recommendation | Maintain a 100 m SPZ Natural Features Zone either side of the river downstream of Barkly River west branch. |  |
| Gippsland FMAs | Ben Cruachan Creek | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone upstream of Avon River. |  |
| Gippsland FMAs | Blue Pool | Locally popular recreation site with high scenic value | Protection provided by Natural Features Zone SPZ either side of the Freestone Creek. |  |
| Gippsland FMAs | Briagolong Forest Red Gum Reserve | High scenic values from Briagolong | Entire reserve included in SPZ. |  |
| Gippsland FMAs | Cobbannah Creek | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone downstream of Cobbannah. |  |
| Gippsland FMAs | Crooked River and Thirty Mile Creek | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone from Grant Historic Area to Basalt Creek. |  |
| Gippsland FMAs | Dargo High Plains Road | High scenic values from road. Indicative aesthetic value (national estate) | Maintain a 100 m SMZ either side of the road. | Develop and comply with a SMZ plan |
| Gippsland FMAs | Dargo River | LCC recommendation. LCC Representative River | Maintain a 100 m SPZ / Natural Features Zone from Little Dargo River to Grant Junction. |  |
| Gippsland FMAs | Freestone and Sportsman Creeks, Old Dargo Road | LCC recommendation, Indicative aesthetic value (national estate) | Maintain a 100 m SPZ / Natural Features Zone along entire length. |  |
| Gippsland FMAs | Glenmaggie Creek (East Branch) | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone downstream of Black Range Reference Area. |  |
| Gippsland FMAs | Heyfield – Licola Road | Scenic views along road, particularly from Licola | Maintain a SMZ to protect scenic values. | Develop and comply with a SMZ plan |
| Gippsland FMAs | Marathon Road | Scenic drive | Manage in GMZ to exhibit forest management practices. |  |
| Gippsland FMAs | Mitchell – Wonnangatta Rivers | Victorian Heritage River | The SPZ coincides with the Heritage River. |  |
| Gippsland FMAs | Moroka River | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone between Moroka Road and Little River. |  |
| Gippsland FMAs | Mt Skene Creek | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone from Barkly River to unnamed tributary above Cullen Creek. |  |
| Gippsland FMAs | Valencia Creek | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone downstream of Lamb Flat. |  |
| Gippsland FMAs | Wongungarra River | LCC recommendation. Nom. RNE (Wongungarra River headwaters) | Maintain a 100 m SPZ / Natural Features Zone upstream of the Wonnangatta River. |  |
| Gippsland FMAs | Hyland Highway | Scenic landscapes along tourist route | Maintain a 50 m SMZ either side of the road. | Develop and comply with a SMZ plan |
| Gippsland FMAs | South Gippsland Highway | Scenic landscapes along road | Maintain prominent views in SMZ. | Develop and comply with a SMZ plan |
| Gippsland FMAs | Strzelecki Highway | Scenic landscapes along tourist route | Maintain a 50 m SMZ either side of the road. | Develop and comply with a SMZ plan |
| Gippsland FMAs | Bruthen – Buchan Road | Scenic corridor along tourist road | Maintain a 50 m SMZ either side of the road. | Develop and comply with a SMZ plan |
| Gippsland FMAs | Deptford Road | Scenic corridor along tourist road | Maintain a 50 m SMZ either side of the road. | Develop and comply with a SMZ plan |
| Gippsland FMAs | Fairy Dell | LCC recommendation, Nom. RNE | Maintain a 50 m SMZ either side of the associated forest drive. | Develop and comply with a SMZ plan |
| Gippsland FMAs | Great Alpine Road | Scenic landscapes from tourist route. Indicative aesthetic value (national estate) (Tambo River Valley Road) | Maintain prominent views in SMZ. | Develop and comply with a SMZ plan |
| Gippsland FMAs | Haunted Stream Valley | Indicative aesthetic value (national estate), Locally popular touring destination | Maintain a 300 m SPZ either side of the river includes 100 m Natural Features Zone and historic sites. |  |
| Gippsland FMAs | Mississippi Creek | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone downstream of Melbourne - Orbost Railway (disused). |  |
| Gippsland FMAs | Mitchell River | Victorian Heritage River | The SPZ coincides with the Heritage River. |  |
| Gippsland FMAs | Nicholson River | LCC Representative River, Scenic views from locally popular touring route | Maintain a 100 m SPZ / Natural Features Zone downstream of Marthavale including SPZ up to track on west side of river and 50 m SMZ west side of Nicholson track. |  |
| Gippsland FMAs | Stutterin’ Fred’s Lookout | Scenic viewshed from lookout | Maintain site as lookout. |  |
| Gippsland FMAs | Tambo River (part) | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone upstream of Bruthen. |  |
| Gippsland FMAs | Wentworth River | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone upstream of Tabberabbera. |  |
| Gippsland FMAs | Bindi Lookout | Scenic viewshed from lookout | Maintain site as lookout. |  |
| Gippsland FMAs | Buenba Flat | Scenic location | Maintain a SMZ to protect aesthetic value of site. | Develop and comply with a SMZ plan |
| Gippsland FMAs | Dead Horse Creek (part) | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone. |  |
| Gippsland FMAs | Elphick Lookout | Scenic viewshed from lookout | Maintain site as lookout. |  |
| Gippsland FMAs | Great Alpine Road (part) | Scenic landscapes from touring route | Maintain prominent views in SMZ. | Develop and comply with a SMZ plan |
| Gippsland FMAs | Haunted Stream Valley/Stirling Ghost Town | Indicative aesthetic value (national estate), Locally popular touring destination | Maintain a 300 m SPZ either side of the river, includes 100 m Natural Features Zone and historic sites. |  |
| Gippsland FMAs | Mitta Mitta River | Victorian Heritage River | Maintain a 200 m SPZ either side of the river. |  |
| Gippsland FMAs | Nunniong Forest Drive | Scenic drive | Manage to exhibit current timber harvesting practices. Maintain a SPZ to protect significant stand of Alpine Ash. |  |
| Gippsland FMAs | Pendergast Lookout | Scenic viewshed from lookout, Indicative aesthetic value (national estate) | Maintain site as lookout. |  |
| Gippsland FMAs | Spring Creek | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone from Spring Creek Reference area to freehold boundary. |  |
| Gippsland FMAs | Tambo River (part) | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone including east branch to Garron Creek. |  |
| Gippsland FMAs | Timbarra River | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone from Tambo River to Nunniong Plains. Natural Features & Scenic Reserve, including area surrounding Timbarra Gorge and waterfall on Back River. |  |
| Gippsland FMAs | The Brothers | Scenic location | Maintain a SMZ to protect aesthetic value of site. | Develop and comply with a SMZ plan |
| Gippsland FMAs | Victoria River (part) | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone. |  |
| Gippsland FMAs | Wentworth River | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone upstream of Tabberabbera. |  |
| Gippsland FMAs | Wilkinson Creek | LCC recommendation | Maintain a 100 m SPZ / Natural Features Zone from Ferntree Creek to Timbarra River. |  |
| Mid Murray | River Track |  | Maintain a 20 m SPZ adjacent to the whole length of the road. |  |
| Mid Murray | Iron Punt Track |  | Maintain a 20 m SMZ adjacent to the whole length of the road. | Develop and comply with a SMZ plan |
| Mid Murray | Rifle Butts Track to Five Sleepers Track to Thompson Track |  | Maintain a 20 m SMZ adjacent to the whole length of the road. | Develop and comply with a SMZ plan |
| Mid Murray | Nursery Track |  | Maintain a 20 m SMZ adjacent to the whole length of the road. | Develop and comply with a SMZ plan |
| Mid Murray | Stanton Break |  | Maintain a 20 m SMZ adjacent to the whole length of the road. | Develop and comply with a SMZ plan |
| Mid Murray | Lock Road |  | Maintain a 20 m SMZ adjacent to the whole length of the road. | Develop and comply with a SMZ plan |
| North East FMAs | Beechworth Forest Drive | Scenic corridor for tourist roads | Maintain a 50 m SMZ each side of road. Most of Stanley Plateau is SPZ. | Develop and comply with a SMZ plan |
| North East FMAs | Big Hill/The Springs Saddle/Bald Hill/Bogong Jack Saddle | Mountain views from Mt Beauty township and Mt Fainter North | Maintain within SMZ. | Develop and comply with a SMZ plan |
| North East FMAs | Buckland Valley | NTA(V) recorded, LCC recommendation | LCC Natural Features Zone at Beveridge Station, from Dingo Ck to Buckland River East Branch. Maintain a 50 m SPZ each side of stream. |  |
| North East FMAs | Buffalo River upstream of Dandongadale | LCC recommendation | Maintain a 200 m SPZ; LCC Streamside conservation Natural Features Zone upstream of Catherine Station on east and west branches. Maintain a 50 m SMZ buffer each side of the remainder of the river. | Develop and comply with a SMZ plan |
| North East FMAs | Buffalo River upstream of Dandongadale | LCC recommendation | Maintain a 200 m SPZ; LCC Streamside conservation Natural Features Zone upstream of Catherine Station on east and west branches. Maintain a 50 m SMZ buffer each side of the remainder of the river. | Develop and comply with a SMZ plan |
| North East FMAs | Clear Creek (Tributary of Buckland River) | Visual corridor and natural water feature | Maintain a 50 m SMZ each side of stream. | Develop and comply with a SMZ plan |
| North East FMAs | Conic Range (partly State forest) | Ridge-line features | Maintain within SPZ. |  |
| North East FMAs | Dandongadale River | Visual corridor and natural water feature | Maintain a 50 m SMZ each side of river. | Develop and comply with a SMZ plan |
| North East FMAs | Dandongadale River | Visual corridor and natural water feature | Maintain a 50 m SMZ each side of river. | Develop and comply with a SMZ plan |
| North East FMAs | Granite Peak | RNE Natural site. Mountain Views from Omeo Hwy | Maintain as SMZ in State forest, above 1100 m elevation. | Develop and comply with a SMZ plan |
| North East FMAs | Harkers Creek (Mitta Mitta) | Visual corridor and natural water feature | Maintain a 50 m SPZ each side of stream. |  |
| North East FMAs | Harrietville Valley (vicinity of Harrietville) and Ovens River, West Branch | NTA(V) recorded, Visual corridor and natural water feature, LCC recommendation | Maintain a 20 m SMZ each side of the Great Alpine Road. 200 m LCC Streamside Conservation Natural Features Zone on Ovens River West Branch downstream of Gunns Creek. | Develop and comply with a SMZ plan |
| North East FMAs | Mitta Mitta Valley (downstream of Mitta Mitta) | NTA(V) classified, Visual corridor and natural water feature | Maintain a 50 m SMZ each side of road where State forest. | Develop and comply with a SMZ plan |
| North East FMAs | Mt Murramurrangbong and Mt Big Ben | Indicative aesthetic value | Mt Murramurrangbong is SPZ and Mt Big Ben (National Estate) summit is SPZ. Coupe design. |  |
| North East FMAs | Mt Pinnibar | Scenic mountain views | Located in LCC Natural Features and Scenic Reserve. |  |
| North East FMAs | Murmungee Basin, Flagstaff Range (western aspect) from Lady Newton Drive to Taylors Gap | NTA(V) classified | SPZ provides protection. |  |
| North East FMAs | Myrtle (Barwidgee) Creek Valley (vicinity of Myrtleford) | NTA(V) classified, Visual corridor and natural water | Maintain a 50 m SPZ on State forest side of Stanley to Barwidgee Road feature. |  |
| North East FMAs | Nariel-Benambra Road | Scenic corridor for tourist road | Maintain a 50 m SMZ each side of road. | Develop and comply with a SMZ plan |
| North East FMAs | Pheasant Creek (Tributary of Buckland River) | Visual corridor and natural water feature | Maintain a 50 m SMZ each side of stream. | Develop and comply with a SMZ plan |
| North East FMAs | Snowy Creek Waterfall | Natural water feature | Maintain a 100 m radius SPZ over the falls. |  |
| North East FMAs | The Pinnacles & Mt Jack including Barwidgee Creek | Indicative aesthetic value (National Estate) | Headwaters of Barwidgee Creek is SPZ, some additional SPZ, also coupe design in GMZ. |  |
| North East FMAs | Trappers Creek (near Mountain Creek) | Visual corridor and natural water feature | Maintain a 50 m SMZ each side of stream. | Develop and comply with a SMZ plan |
| North East FMAs | Upper Nariel valley above Crawford Crossing | Visual corridor and natural water feature | Maintain a 50 m SMZ each side of stream (where in State forest). | Develop and comply with a SMZ plan |
| North East FMAs | Upper Wongungarra River Catchment | LCC recommendation | Located within 200 m LCC Streamside Conservation Natural Features Zone on Selwyn Ck. |  |
| North East FMAs | West Kiewa valley | Viewed from Mt Feathertop, indicative aesthetic National Estate | Located in a Designated Catchment, SPZ and SMZ. | Develop and comply with a SMZ plan |
| North East FMAs | Wongungarra & Humffray Rivers & Teatree Range except ridge | Indicative Natural Landscape (National Estate) | Located in National Park, SMZ and Mt Sarah Scenic Reserve. | Develop and comply with a SMZ plan |
| North East FMAs | Yackandandah Creek (from Nine Mile Creek to State forest boundary) | Visual corridor and natural water feature | Maintain a 50 m SMZ either side of creek. | Develop and comply with a SMZ plan |
| North East FMAs | Yarrabulla Creek | Visual corridor and natural water feature | Maintain a 50 m SMZ each side of stream. Partly within Remote & Natural Area. | Develop and comply with a SMZ plan |
| North East FMAs | Yarrabulla Creek | Visual corridor and natural water feature | Maintain a 50 m SMZ each side of stream. Partly within Remote & Natural Area. | Develop and comply with a SMZ plan |
| North East FMAs | 15 Mile Creek | Visual corridor and natural water feature | Maintain a 50 m SMZ either side of creek. | Develop and comply with a SMZ plan |
| North East FMAs | Bindaree Falls | Unique Water Feature | Maintain a 100 m radius SPZ over the falls. |  |
| North East FMAs | Carters Road Picnic Area | High scenic values in recreation area | Maintain a 50 m radius SMZ over the picnic area. | Develop and comply with a SMZ plan |
| North East FMAs | Craig Hut | Scenic location | Maintain a 200 m radius SPZ over the hut. |  |
| North East FMAs | Goulburn River (Knockwood to Jamieson) | LCC 1991 Victorian Heritage River A5 | Maintain a 200 m SPZ each side of stream (incorporates 200 m LCC Streamside Conservation Natural Features Zone from Knockwood to Stony Creek). |  |
| North East FMAs | Holland Creek (part) | Visual corridor and natural water feature | Maintain a 50 m SMZ either side of creek. | Develop and comply with a SMZ plan |
| North East FMAs | Howqua River (Tobacco Flat to Running Creek) | LCC 1991 Victorian Heritage River A3 | Maintain a 200 m SPZ each side of stream. |  |
| North East FMAs | Jamieson River (above Grannies Flat) | Visual corridor and natural water feature | Maintain a 200 m SPZ each side of stream. Existing roads and recreation facilities to remain. |  |
| North East FMAs | Lightning Ridge (Strathbogie Ranges) | Mountain views from Lake Nillahcootie and Midland Highway | SMZ and SPZ will adequately protect this area. | Develop and comply with a SMZ plan |
| North East FMAs | Moonee Range (Strathbogie Ranges) | Mountain view from Midland Highway | Maintain within SMZ. | Develop and comply with a SMZ plan |
| North East FMAs | Plain Creek Tramway (Sawmill Settlement) | Historic site of local significance | Maintain a 20 m buffer around the historic trestle bridge. |  |
| North East FMAs | Sandy Creek Waterfall (Strathbogie Ranges) | Unique Water Feature | Maintain a 100 m radius SPZ over the falls. |  |
| North East FMAs | Upper Delatite valley (vicinity of Mirimbah) | LCC recommendation | Maintain a 200 m SPZ each side of river (incorporates 200 m LCC Streamside Conservation Natural Features Zone). |  |

## Heritage Rivers and Natural Catchment Areas – fixed zoning

#### Maintain SPZ across all areas of State forest where Heritage River Areas apply to the extent defined within the Heritage River Act 1992. (except where timber harvesting is permitted in parts of the Lower Snowy River, Goulburn and Ovens Heritage River Areas).

#### Maintain SPZ across all areas of State forest where Natural Catchment Areas apply. Exclude all road construction and upgrading activities within Natural Catchment Area SPZ.

## River Zones and Natural Features Zones – fixed zoning

### Central FMAs

#### Maintain SPZ across all areas of State forest where River Zones apply to the extent defined within the Melbourne Area District 2 Review (LCC 1994). River Zones apply along the upper Goulburn, Murrundindi, Acheron, Latrobe and Toorongo Rivers and Snobs Creek.

### East Gippsland FMA

#### Maintain SPZ across all areas of State forest where Natural Features Zones apply to the extent defined by the Land Conservation Council Final Recommendations Gippsland Lakes Hinterland Study Area, Alpine Area Special Investigation and East Gippsland Area Review reports (LCC 1983a, 1983b, 1986), except where superseded by the Heritage River Act 1992.

# Recreation – fixed zoning

### Statewide

#### Maintain SPZs and SMZs for recreation sites in accordance with Table 10 (FMZ rules for recreation sites) below.

#### When conducting timber harvesting operations in SMZs first develop a SMZ plan. SMZ plans must be complied with during timber harvesting operations.

#### Within the Australian Alps Walking Track (AAWT) SPZ:

##### construct and maintain roads that cross the AAWT in a manner that considers the needs of walkers using the track; and

##### consider the use of alternative extraction routes during harvesting operations to protect the AAWT.

### Midlands FMA

#### In the Midlands FMA, include all existing areas of State forest that are currently classified as semi-remote in the SPZ. Exclude timber harvesting and new roads or tracks from these areas and maintain any existing roads and tracks at or below their current road classification.

Table 10 FMZ rules for recreation sites

| FMA | Locality | Place | Zoning management actions |
| --- | --- | --- | --- |
| Central Highlands FMAs | Alexandra | Kendalls picnic areas | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Alexandra | Lower Big River camping areas | Maintain a 200 m radius SPZ over the site. |
| Central Highlands FMAs | Alexandra | Mt Despair Track | Maintain a 50 m radius SPZ either side of the track. |
| Central Highlands FMAs | Alexandra | Mt Torbreck Track | Maintain a 50 m radius SPZ either side of the track. |
| Central Highlands FMAs | Alexandra | Myrtle Creek Track | Maintain a 50 m radius SPZ either side of the track. |
| Central Highlands FMAs | Alexandra | Snobs Creek Falls | Maintain a 100 m radius SPZ over the site. |
| Central Highlands FMAs | Alexandra | Stanley Homestead Historic Area | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Alexandra | Tanglefoot Track | Maintain a 50 m radius SPZ either side of the track. |
| Central Highlands FMAs | Alexandra | Torbreck Station | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Alexandra | Wirrawilla Rainforest Walk | Maintain a 50 m radius SPZ either side of the track. |
| Central Highlands FMAs | Broadford | Anderson Garden picnic area | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Broadford | Blairs Hut - Mt Disappointment summit | Maintain a 50 m radius SPZ either side of the track. |
| Central Highlands FMAs | Broadford | Blairs Hut picnic area | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Broadford | Freemans picnic area - Tallarook SF | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Broadford | Mt Disappointment summit | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Broadford | No 1 Camp picnic area | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Broadford | Regular Camp picnic area | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Broadford | Reservoir Picnic area - Tallarook SF | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Broadford | Strath Creek Falls picnic area | Maintain a 100 m radius SPZ over the site. |
| Central Highlands FMAs | Broadford | Sunday Creek Reservoir Track | Maintain a 50 m radius SPZ either side of the track. |
| Central Highlands FMAs | Toolangi | Anderson’s Mill | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Toolangi | Beauty Spot | Maintain a 50 m radius SPZ either side of the track. |
| Central Highlands FMAs | Toolangi | Big River camp | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Toolangi | Big River camping areas | Maintain a 200 m radius SPZ over the site. |
| Central Highlands FMAs | Toolangi | Dindi Mill picnic area | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Toolangi | Dom Dom Saddle to Mt Donna Buang | Trees in the maintained firebreak area not to be harvested. |
| Central Highlands FMAs | Toolangi | Keppel Track | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Toolangi | Man O’War Track | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Toolangi | Manfern Track | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Toolangi | Michaeldene Track | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Toolangi | Mt Tanglefoot car park picnic area | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Toolangi | Murrindindi Scenic Reserve | Maintain a 100 m radius SPZ over the site. |
| Central Highlands FMAs | Toolangi | Oxley Track | Maintain a 50 m radius SPZ either side of the track. |
| Central Highlands FMAs | Toolangi | Robertson Gully Track | Maintain a 50 m radius SPZ either side of the track. |
| Central Highlands FMAs | Toolangi | Somers Park | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Toolangi | Sylvia Creek Road picnic area | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Toolangi | Taggerty River (Lady Talbot Drive) | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Toolangi | The Triangle | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Toolangi | Tree Fern Gully | Maintain a 50 m radius SPZ either side of the track. |
| Central Highlands FMAs | Toolangi | Trestle Track | Maintain a 50 m radius SPZ either side of the track. |
| Central Highlands FMAs | Toolangi | Woods Lookout Track | Maintain a 50 m radius SPZ either side of the track. |
| Central Highlands FMAs | Toolangi | Yellow Dog Road | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Erica | Blue Rock Reservoir area | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Erica | Cast Iron Point | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Erica | Growlers | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Erica | Jans Hut | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Erica | Jericho | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Erica | O’Shea’s campground | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Erica | Old Growth Forest Track (Middle Tyers track) | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Erica | South Cascade | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Noojee | Comp 22 | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Noojee | Latrobe River (Hawthorn) | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Noojee | Matlock picnic area | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Noojee | Noojee Depot | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Noojee | Tanjil Bren | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Noojee | The Poplars | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Noojee | Toorongo (Link Road) | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Noojee | Toorongo Falls | Maintain a 100 m radius SPZ over the site. |
| Central Highlands FMAs | Dandenong | Ada No. 2 Mill | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Dandenong | Ada Tree (Carpark) | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Dandenong | Ada Tree Loop | Maintain a 50 m radius SPZ over the walking track & 100 m SPZ around the Ada Tree. |
| Central Highlands FMAs | Dandenong | Big Pats Creek | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Dandenong | Federal Mill | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Dandenong | High Lead Carpark | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Dandenong | La La Falls | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Dandenong | La La Falls | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Dandenong | Powelltown (opposite office) | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Dandenong | Reids Tramline loop | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Dandenong | Richards Tramline | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Dandenong | Seven Acre Rock | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Dandenong | Seven Acre Rock | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Dandenong | Spion Kopje | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Dandenong | Starlings Gap | Maintain a 50 m radius SPZ over the site. |
| Central Highlands FMAs | Dandenong | Walk into history - Ada No 2 Mill - Dowey Spur Rd | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Dandenong | Walk into history - Ada No 2 Mill - Federal Road | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Dandenong | Walk into history - Ada No 2 Mill - New Ada Mill | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Dandenong | Walk into history - Big Pats Ck - Starlings Gap | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Dandenong | Walk into history - Dowey Spur Rd - High Lead Carpark | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Dandenong | Walk into history - High lead Carpark - The Bump | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Dandenong | Walk into history - Starlings Gap - Ada No 2 Mill | Maintain a 50 m SPZ either side of the track. |
| Central Highlands FMAs | Dandenong | Walk into history - The Bump - Powelltown Picnic Area | Maintain a 50 m SPZ either side of the track. |
| Gippsland FMAs | Erica (Mt Singleton to Mt Skene Natural Features & Scenic Reserve) | Australian Alps Walking Track. LCC recommendation, Indicative aesthetic value (national estate) | Maintain a 50 m SPZ either side of the track. |
| Gippsland FMAs | Erica | Bruntons Bridge | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Erica | Hanging Rock | Maintain a 50 m SPZ either side of the track. |
| Gippsland FMAs | Erica | Jorgensen’s Flat | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Erica | Locks | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Erica | Merringtons | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Erica | Morning Star Waterwheel | Maintain a 50 m radius SPZ either side of the track. |
| Gippsland FMAs | Erica | O'Tooles 1&2 | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Erica | The Junction | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | 12 Mile Creek | Maintain a 50 m radius SMZ over the site. |
| Gippsland FMAs | Heyfield | 25 Mile Creek | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | 30 Mile Creek | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield (Mt Singleton to Mt Skene Natural Features & Scenic Reserve) | Australian Alps Walking Track. LCC recommendation, Indicative aesthetic value (national estate) | Maintain a 50 m SPZ either side of the track. |
| Gippsland FMAs | Heyfield | Bald Top | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Barkly Bridge | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Bennison Lookout | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Bicentennial National Trail | Maintain a 50 m radius SPZ either side of the track. |
| Gippsland FMAs | Heyfield | Black Flat | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Black Snake | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Blue Pool | Maintain a 100 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Cheynes Bridge | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Collins Flat | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Connors Plain | Maintain a 20 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | East Barkly | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Froam Camp Site | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Gibraltar | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Gows Hotel | Maintain a 50 m radius SMZ over the site. |
| Gippsland FMAs | Heyfield | Harrison’s Cut | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Kinleys Yards | Maintain a 50 m radius SMZ over the site. |
| Gippsland FMAs | Heyfield | Lee Creek | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Lees Creek | Maintain a 50 m SPZ either side of the track. |
| Gippsland FMAs | Heyfield | Lennies Cutting | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | McKinnon’s Point | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Moroka Gorge Lookout | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Noon Road | Maintain a 50 m radius SMZ over the site. |
| Gippsland FMAs | Heyfield | Paddy Lee Crossing | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Pretty Boy Hill | Maintain a 20 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Rumpffs Flat | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Tablelands | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Upper Dargo (including Two Mile Creek, Italian Flat, Jimmy Iversons and Ollies Jumpup) | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Valencia Creek | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Heyfield | Winke Creek | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Yarram | The Gums | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Yarram | White Woman’s Waterhole | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Yarram | White Woman’s Waterhole | Maintain a 50 m radius SPZ either side of the track. |
| Gippsland FMAs | Bairnsdale | Barksheds | Maintain a 100 m radius SPZ over the site. |
| Gippsland FMAs | Bairnsdale | Bicentennial National Trail | Maintain a 50 m SPZ either side of the track. |
| Gippsland FMAs | Bairnsdale | Dawson City | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Bairnsdale | Deptford | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Bairnsdale | Dogtown | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Bairnsdale | Gippsland Lakes Tramway Trail | Maintain a 50 m radius SPZ either side of the track. |
| Gippsland FMAs | Bairnsdale | Jones Hut | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Bairnsdale | Marthavale Hut | Maintain a 20 m radius SPZ over the site. |
| Gippsland FMAs | Bairnsdale | Mount Taylor | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Bairnsdale | Seldom Seen Hut | Maintain a 20 m radius SPZ over the site. |
| Gippsland FMAs | Bairnsdale | Stirling | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Bairnsdale | Turntable Camp | Maintain a 20 m radius SPZ over the site. |
| Gippsland FMAs | Bairnsdale | Wallers Hut | Maintain a 20 m radius SPZ over the site. |
| Gippsland FMAs | Swifts Creek | Australian Alps Walking Track. LCC recommendation, Indicative aesthetic value (national estate) | Maintain a 50 m SPZ either side of the track. |
| Gippsland FMAs | Swifts Creek | Bicentennial National Trail | Maintain a 50 m SPZ either side of the track. |
| Gippsland FMAs | Swifts Creek | Dog’s Grave | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Swifts Creek | Timbarra River | Maintain a 50 m radius SPZ over the site. |
| Gippsland FMAs | Swifts Creek | Washington Winch | Maintain a 200 m radius SMZ over the site. |
| Midlands | Mount Cole SF | Ben Nevis | Maintain a SPZ over the site and environs. |
| Midlands | Mount Cole SF | Chinamans | Maintain a SPZ over the site and environs. |
| Midlands | Mount Cole SF | Ditchfields | Maintain a SPZ over the site and environs. |
| Midlands | Mount Cole SF | Mugwamp | Maintain a SPZ over the site and environs. |
| Midlands | Mount Cole SF | Richards Tramline | Maintain a SPZ over the site and environs. |
| Midlands | Mount Cole SF | The Glut | Maintain a SPZ over the site and environs. |
| Midlands | Mount Cole SF | Victoria Mill | Maintain a SPZ over the site and environs. |
| Midlands | Pyrenees SF | Governors Rock | Maintain a SPZ over the site and environs. |
| Midlands | Pyrenees SF | Waterfalls | Maintain a SPZ over the site and environs. |
| Midlands | Wombat SF | Binks | Maintain a SPZ over the site and environs. |
| Midlands | Wombat SF | Firth Park | Maintain a SPZ over the site and environs. |
| Midlands | Wombat SF | Lyonville Springs | Maintain a SPZ over the site and environs. |
| Midlands | Wombat SF | Nolans | Maintain a SPZ over the site and environs. |
| Midlands | Wombat SF | Werribee | Maintain a SPZ over the site and environs. |
| Midlands | Wombat SF | Wombat Creek Dam | Maintain a SPZ over the site and environs. |
| North East FMAs | Beechworth | Bates Dam | Maintain a 20 m radius SPZ over the site. |
| North East FMAs | Beechworth | Beechworth Forest Drive (Includes Clarkes Corner, Murmungee Lookout and Bates Dam en route) | Maintain a SMZ over the site and environs. |
| North East FMAs | Beechworth | Clarkes Corner | Maintain a 20 m radius SPZ over the site. |
| North East FMAs | Beechworth | Murmungee Lookout | Maintain a 20 m radius SPZ over the site. |
| North East FMAs | Beechworth | Yackandandah Creek | Maintain a SPZ over the site and environs. |
| North East FMAs | Beechworth | Yackandandah Forest Drive (Includes Yackandandah Creek en route) | Maintain a SPZ over the site and environs. |
| North East FMAs | Bright | Apex Lookout | Maintain a 50 m SPZ either side of the track. |
| North East FMAs | Bright | Bakers Gully | Maintain a 50 m SPZ either side of the track. |
| North East FMAs | Bright | Bakers Gully Reservoirs | Maintain a 20 m radius SPZ over the site. |
| North East FMAs | Bright | Bright Memorial Arboretum | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Bright | Bright Memorial Arboretum | Maintain a 50 m SPZ either side of the track. |
| North East FMAs | Bright | Buckland Valley | Maintain a 100 m radius SPZ along the length of the river. |
| North East FMAs | Bright | East Kiewa Pack Track | Maintain a 50 m SPZ either side of the track. |
| North East FMAs | Bright | Huggins Lookout | Maintain a 50 m SPZ either side of the track. |
| North East FMAs | Bright | Mt Emu (Eskdale Spur) | Maintain a 100 m radius SPZ over the site. |
| North East FMAs | Bright | Mt Porepunkah | Maintain a Scenic Reserve. |
| North East FMAs | Bright | Snowy Creek (Dungeys Track) | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Bright | Valley View | Maintain a 50 m radius SPZ either side of the track. |
| North East FMAs | Corryong | Barlows Creek | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Corryong | Bullocky Crossing | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Corryong | Cattlemans Creek | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Corryong | Dart River | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Corryong | Dunstan Logging Huts | Maintain a 20 m radius SPZ over the site. |
| North East FMAs | Corryong | Gibson Hut | Maintain a 20 m radius SPZ over the site. |
| North East FMAs | Corryong | Gray’s Track Camp | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Corryong | Little Bunroy Ck Camp | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Corryong | O'Hagans Camp | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Corryong | Paddy Joy’s Camp | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Corryong | Sassafras township site | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Corryong | Ski Hut | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Corryong | Surveyors Creek Camp | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Corryong | Wheeler Creek Hut | Maintain a 20 m radius SPZ over the site. |
| North East FMAs | King Valley | 15 Mile Creek | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | King Valley | Boggy Creek | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | King Valley | Carboor Range North | Maintain a 20 m radius SPZ over the site. |
| North East FMAs | King Valley | Cheshunt Right Arm | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | King Valley | Evans Creek | Maintain a 20 m radius SPZ over the site. |
| North East FMAs | King Valley | Meadow Creek | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | King Valley | Yackandandah Creek | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Myrtleford | Buffalo River. (Includes Blades, Tea Tree, Manna Gum, Abbeyard and Cobbler Lake Junction) | Maintain a 100 m SPZ along length of river. |
| North East FMAs | Myrtleford | Dandongadale River | Maintain a 50 m SMZ. |
| North East FMAs | Myrtleford | Humffray River & Riley Creek Area | Maintain a 200 m Natural Features Zone. |
| North East FMAs | Myrtleford | Reform Hill Walking | Entire State forest area of Reform Hill. |
| North East FMAs | Tallangatta | Australian Alps Walking Track, Long Spur (ANP boundary to Mt Wills Historic Area). LCC recommendation | Maintain a 50 m SPZ either side of the track. |
| North East FMAs | Tallangatta | Deep Gully Walk | Maintain a 50 m SPZ either side of the track. |
| North East FMAs | Tallangatta | Lightning Creek Picnic Area | Within SPZ for frog. |
| North East FMAs | Tallangatta | Little Snowy Creek | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Tallangatta | Mitta Mitta Historic Reserve | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Tallangatta | Pioneer Mine | Maintain a 50 m SPZ either side of the track. |
| North East FMAs | Tallangatta | River Walk | Maintain a 50 m SPZ either side of the track. |
| North East FMAs | Tallangatta | Snowy Creek Picnic Area | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Tallangatta | The Hollow (Walk to Mt Bogong via Granite Flat Spur) | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Tallangatta | The Mill (beside Mitta Mitta River) | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Tallangatta | The Walnuts | Maintain a 20 m radius SPZ over the site. |
| North East FMAs | Tallangatta | Water Race Track and Mt Welcome Track | Maintain a 50 m radius SPZ either side of the track. |
| North East FMAs | Benalla | James Reserve | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Benalla | Jones | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Benalla | Kelly Tree | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Benalla | Leerson Hill Track | Maintain a 50 m SPZ either side of the track. |
| North East FMAs | Benalla | Lima Creek Falls Track | Maintain a 50 m SPZ either side of the track. |
| North East FMAs | Benalla | Lima Creek Track | Maintain a 50 m SPZ either side of the track. |
| North East FMAs | Benalla | Stringybark Creek | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Benalla | Toombullup North School Site | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | 12 Mile Reserve | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Australian Alps Walking Track (Mt Sunday to Mt McDonald). LCC recommendation | Maintain a 50 m radius SPZ either side of the track. |
| North East FMAs | Mansfield | Bains Bridge | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Bindaree Falls | Maintain a 50 m radius SPZ either side of the track. |
| North East FMAs | Mansfield | Blue Range Creek | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Burns Bridge | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Buttercup Creek | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Carters Road | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Craig Hut | Maintain a 200 m radius SPZ over the site. |
| North East FMAs | Mansfield | Craig Hut | Maintain a 50 m radius SPZ either side of the track. |
| North East FMAs | Mansfield | Grannys Flat | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Holylands | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Howqua Bridle Track (also known as Howqua Feeder Track) | Maintain a 50 m radius SPZ either side of the track. |
| North East FMAs | Mansfield | Knockwood Reserve | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Low Saddle | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Mitchell's Bridle Track | Maintain a 50 m radius SPZ either side of the track. |
| North East FMAs | Mansfield | Plain Creek | Maintain a 50 m radius SPZ either side of the track. |
| North East FMAs | Mansfield | Running Creek | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Snake Reserve | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Timbertop | Maintain a 50 m radius SPZ either side of the track. |
| North East FMAs | Mansfield | Tomahawk Hut | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Tunnel Bend | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Upper Jamieson Hut | Maintain a 50 m radius SPZ over the site. |
| North East FMAs | Mansfield | Wrens Flat | Maintain a 50 m radius SPZ over the site. |

# Research – fixed zoning

### Statewide

#### Maintain SPZs and SMZs for research sites in accordance with table 11 below.

Table 11 FMZ rules for research sites.

| FMA | Locality | Research project | Zone | Management Actions |
| --- | --- | --- | --- | --- |
| Central Highlands FMAs | Black, Goulburn and Snake Rivers | Spotted Tree Frog monitoring transect | SPZ |  |
| Gippsland FMAs | Harraps Highway | Regrowth of Phytophthora cinnamomi dieback | SMZ | Limit activities to those that are compatible with the experiment objectives. |
| Gippsland FMAs | Old Rosedale Road | Planting on dieback affected sites | SMZ | Limit activities to those that are compatible with the experiment objectives. |
| Gippsland FMAs |  | Boola Silvertop thinning trials | SMZ | Limit activities to those that are compatible with the experiment objectives. |
| Gippsland FMAs |  | Mullungdung Phytophthora cinnamomi progeny trial EUC475 | SMZ | Limit activities to those that are compatible with the experiment objectives. |
| Gippsland FMAs | Bentley's Plain Road | E. delegatensis provenance trial | SMZ | Limit activities to those that are compatible with the experiment objectives. |
| Gippsland FMAs | Engineers Road, Barmouth Spur Track, Clarks Track, Angora Range Road | Gippsland mixed species growth study | SMZ | Limit activities to those that are compatible with the experiment objectives. |
| Midlands | Wombat and Mt Cole SF | Armillaria research | SMZ | Exclude timber harvesting and other management activities which will affect research objectives until 2020. |
| Midlands | Wombat SF | Fire effects study area | SMZ | Exclude timber harvesting and other management activities until such time as the areas are no longer required. |
| Midlands | Wombat SF | Stewart's Creek catchment hydrology | SPZ |  |
| Midlands | Wombat SF | Regrowth thinning and fertiliser trials | SMZ | Restrict timber harvesting and other management operations to those that are compatible with the objectives of the research until 2020. |
| Midlands |  | Forest growth plots | SMZ | Exclude timber harvesting and other management activities until such time as the areas are no longer required. |
| Midlands |  | Continuous Forest Inventory plots | SMZ | Protect unharvested plots and a 50m buffer until 2020. Review the need for further protection at this time. |
| North East FMAs | Clear Hills | 1957 Alpine Ash thinning trials | SMZ | Limit activities to those that are compatible with the experiment objectives. |
| North East FMAs | Flagstaff | KTRI Paterson's Curse Crown Boring Weevil research site | SMZ | Limit activities to those that are compatible with the experiment objectives. |
| North East FMAs | Highett Ridge Eildon Mount Elliot | KTRI St John's Wort Mite research sites | SMZ | Limit activities to those that are compatible with the experiment objectives. |
| North East FMAs | Jamieson River North Branch Howqua River King River Buffalo Creek Buckland River Wongungarra River West Kiewa River Snowy Creek Lightning Creek Big River Wheeler Creek | Spotted Tree Frog monitoring transect | SPZ |  |
| North East FMAs | Lords Creek Mt Wills Creek / Snowy Creek Lightning Creek Track Bullhead Callaghan Creek Long Spur | KTRI Blackberry Rust Fungus research sites | SMZ | Limit activities to those that are compatible with the experiment objectives. |
| North East FMAs | Mt Pinnibar | 1928 Alpine Ash thinning trial | SMZ | Limit activities to those that are compatible with the experiment objectives. |
| North East FMAs | Myrtleford | Cropper Creek hydrologic research project | SMZ | Limit activities to those that are compatible with the experiment objectives. |
| North East FMAs | Old Tolmie Road, Toombullup | CFTT Tree Breeding Research provenance trials for E. delegatensis (EUC 411 1979), E. globulus (EUC413 1981) and E. nitens (EUC 426 1990) | SMZ | Limit activities to those that are compatible with the experiment objectives. |
| North East FMAs | Riley Creek | Long-footed Potoroo research trapping grid | SMZ | Manage in accordance with Long-footed Potoroo prescriptions. |